3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210811

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	Ericsson

Title:	Selected T-EAS notification

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution proposes an EDGE-3 interaction for notifying selected T-EAS in the S-EAS decided ACR.

2. Reason for Change

In the case of S-EES triggered T-EAS discovery, the S-EES determines the selected T-EAS and T-EES.

In the case of S-EAS triggered T-EAS discovery (in S-EAS decided ACR scenario), the S-EAS makes the decision for the selection of T-EAS among all T-EAS candicates (possibly offered by several T-EESes). In this case, without being told by S-EAS, the S-EES cannot determine the selected T-EAS and consequently the selected T-EES to be sent to the EEC in the target information notification message.

It is proposed to add EAS selection notification for EDGE-3 interface.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

#### 8.8.3.2	Discover T-EAS

Figure 8.8.3.2-1 illustrates the procedure for fetching T-EAS information. This procedure may be utilized by a S-EAS, which undertakes the transfer of application context information to a T-EAS directly, or can be invoked by the S-EES itself on deciding to execute ACR.

Pre-conditions:

1.	Information related to the EES is available with the S-EAS.

Figure 8.8.3.2-1: Discover T-EAS

1.	The S-EAS sends the EAS discovery request to the S-EES or the S-EES decides to execute the ACR. The EAS discovery request from the S-EAS includes the requestor identifier [EASID] along with the security credentials and includes EAS discovery filter matching its EAS profile. If target DNAI is available at the S-EAS via User Plane Path change event, the S-EAS provides the S-EES with the target DNAI.

NOTE:	The trigger condition to invoke the Discover T-EAS API is up to application service logic, which is out of scope of this specification.

2.	If the request is received from the S-EAS, the S-EES checks whether the requesting EAS is authorized to perform the discovery operation. If the S-EES decided to execute the ACR or when the requesting EAS is authorized, the S-EES checks if there exists a T-EAS information (registered or cached) that can satisfy the requesting EAS information, additional query filters and the Expected service KPIs and the Minimum required service KPIs if received from the EEC during the EAS discovery or from the EAS in step 1. If the source Edge Enable Server finds the T-EAS(s) in the cached or registered information, the flow continues with step 5, else the S-EES retrieves the T-EES address from the ECS as specified in clause 8.8.3.3 and continues with step 3.

3.	The S-EES invokes the EAS discovery request on the T-EES. The S-EES discovery request includes the requestor identifier [EESID] along with the security credentials and includes EAS discovery filter. In the EAS discovery filter, the S-EES may include the Expected service KPIs and the Minimum required service KPIs if received from the EEC during the EAS discovery or from the EAS in step 1.

Upon receiving the request, the T-EES may trigger the EAS management system to instantiate the T-EAS that matches with EAS discovery filter IEs (e.g. ACID) as in clause 8.11.

4.	The T-EES discovers the T-EAS(s) and responds with the discovered T-EAS information to the S-EES. To filter T-EAS(s), the T-EES utilizes the discovery filters (e.g. Expected service KPIs and the Minimum required service KPIs). The S-EES may cache the T-EAS information.

5.	If the request was received from the S-EAS, the S-EES responds to the EAS with the discovered T-EAS Information.

6.	After the S-EAS selects the T-EAS from the discovered T-EAS information received in step 5, the S-EAS declares the selected T-EAS to the S-EES as described in clause 8.8.3.x.

* * * Next Change * * * *

#### 8.8.3.x	Selected T-EAS declaration

Figure 8.8.3.x-1 illustrates the interactions between the S-EAS and the S-EES for the selected T-EAS declaration.

Pre-conditions:

1.	The S-EAS has discovered and selected the T-EAS as described in clause 8.8.3.2.

Figure 8.8.3.x-1: Selected target EAS declaration procedure

1.	The S-EAS sends Selected target EAS declaration request message to the S-EES. The request includes the information of the selected T-EAS.

2.	The S-EES checks whether the requesting EAS is authorized to perform operation. If authorized, the S-EES responds to the received request with Selected target EAS notification declaration response message. The S-EES also determines the selected T-EES based on the declared T-EAS selection, which may be included in the target information notification sent to the EEC as described in clause 8.8.3.5.

Editor's Note:	Updates to reflect/clarify the use of selected T-EAS declaration in scenario 8.8.2.4 are to be provided.

* * * Next Change * * * *

#### 8.8.4.x	Selected target EAS declaration request

Table 8.8.4.x-1 describes information elements for the selected target EAS declaration request sent from the S-EAS to the S-EES.

Table 8.8.4.x-1: Selected target EAS declaration request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | The identifier of the UE. |
| Security credentials | M | Security credentials. |
| Selected EAS ID | M | Selected EAS identifier. |
| Selected EAS Endpoint | M | Endpoint of the selected EAS. |

* * * Next Change * * * *

#### 8.8.4.y	Selected target EAS declaration response

Table 8.8.4.y-1 describes information elements for the Selected target EAS declaration response sent from the S-EES to the S-EAS.

Table 8.8.4.y-1: Selected target EAS declaration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the request was successful. |
| Failure response | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |

* * * Next Change * * * *

#### 8.8.5.1	General

Table 8.8.5.1-1 illustrates the APIs for ACR.

Table 8.8.5.1-1: ACR APIs

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TargetEASDiscovery | Request | Request/Response | EAS, EES |
|  | DeclareSelectedEAS | Request/Response | EAS |
| Eecs_TargetEESDiscovery | Request | Request/Response | EES |
| Eees_AppContextRelocation | Request | Request/Response | EEC |
| Eees_ACREvents | Subscribe | Subscribe/Notify | EEC |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |
| Eees_AutomatedACR | Request | Request/Response | EAS |

##### 8.8.5.2.x	Eees_TargetEASDiscovery_DeclareSelectedEAS operation

API operation name: Eees_TargetEASDiscovery_DeclareSelectedEAS

Description: The consumer declares the selected T-EAS information to the EES.

Inputs: See clause 8.8.4.x.

Outputs: See clause 8.8.4.y.

See clause 8.8.3.x for details of usage of this operation.

* * * End of Change * * * *