3GPP TSG-SA WG6 Meeting #42-e	S6-210876

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

Source:	Samsung

Title:	Pseudo-CR on AC information API

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution provides additional procedures, information flows and API definition for the AC information API.

2. Reason for Change

Incomplete procedures.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0.

* * * First Change * * * *

##### 8.6.4.2.4	Subscription update

Figure 8.6.4.2.4-1 illustrates the AC information subscription update procedure between the EAS and the EES.

Pre-conditions:

1.	The EAS has subscribed for AC information as described in clause 8.6.4.2.2.

Figure 8.6.4.2.4-1: AC information subscription update

1.	The EAS sends an AC information subscription update request to the EES. The request may contain updated filters to retrieve information about particular ACs e.g. AC profile parameters or parameter ranges to be matched, specific UE Identifiers, location ranges, etc.

2.	Upon receiving the request from the EAS, the EES performs an authorization check to verify whether the EAS has authorization to perform the operation. If authorized, the EES updates the stored subscription information.

3.	The EES sends an AC information subscription update response to the EAS.

##### 8.6.4.2.5	Unsubscribe

Figure 8.6.4.2.5-1 illustrates the AC information unsubscribe procedure between the EAS and the EES.

Pre-conditions:

1.	The EAS has subscribed for AC information as described in clause 8.6.4.2.2.

Figure 8.6.4.2.5-1: AC information unsubcribe

1.	The EAS sends an AC information unsubscribe request to the EES.

2.	Upon receiving the request from the EAS, the EES performs an authorization check to verify whether the EAS has authorization to perform the operation. If authorized, the EES cancels the subscription as requested in step 1.

3.	The EES sends an AC information unsubscribe response to the EAS.

* * * Next Change * * * *

##### 8.6.4.3.2	AC information subscription request

Table 8.6.4.3.2-1 describes the information flow for an AC information subscription request from the EAS to the EES.

Table 8.6.4.3.2-1: AC information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Subscription type | M | Choice of "One time", "Event based", determining the type of processing requested |
| Filter | O | List of characteristics for discovery purposes, as detailed in Table 8.6.4.3.2-2. |
| Event conditions | O | Parameters indicating event conditions e.g. number of notifications instances, subscription timeout, etc. |

Table 8.6.4.3.2-2: Filter

| Information element | Status | Description |
| --- | --- | --- |
| AC Type | O | Category or type of AC (e.g. V2X) to be matched |
| ECSP IDs | O | Identifiers for the ECSP associated with the EEC. |
| Operation Schedule | O | EAS operation schedule (e.g. time windows) to be matched |
| Geographical Service Area | O | EAS service area for identifying UEs with matching expected geographical location(s) (e.g. route). This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| Topological Service Area | O | EAS service area for identifying UEs with matching expected topological location(s). See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | EAS KPIs (Table 8.2.5-1) for identifying the EEC and ACs to be matched, |
| AC Profile Parameter Ranges | O | Parameter ranges to be matched to Profiles of ACs for which the request applies. |
| ACID(s) | O | List of identities of AC(s) to be matched |
| UE ID(s) | O | List of UE identifiers to be matched |
| UE location(s) | O | List of locations (e.g. routes) of the UE(s) for which the request applies |

##### 8.6.4.3.3	AC information subscription response

Table 8.6.4.3.3-1 describes the information flow for the AC information subscription response from EAS to the EES.

Table 8.6.4.3.3-1: AC information subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

* * * Next Change * * * *

##### 8.6.4.3.5	AC information subscription update request

Table 8.6.4.3.5-1 describes the information flow for an AC information subscription update request from the EAS to the EES.

Table 8.6.4.3.5-1: AC information subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Subscription ID | M | Subscription identifier corresponding to the subscription. |
| Filter | O | List of characteristics for discovery purposes, as detailed in Table 8.6.4.3.2-2. |
| Event conditions | O | Parameters indicating event conditions e.g. number of notifications instances, subscription timeout, etc. |

##### 8.6.4.3.6	AC information subscription update response

Table 8.6.4.3.6-1 describes the information flow for the AC information subscription update response from EAS to the EES.

Table 8.6.4.3.6-1: AC information subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

##### 8.6.4.3.7	AC information unsubscribe request

Table 8.6.4.3.7-1 describes the information flow for an AC information unsubscribe request from the EAS to the EES.

Table 8.6.4.3.7-1: AC information unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Subscription Identifier | M | Subscription identifier for the corresponding subscription request. |

##### 8.6.4.3.8	AC information unsubscribe response

Table 8.6.4.3.8-1 describes the information flow for the AC information unsubscribe response from EAS to the EES.

Table 8.6.4.3.8-1: AC information unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the unsubscribe request was successful. |
| Failure response | O | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure |

* * * Next Change * * * *

#### 8.6.4.4	APIs

##### 8.6.4.4.1	General

Table 8.6.4.4.1-1 illustrates the API for AC information exposure.

Table 8.6.4.4.1-1: Eees_AppClientInformation API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_AppClientInformation | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

##### 8.6.4.4.2	Eees_AppClientInformation_Subscribe operation

API operation name: Eees_AppClientInformation_subscribe

Description: The consumer subscribes to receive an AC information.

Inputs: See clause 8.6.4.3.2.

Outputs: See clause 8.6.4.3.3.

See clause 8.6.4.2.2 for details of usage of this operation.

##### 8.6.4.4.3	Eees_AppClientInformation_Notify operation

API operation name: Eees_AppClientInformation_Notify

Description: The consumer is notified of an event by the EES.

Inputs: See clause 8.6.4.3.4.

Outputs: None.

See clause 8.6.4.2.3 for details of usage of this operation.

##### 8.6.4.4.4	Eees_AppClientInformation_UpdateSubscription operation

API operation name: Eees_AppClientInformation_UpdateSubscription

Description: The consumer updates an existing subscription for AC information.

Inputs: See clause 8.6.4.3.5.

Outputs: See clause 8.6.4.3.6.

See clause 8.6.4.2.4 for details of usage of this operation.

##### 8.6.4.4.5	Eees_AppClientInformation_Unsubscribe operation

API operation name: Eees_AppClientInformation_Unsubscribe

Description: The consumer unsubscribes for the previously subscribed events.

Inputs: See clause 8.6.4.3.7.

Outputs: See clause 8.6.4.3.8.

See clause 8.6.4.2.5 for details of usage of this operation.