3GPP TSG-SA WG6 Meeting #42-BIS-e	S6-210887

e-meeting, 12th – 20th April 2021

Source:	Huawei, Hisilicon

Title:	Clarification on dynamic EAS instantiation

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Cuili Ge (gecuili@huawei.com)

1. Introduction

The contribution is introduced to clarify on dynamic EAS instantiation.

2. Reason for Change

In clause 8.11, it provides the dynamic EAS instantiation at the EES. The clause currently only capture that the EES can trigger the dynamic EAS instantiation under certain conditions e.g., fail to discover and select the proper EAS that matches the requestor’s requirements.

However, in the real business case, the ASP may not deploy the EAS to all the EDNs especially at the initial phase. It means that not all the applications can be dynamic deployed at each EDN. So the EES has to do some authroization check on which application can be dynamic initantiated at which EDN(s).

This pCR is proposed to clarify on the EES has to do the authroization check on the dynamic EAS instantiation.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0.

* * * First Change * * * *

## 8.11	Dynamic EAS instantiation triggering

### 8.11.1	General

The EES is configured with the list of EAS(s) which can dynamically instantiated. The EES may trigger the EAS instantiation dynamically due to e.g., EAS discovery request, UE mobility.

Upon receiving the EAS discovery request with EAS discovery filter from the EEC or the S-EES during the procedures for EAS discovery or ACR, the EES may fail to discover and select the EAS that matches the UE location and the requesting application characteristics specified in table 8.5.3.2-2 due to no EAS is available or instantiated. The EES checks the configuration if the EES can initiate dynamic instantiation for the EAS and further if allowedtriggers the EAS management system to instantiate the EAS serving the AC in the EDN before returning the EAS information to the EEC or S-EES.

Editor's Note:	[SA5] The triggering procedure is outside the scope of this specification and the reference to the SA5 TS for EAS management system is FFS.

* * * End of Change * * * *