3GPP TSG-SA WG6 Meeting #42-BIS-e	S6-210897

e-meeting, 12th – 20th April 2021

Source:	Huawei, Hisilicon

Title:	Improvement of service provisioning response for EEC selecting EES

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Cuili Ge (gecuili@huawei.com)

1. Introduction

The contribution provides ACID in service provisioning response for EEC selecting EES.

2. Reason for Change

Generally, the EEC has multiple ACs and capability of  EES(s) is different, so the EEC may choose to connect different EES(s) to satisfy the different service KPIs for different AC.

Therefore, the EEC performs service provisioning for multiple ACs, and EEC send service provisioning request to ECS providing AC profile(s), and the ECS returns multiple EES(s) for the EEC.

However, the existing service provisioning response does not return the discovered list of EES(s) corresponding with AC, so the EEC cannot select a suitable EES for each AC.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0.

* * * First Change * * * *

### 8.2.6	EES Profile

The EES profile includes information about the EES and the services it provides.

Table 8.2.6-1: EES Profile

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | The identifier of the EES |
| EES Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EES. This information is provided to the EEC to connect to the EES. |
| EASIDs | M | List of EASIDs registered with the EES. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS. |
| EES Provider Identifier | O | The identifier of the EES Provider (such as ECSP) |
| EES Topological Service Area | O | The list of Cell IDs (or TAIs) serviced by this EES. EECs of UEs that are connected to the Core Network from a cell that is not in this list shall not be served by this EES. See possible formats in Table 8.2.7-1. |
| EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| List of EES/EAS DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |

Editor's Note:	Whether or not to split EAS/EES DNAI (s) is FFS.

* * * Next Change * * * *

- 8.3.3.3.3	Service provisioning response

Table 8.3.3.3.3-1 describes the information elements for service provisioning response from the ECS to the EEC.

Table 8.3.3.3.3.-1: Service provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the service provisioning request was successful. |
| > List of EDN configuration information | M | List of EDNs. |
| >> EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| >>> DNN/APN | M | Data Network Name/Access Point Name |
| >>> S-NSSAI | O | Network Slice information |
| >>> EDN Topological Service Area | O | A list of Cell IDs, a list of TAIs, or a PLMN ID. The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| >>List of EESs | M | List of EESs of the EDN. |
| >>> EESID | M | The identifier of the EES |
| >>> EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| >>> EASIDs (NOTE 2) | O | List of EASIDs registered with the EES. |
| >>>> ACID(s)(NOTE2) | O | Identifies the AC(s) that can be served by the EAS. |
| >>> ECSP info | O | Information for Edge Computing Service Provider |
| >>> EES Topological Service Area | O | The list of Cell IDs (or TAIs) serviced by this EES. EECs of UEs that are connected to the Core Network from a cell that is not in this list shall not be served by this EES. |
| >>> EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| >>> List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| >> Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| Failure response | O | Indicates that the service provisioning request failed. |
| > Cause | O | Indicates the cause of service provisioning request failure. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in 3GPP TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). |  |  |

* * * Next Change * * * *

- 8.3.3.3.6	Service provisioning notification

Table 8.3.3.3.6-1 describes the information elements for service provisioning notification from the ECS to the EEC.

Table 8.3.3.3.6.-1: Service provisioning notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the ECS for the request |
| List of EDN configuration information | M | List of EDNs. |
| > EDN connection information (NOTE 1) | M | Information required by the UE to establish connection with the EDN. |
| >> DNN/APN | M | Data Network Name/Access Point Name |
| >> S-NSSAI | O | Network Slice information |
| >> EDN Topological Service Area | O | A list of Cell IDs, a list of TAIs, or a PLMN ID. The EDN serves UEs that are connected to the Core Network from one of the cells included in this service area. See possible formats in Table 8.2.7-1. |
| >List of EESs | M | List of EESs of the EDN. |
| >> EESID | M | The identifier of the EES |
| >> EES Endpoint | M | The endpoint address (e.g. URI, IP address) of the EES |
| >> EASIDs (NOTE 2) | O | List of EASIDs registered with the EES. |
| >>> ACID(s) | O | Identifies the AC(s) that can be served by the EAS. |
| >> ECSP info | O | Information for Edge Computing Service Provider |
| >> EES Topological Service Area | O | The list of Cell IDs (or TAIs) serviced by this EES. EECs of UEs that are connected to the Core Network from a cell that is not in this list shall not be served by this EES. |
| >> EES Geographical Service Area | O | The area being served by the EES in Geographical values (as specified in clause 7.3.3.3) |
| >> List of EES DNAI(s) | O | DNAI(s) associated with the EES/EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. |
| > Lifetime | O | Time duration for which the EDN configuration information is valid and supposed to be cached in the EEC. |
| NOTE 1: If the UE is provisioned or pre-configured with URSP rules by the HPLMN, the UE handles the precedence between EDN connection info and URSP rules as defined in TS 23.503 [12] clause 6.1.2.2.1. EDN connection info is considered to be part of UE Local Configurations. NOTE 2: EAS information is limited to the EEC requested applications. If no AC profiles were present in the service provisioning request, the EAS information is subject to the ECSP policy (e.g. no EAS information or a subset of EAS information related to the EES). |  |  |

* * * End of Change * * * *