3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210935 rev 1

e-meeting, 12th-20th April 2021	(revision of S6-21xxxx)

Source:	Convida Wireless LLC, Huawei, HiSilicon

Title:	Service continuity planning clarifications

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

In clause 8.8.1 the following EN requests more clarification to be brought to the service continuity planning feature

Editor's Note:	Any potential change required to the pre-conditions and other steps of the ACR procedures to support service continuity planning is FFS. Further, a formal definition of service continuity planning is also FFS.

2. Reason for Change

To provide clarifications for service continuity planning

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558.

* * * First Change * * * *

- 8.1	General

When a UE moves to a new location, different EASs can be more suitable for serving the ACs in the UE. Such transitions can result from a non-mobility event also, requiring support from the enabling layer to maintain the continuity of the service.

This clause describes the features that support service continuity for ACs in the UE to minimize service interruption while replacing the S-EAS, with a T-EAS.

Generally, the S-EAS is associated with an application context. To support service continuity, this application context from the S-EAS is transferred to a T-EAS.

The capabilities for supporting service continuity provided at the Edge Enabler Layer may consider various application layer scenarios in which there may be involvement of AC and one or more EAS(s).

Following intra-EDN, inter-EDN and LADN related scenarios are supported for service continuity:

-	UE mobility, including predictive or expected UE mobility for the following cases:

-	Overload situations in S-EAS or EDN for the following cases:

-	Maintenance aspects such as graceful shutdown of an EAS.

To support the need of ACR, following entity roles are identified:

-	detection entity, detecting or predicting the need of ACR;

-	decision-making entity, deciding that the ACR is required; and

-	execution entity, executing ACR.

A detection entity detects the probable need for ACR by monitoring various aspects, such as UE's location or predicted/expected UE location and indicates to the decision-making entity to determine if the ACR is required. The AC, EEC, EES and EAS can potentially perform the detection role:

A decision-making entity determines that ACR is required and instructs the execution entity to perform ACR.

An execution entity performs ACR as and when instructed by the decision-making entity.

NOTE:	After a decision that another EAS is to serve the UE, the S-EAS can decide if the existing Application Context is transferred to the new EAS.

The EAS may utilize the following capabilities provided by the EES for supporting service continuity at the application layer:

-	Subscribe to service continuity related events and receive corresponding notifications;

-	Fetch the T-EAS; and

-	ACR from a S-EAS to a T-EAS.

The EES can utilize the following capabilities provided by the ECS for supporting service continuity at the application layer:

-	Fetch the T-EES.

The EEC may determine if the ACR is required by detecting that the UE moved or is predicted or expected to move outside the service area (see clause 7.3.3). The service area can be provided to the EEC by either the ECS during Service Provisioning or EES during EAS Discovery. For the PDU Session of SSC mode 3, if the UE receives PDU Session Modification Command as specified in clause 4.3.5.2 of 3GPP TS 23.502 [3], the EEC may determine that the ACR is required. For IPv6 multi-homed PDU Session of SSC mode 3, the EEC may determine that ACR is required if the UE is notified of the existence and availability of a new IPv6 prefix as specified in clause 4.3.5.3 of 3GPP TS 23.502 [3].

NOTE:	For IPv6 multi-homed PDU Session of SSC mode 3, the EEC can be aware of the notification about the IPv6 prefix configuration due to change of PSA UPF based on the UE implementation.

After successful ACR:

-	The EES is informed of the completion by the EAS; and

-	The EEC is informed of the completion by the EES.

In general, a number of steps are required in order to perform the ACR procedure. The potential roles of an edge enablement layer in the ACR procedure include:

-	providing detection events;

-	selecting the T-EAS(s); and

-	supporting the transfer of the Application context from the S-EAS(s) to the T-EAS(s).

If the UE is connected to the 5GC, the EES/EAS acting as AF may utilize AF traffic influence functionality from the 3GPP CN as specified in 3GPP TS 23.502 [3].

A high level overview of ACR is illustrated in Figure 8.8.1-1.

Figure 8.8.1-1: High level overview of ACR procedure

Editor's Note:	Any potential change required to the pre-conditions and other steps of the ACR procedures to support service continuity planning is FFS.

ACR can be performed for service continuity planning, which means that the first three steps of the ACR procedure, detection, decision and execution, are performed for an expected/predicted location of the UE. In such a case the T-EAS is to service the UE when it moves to the expected location.

* * * Next Change * * * *

- 8.8.2.x	ACR with service continuity planning

Service continuity planning is an Edge Enabler Layer value-add feature of providing support for seamless service continuity, when information about planned, projected, or anticipated behaviour is available at EESs or provided by EECs.

Service continuity planning facilitates the application to minimise the service interruption when switching between S-EAS and T-EAS. The ACR detection, decision and execution can be made in advance. If the ACT is performed before UE moves the predicted/expected location, how the application context is relocated with up-to-date information between the S-EAS and T-EAS from the time when UE has not yet moved to the predicted/expected location to the time when UE really moves to the predicted/expected location is out the scope of 3GPP.

To implement this functionality an EES may utilize:

- 	information provided by the EEC e.g. AC Schedule, Expected AC Geographical Service Area, Expected Service KPIs, Preferred ECSP list;

-	3GPP core network capabilities utilized by EES as described in clause 8.9.2

See clauses 8.8.2.2, 8.8.2.3, 8.8.2.4, 8.8.2.5, 8.8.2.6 for the details in ACR with service continuity planning.