3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210745-rev2

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	one2many

Title:	Pseudo-CR on Generic description of the MSGin5G Service

Spec:	3GPP TS 23.554 v0.2.0

Agenda item:	7.9

Document for:	Approval

Contact:	Peter Sanders (peter.sanders@everbridge.com)

1. Introduction

The proposed section gives an overview of the MSGin5G Service and intends to provide a general understanding how the procedures, specified in clause 7, "hang together".

2. Reason for Change

New text.

3. Conclusions

The text in the new clause will need to be aligned when the procedures and service flows are developed. The proposed text is a first step.

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.2.0.

N.B. The figure is a picture and is also included in a editable PPT file.

* * * First Change * * * *

# X	Generic description of the MSGin5G Service (informative)

Editor's Note:	The text in this clause needs to be aligned with the procedures specified below.

## X.1	General

Massive Internet of Things (MIoT) is one of key market segments of 5G. The typical IoT device communication is sending and receiving small data which can be delivered just in a message. The MSGin5G Service is designed and optimized for massive IoT device communication including thing-to-thing communication and person-to-thing communication.

The MSGin5G Service is a message enabler for applications. An Application Client in a UE utilizes MSGin5G Service to send a message to another UE, to multiple UEs or to the Application Server, or the Application Server utilizes the MSGin5G Service to send a message to a UE or to multiple UEs. All messages will be routed via the MSGin5G Server in the 5G system. The MSGin5G service flow is shown in figure X.1-1.

If the UE supports a legacy 3GPP message service (e.g. SMS, NIDD, or CB) and does not support the MSGin5G Service (i.e. UE has no MSGin5G Client), the message will be translated to the appropriate message delivery mechanism by the Legacy 3GPP Message Gateway. A UE that does not support any 3GPP message service can connect to the MSGin5G Service via Non-3GPP Message Gateway that facilitates the translation between the MSGin5G Service and non-3GPP message delivery mechanism. The connection between such UE and the gateway can be via 3GPP access or non 3GPP access (e.g. WLAN) and is out of scope of the present specification.

An Application Server resides outside the 3GPP domain and connects to the MSGin5G Server via a CAPIF-aware reference point.

The message communication models include:

-	point-to-point messaging: message that is originated at a UE (UE A) and terminated at another UE (UE B, a Legacy 3GPP UE or a Non-3GPP UE).

-	application-to-point messaging: message that is originated at an Application Sever and terminated at a UE.

-	point-to-application messaging: message that is originated at a UE and terminated at an Application Sever

-	group messaging: message that is originated at a UE or an Application Server and is terminated at a group of UEs (a group member can be of type UE A, Legacy 3GPP UE or Non-3GPP UE).

-	broadcast messaging: message that is originated at an Application Sever or a UE and broadcasted to all the Legacy 3GPP UEs in a specific Service Area within coverage of a cell or of multiple cells.

-	topic message: message that is originated at an Application Sever or a UE and is delivered to all UEs and Application Server(s) that have subscribed to the topic.

Figure X.1-1: The MSGin5G Service overview

## X.2	Service flow

Before a UE or an Application Server can use the MSGin5G Service it needs to register with the MSGin5G Server.

A UE registers with its identity, its security credentials, the capabilities that it supports, and its availability. The MSGin5G Server will use the availability registration and de-registration to determine if the UE is available for message delivery. If a UE is unavailable for message delivery, the MSGin5G Server will store the message and deliver it once the UE becomes available again.

When a UE or an Application Server sends a MSGin5G message to a recipient UE then such message will be sent to the MSGin5G Server and this server will deliver the message to the UE based on the capabilities of the recipient UE (MSGin5G UE, Legacy 3GPP UE or Non-3GPP UE). If the recipient UE is a Legacy 3GPP UE then the MSGin5G Server will forward the message to the Legacy 3GPP Message Gateway and this gateway will convert the message to a message that is supported by the recipient UE (e.g. SMS, NIDD or CB). If the recipient UE is a Non-3GPP UE then the MSGin5G Server will forward the message to the Non-3GPP Message Gateway and this gateway will convert the message to a message that is supported by the recipient UE.

A Legacy 3GPP UE sends the application payload to the Legacy 3GPP Message Gateway and this gateway will forward the application payload in a MSGin5G Message to the MSGin5G Server, which will deliver the message to the recipient UE(s). A Non-3GPP UE sends the application payload to the Non-3GPP Message Gateway and this gateway will forward the application payload in a MSGin5G Message to the MSGin5G Server, which will deliver the message to the recipient UE(s).

NOTE: Conversion by a Message Gateway is out of scope of the present specification.

If a UE or an Application Server sends a MSGin5G message to a group of UEs, the MSGin5G Server will deliver the message to all group members taking into account if such a UE is a MSGin5G UE, a Legacy-3GPP UE or a non-3GPP UE and if such UE is available for delivery.

If a UE or an Application Server sends a MSGin5G message containing a Messaging Topic to the MSGin5G Server, the MSGin5G Server will distribute the message to all UEs and Application Servers that have subscribed to that topic.

If a UE or an Application Server sends a broadcast message to a Broadcast Area, the MSGin5G Server forwards the message to the Legacy-3GPP Message Gateway and this gateway will forward the message to the broadcast function.

* * * End of Changes * * * *