3GPP TSG-SA WG6 Meeting #42-BIS-e meeting	S6-210814 Rev1

12th – 20th April 2021, Online	(revision of S6-21xxxx)

Source:	China Mobile

Title:	Pesudo-CR on MSGin5G identities

Spec:	3GPP TS 23.554 v0.2.0

Agenda item:	7.9

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

- 1. Introduction

Contribute new text addressing identities in MSGin5G.

- 2. Reason for Change

Contribute new text addressing identities in MSGin5G.

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.2.0.

* * * First Change * * * *

# 6	Identities

6.1	Identities for MSGin5G Service Endpoints

MSGin5G Service endpoints shall be identified by unique identifiers within the MSGin5G service domain. For each MSGin5G Service endpoint the identifier shall be an unique URI that can be used to identify the MSGin5G Service endpoint's home service domain.

The following clauses describe different types if MSGin5G Service endpoint identifiers

### 6.1.1	MSGin5G UE Service Identity (UE Service ID)

UE Service ID is the MSGin5G Service ID of a UE (i.e. MSGin5G UEs, Legacy 3GPP UEs, Non-3GPP UEs).

For a MSGin5G UE, the assigned UE Service ID is used by the MSGin5G Client to register with the MSGin5G Server and to send and receive MSGin5G messages. It is used by the MSGin5G Server to authenticate and authorize the associated UE to the MSGin5G service at the application layer.

For a Legacy 3GPP UE or a Non-3GPP UE, the assigned MSGin5G Service ID is used by the Message Gateway to register with the MSGin5G Server, to map into the Service ID in their defined message delivery mechanisms for interworking, and to send/receive MSGin5G messages to/from MSGin5G Server onbehalf of Legacy 3GPP UE or Non-3GPP UE.

### 6.1.2	Application Server Service Identity (AS Service ID)

AS Service ID is the MSGin5G Service ID of an Application Server. It is used to perform mutual authentication with the MSGin5G Server for establishing a secured service connection, and it is used to send/receive MSGin5G messages to/from other MSGin5G Service Endpoints via MSGin5G Server.

### 6.1.3	Message Gateway Service Identity (MGW Service ID)

MGW Service ID is the MSGin5G Service ID of Message Gateway. It is used to perform mutual authentication with the MSGin5G Server for establishing a secured service connection, and it is used to send/receive MSGin5G Messages to/from MSGin5G Server on behalf of Legacy 3GPP UE or Non-3GPP UE.

## 6.2	MSGin5G Group Service Identity (Group Service ID)

The Group Service ID is a unique identifier within the MSGin5G service that represents a pre-defined MSGin5G group. A pre-defined MSGin5G group is established before the MSGin5G Group messages are sent to it, and is assigned a unique and permanent Group Service ID when it is established. This Group Service ID shall be communicated to all members of the group. A Service Endpoint (UE or Application Server) shall use this Group Service ID to send a message to all members of the group.

The Group Service ID shall be a unique URI that can be used to identify where the group is hosted.

## 6.3	 Broadcast Area Service Identity (Broadcast Area ID)

The Broadcast Area ID is the identity of the Service Area where the broadcast message is delivered.

## 6.4	MSGin5G UE Identity (MSGin5G UE ID)

The MSGin5G UE ID is a unique identifier that represents the MSGin5G UE (i.e. the device identifier of the MSGin5G UE). The MSGin5G UE ID may be pre-configured to the MSGin5G UE by its vendor.

## 6.5	 Application Identity (Application ID)

Application ID is an identifier that represents the application for which the payload of message is intended. The content of Application ID is outside the scope of this document.