3GPP TSG-SA WG6 Meeting #42-BIS-e meeting	S6-210816 Rev2

12th – 20th April 2021, Online	(revision of S6-21xxxx)

Source:	China Mobile

Title:	Pesudo-CR on Non-MSGin5G UE Configuration

Spec:	3GPP TS 23.554 v0.2.0

Agenda item:	7.9

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

- 1. Introduction

Contribute new text addressing Non-MSGin5G UE Configuration in MSGin5G.

- 2. Reason for Change

Contribute new text addressing Non-MSGin5G UE Configuration in MSGin5G.

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.2.0.

* * * First Change * * * *

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

MSGin5G Server: A server in MSGin5G Service that delivers MSGin5G messages among MSGin5G Service endpoints.

Legacy 3GPP Message Gateway: the entity in MSGin5G Service to support interworking with Legacy 3GPP UEs.

Non-3GPP Message Gateway: the entity in MSGin5G Service to support interworking with Non-3GPP UEs.

Message Gateway: general terminology for Legacy 3GPP Message Gateway or Non-3GPP Message Gateway.

MSGin5G Client: the client that enables MSGin5G Message sending and receiving.

MSGin5G UE:  the UE that uses MSGin5G Client in MSGin5G Service.

Legacy 3GPP UE: the UE that supports legacy 3GPP message sending and receiving (e.g. SMS, NIDD, etc) in MSGin5G Service.

Non-3GPP UE: the UE that supports non-3GPP message sending and receiving (e.g. RCS message as specified in GSMA PRD RCC.07 [3], OMA LWM2M message as specified in OMA OMA-ERELD-LightweightM2M [4]) in MSGin5G Service.

NOTE:	The MSGin5G UE utilizes MSGin5G Client in MSGin5G service. The Legacy 3GPP UE and Non-3GPP UE does not utilize MSGin5G Client in MSGin5G service.

MSGin5G Group: the group of UEs which members may be MSGin5G UE, Legacy 3GPP UE and Non-3GPP UE.

Editor's note: Definitions alignment to Architecture is FFS.

Non-MSGin5G UE: general terminology for Legacy 3GPP UE or Non-3GPP UE.

* * * Next Change * * * *

## 8.1.3 Non-MSGin5G UE Configuration

When a Non-MSGin5G UE interacts to the Message Gateway  first time (e.g. a Non-MSGin5G UE sends a message to a MSGin5G UE/Application Server or vice vice versa), the Message Gateway records the Non-MSGin5G UE ID and performs the configuration procedure on behalf of the Non-MSGin5G UE to get the MSGin5G Service configuration information (e.g. MSGin5G Service ID).

In the Non-MSGin5G UE configuration procedure, the Message Gateway acts as Configuration management client specified in 3GPP TS 23.434 [5] on behalf of each Non- MSGin5G UE.

The following steps of configuration management service may apply for the Message Gateway:

-	Send the Get VAL UE configuration request specified in subclause 11.3.2.1 of 3GPP TS 23.434 [5];

-	Receive the related Get VAL UE configuration response specified in subclause 11.3.2.2 of 3GPP TS 23.434 [5];

The usage of the above information flows is clarified as below:

-	An Information Element contains both Non-MSGin5G UE ID and GW service ID works as VAL UE ID which is mandatory in the Get VAL UE configuration request;

NOTE:	The non-MSGin5G UE ID may differ among different VALs and may not unique among different Gateways. It is not enough to use only non-MSGin5G UE ID as VAL UE ID.

-	The UE service ID works as VAL user ID;

-	The service identifier of MSGin5G Service works as VAL service ID;

Besides the IEs specified in subclause 11.3.2.1 of 3GPP TS 23.434 [5], the information in table 8.1.3-1 is also included in the Get VAL UE configuration request.

Table 8.1.3-1: Additional Information in the  Get VAL UE configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Non-MSGin5G UE information | O | Other information needed by the configuration procedure.(NOTE 1) |

NOTE 1:	The information can be the device type, device Vendor, etc. It is specified by application provider or MSGin5G service provider and is out of scope of this document. The MSGin5G service provider can configure the Non-MSGin5G UE with different configuration data based on this IE. E.g. all sensors can be configured to a same MSGin5G Server.

Editor's Note:	 It is recommended that a new generic container IE is needed to be added in subclause 11.3.2.1 of 3GPP TS 23.434 to cover the Non-MSGin5G UE information

-	Besides the IEs specified in subclause 11.3.2.2 of 3GPP TS 23.434 [5]. The information in table 8.1.3-2 is also included in the Get VAL UE configuration response as a part of VAL UE configuration data.

Table 8.1.3-2: Additional information in the Get VAL UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| UE service ID | M | UE service ID assigned to the requesting Non-MSGin5G UE. |
| MSGin5G Server | M | The MSGin5G Server which serves this MSGin5G UE. |
| MSGin5G service specified information | O | The specific information of the MSGin5G Service specified by the MSGin5G Service Provider. (NOTE 2) |

NOTE 2:	e.g. the segment size of MSGin5G Message in this Service Provider, The detailed definition is out of scope of this document.

Besides the functionalities of Configuration Management Server specified in 3GPP TS 23.434 [5], the MSGin5G Configuration Function should also checks whether the MSGin5G UE ID (i.e. VAL UE ID) is included in a former Get VAL UE configuration request.

-	If so, the MSGin5G Configuration Function included the UE Service ID assigned to the MSGin5G UE in the former configuration procedure as a part of VAL UE configuration data,

-	Otherwise a new UE Service ID is assigned to the MSGin5G UE, and included in the VAL UE configuration data.

Then the MSGin5G Configuration Function processes the configuration request according to the service policy.

* * * Next Change * * * *

<Proposed change in revision marks>