3GPP TSG-SA WG6 Meeting #42-BIS-e meeting	S6-210818 Rev1

12th – 20th April 2021, Online	(revision of S6-21xxxx)

Source:	China Mobile

Title:	Pesudo-CR on non-MSGin5G message delivery status report

Spec:	3GPP TS 23.554 v0.2.0

Agenda item:	7.9

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

- 1. Introduction

Contribute new text addressing non-MSGin5G message delivery status report.

- 2. Reason for Change

Contribute new text addressing non-MSGin5G message delivery status report.

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.2.0.

* * * First Change * * * *

### 5.3.4	Message Gateway

A Message Gateway in MSGin5G application architecture provides functionality to deliver MSGin5G messages to non-MSGin5G UEs.

A Message Gateway performs the role of interconnecting two different messaging delivery mechanisms and assure the message integrity between different message delivery mechanisms. A message delivery mechanism comprises the specific set of protocols, procedures and rules.

Functionalities of Message Gateway:

-	Enables seamless delivery of a MSGin5G message between different message delivery mechanisms with integrity;

-	Communicates with the MSGin5G Server using either a MSGin5G Client functionality or similar functions to enable sending and receiving MSGin5G messages;

-	Delivers payload of a MSGin5G message to the non-MSGin5G UE using the specific message delivery mechanism available to that non-MSGin5G UE and vice versa; Performs message sender and receiver addresses conversion according to the two connected message delivery mechanisms and maintain the mapping of the address pair used for a response message delivery;

-	Perform registration and de-registration with the MSGin5G Server on behalf of the non-MSGin5G UEs;

-	Act as a service endpoint to perform message segmentation and reassembly for the non-MSGin5G UEs when needed;

and

-	Performs protocols and non-message payload information conversion according to the service supported by the target UE;

-	Supports the MSGin5G message delivery status report

1.	If application level message delivery status report is not supported by the Non-MSGin5G message delivery mechanisms, based on the information (e.g. response to the message delivery request, transport level information, etc) obtained from the Non-MSGin5G message delivery mechanisms, the Message Gateway fetches the delivery status from the above information and uses it to create a MSGin5G message delivery status report on behalf of non-MSGin5G UE. If the delivery status is failure, also fetch the suitable failure reason from the above information and use it as reason of failure in the MSGin5G message delivery status report.

2.	If application level message delivery status report is supported by the Non-MSGin5G message delivery mechanisms (e.g. RCS specificed in GSMA PRD RCC.07 [3]), translates the application level message delivery status report in the Non-MSGin5G message delivery mechanisms to MSGin5G message delivery status report.

There are two types of Message Gateways used to deliver MSGin5G messages to different UE types: the Legacy 3GPP Message Gateway and the non-3GPP Message Gateway

NOTE:	Implementation of the Legacy 3GPP Message Gateway and the MSGin5G Server together is deployment option that is out of this specification.

* * * Next Change * * * *

### 8.x	MSGin5G messaging procedure on Message Gateway

Figure 8.x-1 shows the MSGin5G messaging procedure on Message Gateway.

Figure8.x-1: MSGin5G messaging procedure on Message Gateway.

1)	The MSGin5G server determines to send the MSGin5G Message request to a non-MSGin5G UE and forwards it to the Message Gateway for delivery. A Delivery Status Required IE is included in the MSGin5G Message request.

2)  The Message Gateway records that delivery report is requested in the message. Then it translates the MSGin5G Message to Non-MSGin5G message (e.g. SMS, RCS message as specified in GSMA PRD RCC.07 [3]) with delivery report requested and finishes the information exchange procedure with non-MSGin5G UE (e.g. sends the non-MSGin5G Message to the non-MSGin5G UE and receives the needed response).

NOTE:	The information exchange procedure between Message Gateway and non-MSGin5G UE is out of scope of this specification.

3)  The Message Gateway checks whether application level message delivery status report is supported by the Non-MSGin5G message delivery mechanisms. If not supported, step 4a will be used; otherwise step 4b-5b will be used..

4a)	Based on the information (e.g. response to the non-MSGin5G message delivery request, transport level information, etc) obtained from the Non-MSGin5G message delivery mechanisms,  the Message Gateway fetches the delivery status from the above information and uses it to create a MSGin5G message delivery status report. If the delivery status is failure, also fetch the suitable failure reason from the above information and use it as reason of failure in the MSGin5G message delivery status report. The Information Elements listed in table 7.1.2-2  is included the MSGin5G message delivery status report.

4b)	A non-MSGin5G application level message delivery status report is received by the Message Gateway.

NOTE:	The procedure of non-MSGin5G application level message delivery status report is out of scope of this specification.

5b)	The Message Gateway translates the non-MSGin5G application level message delivery status report to MSGin5G message delivery report. The Information Elements listed in table7.1.2-2 is also included this MSGin5G message delivery status report, but the Delivery Status and Failure Cause IEs are fetched from the non-MSGin5G application level message delivery status report.

6)	The Message Gateway sends the MSGin5G message delivery report to the MSGin5G Server.