3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210835

e-meeting, 12th – 20th April 2021	(revision of S6-21xxxx)

Source:	Huawei, Hisilicon

Title:	5GMARCH E2E Message delivery procedures

Spec:	3GPP TS 23.554

Agenda item:	0.2.0

Document for:	Approval

Contact:	Hongxia Hao, haohongxia@huawei.com

1. Introduction

This pCR proposes E2E message delivery procedures for point to point messaging.

2. Reason for Change

This is based on current conclusions. It’s necessary to clearly specify how the message is going end to end.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.2.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.262: "Message Service within the 5G System".

[3]	GSMA PRD RCC.07: "RCC.07 Rich Communication Suite 9.0 Advanced Communications Services and Client Specification".

[4]	OMA OMA-ERELD-LightweightM2M-V1_1-20180612-C: "Enabler Release Definition for LightweightM2M".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[x0]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[x1]	3GPP TS 23.502: "Procedures for the 5G System".

[x2]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[x3]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[x4]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[x5]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[x6]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

## 7.D	E2E Message delivery procedures

7.D.1	Point-to-Point Message delivery procedures

#### 7.D.1.1 From MSGin5G UE to MSGin5G UE

Figure 7.D.1.1-1 shows the message delivery procedure from MSGin5G UE-1 to MSGin5G UE-2.

Pre-conditions:

1.	Both MSGin5G Client-1 in MSGin5G UE-1 and MSGin5G Client-2 in MSGin5G UE-2 registered with the MSGin5G server.

Figure 7.D.1.1-1 Message delivery between MSGin5G UEs

1)	The MSGin5G Client 1 sends a MSGin5G Message Request to MSGin5G Server as specified in clause 7.1.2 with following clarifications:

a)	The MSGin5G Message Request includes Originating MSGin5G Service ID, Recipient MSGin5G Service ID and Message ID information elements in Table 7.1.2-1, and may include Delivery Status Required, Application ID, Payload, Priority type information elements from Table 7.1.2-1.

b)	Upon receiving the MSGin5G Message Request, the MSGin5G Server determines the recipient is a MSGin5G UE and the MSGin5G Client 1 is authorized to send message to MSGin5G Client 2.

2)	The MSGin5G Server forwards the MSGin5G Message Request to MSGin5G Client 2 as specified in clause 7.1.3.

3)	If Delivery Status Required is included in the MSGin5G Message Request, MSGin5G Client 2 sends delivery status report to the MSGin5G Client 1.

#### 7.D.1.2 From MSGin5G UE to Legacy 3GPP UE

Figure 7.D.1.2-1 shows the message delivery procedure from MSGin5G UE to Legacy 3GPP UE.

Pre-conditions:

1)	MSGin5G Client in MSGin5G UE is registered with the MSGin5G Server.

2)	Legacy 3GPP Message Gateway is aware of the MSGin5G Service ID of Legacy 3GPP Message UE and maintains the mapping to IDs used in the legacy network.

3)	The MSGin5G Server is able to determine whether a terminating UE is a Legacy 3GPP UE, and which message delivery mechanisms are available.

Figure 7.D.1.2-1 Message delivery from MSGin5G UE to Legacy 3GPP UE

1)	The MSGin5G Client 1 sends a MSGin5G Message Request to MSGin5G Server as specified in clause 7.1.2 with following clarifications:

a)	The MSGin5G Message Request includes Originating MSGin5G Service ID, Recipient MSGin5G Service ID and Message ID information elements in Table 7.1.2-1, and may include Delivery Status Required, Application ID, Payload, Priority type information elements from Table 7.1.2-1.

2)	Upon receiving the MSGin5G Message Request, the MSGin5G Server determines the recipient is a Legacy 3GPP UE and the MSGin5G Client 1 is authorized to send message to the Legacy 3GPP UE.

3) The MSGin5G Server forwards the MSGin5G Message Request to Legacy 3GPP Gateway as specified in 7.1.3.

4) The Legacy 3GPP Message Gateway determines which legacy 3GPP message delivery mechanism (e.g. SMS, NIDD, Device triggering, etc.) will be used based on Legacy 3GPP UE capability, the UE communication status, the MSGin5G service configuration, etc. When selected, the Legacy 3GPP Message Gateway maps the MSGin5G service ID to the corresponding identifier. For example (not an exhaustive list):

i.		if the Legacy 3GPP Message Gateway selected the device triggering delivery mechanism, it maps the service ID to MSISDN and Application port ID

ii.		if the Legacy 3GPP Message Gateway selected the NIDD delivery mechanism, it maps the service ID to External Identifier or MSISDN.

iii	if the Legacy 3GPP Message Gateway selected the SMS delivery mechanism, it maps the service ID to  MSISDN.

4-6) The Legacy 3GPP Message Gateway sends the payload of the MSGin5G message to the terminating Legacy 3GPP UE using the determined delivery mechanism. For example:

i.	For Device triggering delivery mechanism, the Legacy 3GPP Message Gateway interacts with the MTC-IWF/SCEF/NEF and maps the payload of the MSGin5G message to one or more Device Triggering requests. The MTC-IWF/SCEF/NEF interacts with SMS-SC for delivery to the UE and to receive the Message Delivery Report (see TS 23.682 [x2] clause 5.2, TS 29.122 [x4] clause 4.4.6, TS 29.522 [x5] clause 4.4.3)

ii.	For NIDD delivery mechanism, the Legacy 3GPP Message Gateway may interact with the SCEF/NEF and maps the payload of the MSGin5G message to one or more NIDD submit request messages. The Reliable Data Service Configuration, Maximum Latency, Priority, PDN Connection Establishment Option settings are based on pre-configurations (see TS 23.682 [x2] clause 5.13, TS 29.122 [x4] clause 4.4.5.3, TS 29.522 [x5] clause 4.4.12.3). Alternatively, if tunnel parameters are provisioned in the Legacy 3GPP Message Gateway and UPF/P-GW the payload could be tunnelled via the UPF/P-GW (see 23.401 [x6] (clause 4.3.17.8.3.3) , 23.501[x0] clause 5.6.10.3, 23.502 [x1] clause  4.24);

iii.	For SMS delivery mechanism, the Legacy 3GPP Message Gateway sends SMS to the Legacy 3GPP UE through the SMSC according the procedure in TS 23.204 [x3] or the procedure in clause 4.13.3 of TS 23.502[x1].

7-8) If delivery report is requested in step 2, the Legacy 3GPP Message Gateway sends MSGin5G message delivery report to the MSGin5G Server, the MSGin5G Server sends the delivery report to the MSGin5G Client as specified in 7.1.2.

7.D.1.3 From MSGin5G UE to Non-3GPP UE

Figure 7.D.1.3-1 shows the message delivery procedure from MSGin5G UE to Non-3GPP UE.

Pre-conditions:

1.	MSGin5G client in MSGin5G UE is registered with the MSGin5G server.

2.	Non-3GPP message gateway is aware of the non-3GPP message client in non-3GPP UE and provides the mapping to MSGin5G service ID.

Figure 7.D.1.3-1 Message Delivery from MSGin5G UE to Non-3GPP UE

1)	The MSGin5G Client sends a message to the MSGin5G Server as specified in 7.1.2.

2)	The MSGin5G Server determines the recipient is a Non-3GPP UE and the MSGin5G client is authorized to send message to the Non-3GPP UE.

3)	The MSGin5G server forwards the MSGin5G Message to the Non-3GPP Message Gateway as specified in 7.3.1.

4)	The Non-3GPP Message Gateway translates the MSGin5G Message to the Non-3GPP message with delivery report requested and sends it to the Non-3GPP Message Client. This step is outside the scope of the current specification.

5)	If delivery status report is required, the Non-3GPP Message Gateway sends the MSGin5G message delivery report to the MSGin5G server, the MSGin5G Server forwards the MSGin5G message delivery report to the MSGin5G client.

#### 7.D.1.4 From Legacy 3GPP UE to MSGin5G UE

This procedure is used for message reply from Legacy 3GPP UE (i.e. SMS UE) to MSGin5G UE.

Figure 7.D.1.4-1 shows the message delivery procedure from Legacy 3GPP UE (i.e. SMS UE) to MSGin5G UE.

Pre-conditions:

1)	MSGin5G Client in MSGin5G UE and SMS Client in SMS UE has registered with the MSGin5G server.

2)	The SMS UE received a message from the MSGin5G UE.

3)	The Legacy 3GPP Message Gateway is aware of the SMS Client and provides the mapping to MSGin5G Service ID.

4)	The SMS UE replies to the MSGin5G UE upon receiving the message from the MSGin5G UE.

Figure 7.D.1.4-1: SMS UE replies to MSGin5G UE

1)	The SMS UE sends an SMS message request to the Legacy 3GPP Message Gateway through SMSC according the procedure in 3GPP TS 23.204 [x3] or the procedure in clause 4.13.3 of TS 23.502 [x1]. The delivery report is requested in the message.

NOTE:	If the delivery report is not requested this procedure will stop after step 3 and step 4 to 6 will not be needed.

2)	The Legacy 3GPP Message Gateway translates the SMS message to MSGin5G message with delivery report requested and sends a MSGin5G message request to the MSGin5G Server as specified in 7.1.2.

3)	The MSGin5G Server forwards the MSGin5G message request to the target MSGin5G Client as specified in 7.1.3.

4)	The MSGin5G Client in MSGin5G UE sends an MSGin5G message delivery report to the MSGin5G Server.

5)	The MSGin5G Server forwards the MSGin5G message delivery report to the Legacy 3GPP Message Gateway.

6)	The Legacy 3GPP Message Gateway translates the MSGin5G message delivery report to a SMS message delivery report and sends it to the SMS Client through SMSC.

#### 7.D.1.5 From Non-3GPP UE to MSGin5G UE

This procedure is used for message reply from Non-3GPP UE to MSGin5G UE.

Figure 7.D.1.5-1 shows the message delivery procedure from Non-3GPP UE to MSGin5G UE.

Pre-conditions:

1)	MSGin5G client in MSGin5G UE and Non-3GPP UE has registered with the MSGin5G server.

Editor's Note: The Non-3GPP UE registration procedure is FFS.

2)	The Non-3GPP UE received a message from the MSGin5G UE.

3)	The Non-3GPP Message Gateway is aware of the Non-3GPP message client on the Non-3GPP UE and provides the mapping to MSGin5G service ID.

4)	The Non-3GPP UE replies to the MSGin5G UE upon receiving the message from the MSGin5G UE.

Figure 7.D.1.5-1: Non-3GPP UE replies to MSGin5G UE

1)	The Non-3GPP message client in Non-3GPP UE sends a Non-3GPP message request to the Non-3GPP Message Gateway. The delivery report is requested in the message.

NOTE:	If the delivery report is not requested this procedure will stop after step 3 and step 4 to 6 will not be needed.

2)	The Non-3GPP Message Gateway translates the Non-3GPP message to MSGin5G message with delivery report requested and sends a MSGin5G message request to the MSGin5G Server as specified in 7.1.2.

3)	The MSGin5G Server forwards the MSGin5G message request to the target MSGin5G Client as specified in 7.1.3.

4)	The MSGin5G Client in MSGin5G UE sends an MSGin5G message delivery report to the MSGin5G Server.

5)	The MSGin5G Server forwards the MSGin5G message delivery report to the Non-3GPP Message Gateway.

6)	The Non-3GPP Message Gateway translates the MSGin5G message delivery report to a Non-3GPP message delivery report and sends it to the Non-3GPP message client.

* * * Next Change * * * *

### 7.D.2	Application-to-Point Message delivery procedures

#### 7.D.2.1 From Application Server to MSGin5G UE

Figure 7.D.2.1-1 shows the message delivery procedure from Application Server to MSGin5G UE.

Pre-conditions:

1.	The MSGin5G client has registered with the MSGin5G server.

2.	The Application server has established a secured communication with the MSGin5G server

Figure 7.D.2.1-1 Message delivery from Application server to MSGin5G UE

1)	The Application Server sends API Request to MSGin5G Server for delivery MSGin5G Message as specified in clause 7.1.2

2)	The MSGin5G Server sends MSGin5G Message Request to MSGin5G Client 2 as specified in clause 7.1.3.

3)	If Delivery Status Required is included in the MSGin5G Message Request, MSGin5G Client 2 sends delivery status report to the MSGin5G Server.

4)  The MSGin5G Server sends the delivery status to the AS as specified in 7.1.2

#### 7.D.2.2 From Application Server to Legacy 3GPP UE

Figure 7.D.2.2-1 shows the message delivery procedure from Application Server to Legacy 3GPP UE.

Pre-conditions:

1.	The Application server has established a secured communication with the MSGin5G server

2.	Legacy 3GPP Message Gateway is aware of the legacy 3GPP message client (i.e. SMS client) in legacy 3GPP UE and provides the mapping to MSGin5G service ID.

Figure 7.D.2.2-1 MSGin5G UE to Legacy 3GPP UE messaging

1)	The AS sends a API Request to MSGin5G Server for sending MSGin5G Message as specified in clause 7.1.2.

2)	Same with step 2 in 7.D.1.2.

3) The MSGin5G Server sends MSGin5G Message Request to Legacy 3GPP Gateway as specified in 7.1.3.

4-8) Same with step 4-7 in 7.D.1.2

9) The MSGin5G Server sends the delivery report to the AS as specified in 7.1.2.

#### 7.D.2.3 From Application Server to Non-3GPP UE

Figure 7.D.2.3-1 shows the message delivery procedure from Application Server to Non-3GPP UE.

Pre-conditions:

1.	The Application server has established a secured communication with the MSGin5G server

2.	Non-3GPP Message Gateway is aware of the non-3GPP message client in non-3GPP UE and provides the mapping to MSGin5G service ID.

Figure 7.D.2.3-1 Application server to Non-3GPP UE messaging

1)	The Application Serve sends a message to the MSGin5G Server as specified in 7.1.2.

2)	The MSGin5G Server determines the recipient is a Non-3GPP UE and the MSGin5G client is authorized to send message to the Non-3GPP UE.

3)	The MSGin5G server forwards the MSGin5G Message to the Non-3GPP Message Gateway as specified in 7.3.1.

4)	The Non-3GPP Message Gateway translates the MSGin5G Message to the Non-3GPP message with delivery report requested and sends it to the Non-3GPP Message Client. This step is outside the scope of the current specification.

5)	If delivery status report is required, the Non-3GPP Message Gateway sends the MSGin5G message delivery report to the MSGin5G server, the MSGin5G Server forwards the delivery report to the AS as specified in 7.1.2.

* * * Next Change * * * *

### 7.D.3	Point-to-Application Message delivery procedures

#### 7.D.3.1 From MSGin5G UE to Application Server

Figure 7.D.3.1-1 shows the message delivery procedure from MSGin5G UE to Application Server.

Pre-conditions:

1)	The APP Server and MSGin5G Client in MSGin5G UE has registered with the MSGin5G server.

Figure 7.D.3.1-1: Message delivery from MSGin5G UE to App Server

1)	The MSGin5G Client 1 sends a MSGin5G Message Request to MSGin5G Server as specified in clause 7.1.2 .

2)	The MSGin5G Server forwards the MSGin5G Message to AS as specified in clause 7.1.3.

3)	If Delivery Status Required is included in the MSGin5G Message Request, AS sends delivery status report to the MSGin5G Server.

4)	The MSGin5G Server forwards the delivery status to MSGin5G Client as specified in 7.1.2.

#### 7.D.3.2 From Legacy 3GPP UE to Application Server

This procedure is used for message reply from Non-3GPP UE to Application Server.

Figure 7.D.3.2-1 shows the message delivery procedure from Legacy 3GPP UE to Application Server.

Pre-conditions:

1)	APP Server and SMS Client in SMS UE has registered with the MSGin5G server.

2)	The SMS UE received a message from the App Server.

3)	The Legacy 3GPP message gateway is aware of the SMS client and provides the mapping to MSGin5G service ID.

4)	The SMS UE replies to the MSGin5G UE upon receiving the message from the APP Server.

Figure 7.D.3.2-1: SMS UE replies to Application Server

1)	The SMS UE sends an SMS message request to the Legacy 3GPP Message Gateway through SMSC according the procedure in 3GPP TS 23.204 [x3] or the procedure in clause 4.13.3 of TS 23.502 [x1]. The delivery report is requested in the message.

NOTE:	If the delivery report is not requested this procedure will stop after step 3 and step 4 to 6 will not be needed.

2)	The Legacy 3GPP Message Gateway translates the SMS message to MSGin5G message with delivery report requested and sends a MSGin5G message request to the MSGin5G Server as specified in 7.1.2.

3)	The MSGin5G Server forwards the MSGin5G message to Application Server as specified in 7.1.3.

4)	The AS sends delivery status report by API request to MSGin5G Server.

5)	The MSGin5G Server forwards the delivery status report to the Legacy 3GPP Message Gateway.

6)	The Legacy 3GPP Message Gateway translates the MSGin5G message delivery report to a SMS message delivery report and sends it to the SMS Client through SMSC.

#### 7.D.3.3 From Non-3GPP UE to Application Server

This procedure is used for message reply from Legacy 3GPP UE to Application Server.

Figure 7.D.3.3-1 shows the message delivery procedure from Non-3GPP UE to Application Server.

Pre-conditions:

1)	APP Server and Non-3GPP UE has registered with the MSGin5G server.

2)	The Non-3GPP UE received a message from the App Server.

3)	The Non-3GPP Message Gateway is aware of the Non-3GPP message client on the Non-3GPP UE and provides the mapping to MSGin5G service ID.

4)	The Non-3GPP UE replies to the App Server upon receiving the message from the App Server.

Figure 7.D.3.3-1: Non-3GPP UE replies to Application Server

1)	The Non-3GPP UE sends an Non-3GPP message request to the Non-3GPP Message Gateway

2)	The Non-3GPP Message Gateway translates the Non-3GPP Message to MSGin5G Message with delivery report requested and sends a MSGin5G message request to the MSGin5G Server as specified in 7.1.2.

3)	The MSGin5G Server forwards the MSGin5G message to Application Server as specified in 7.1.3.

4)	The AS sends delivery status report by API request to MSGin5G Server.

5)	The MSGin5G Server forwards the delivery status report to the Non-3GPP Message Gateway as specified in 7.1.2.

6)	The Non-3GPP Message Gateway translates the MSGin5G message delivery report to a Non-3GPP message delivery report and sends it to the Non-3GPP Message Client through SMSC.