3GPP TSG-SA WG6 Meeting #42-bis-e	S6-210930 rev1

e-meeting, 12th – 20th  April 2021	(revision of S6-210xxx)

Source:	Convida Wireless LLC

Title:	5GMARCH store and forward

Spec:	3GPP TS 23.554 v0.2.0

Agenda item:

Document for:	Approval

Contact:	Mladin.Catalina@convidawireless.com

1. Introduction

This contribution merges in the TR solution 21 on store-and-forward

2. Reason for Change

To enable the MSGin5G Service to provide store-and-forward monitoring capabilities in the 3GPP Network.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.2.0

* * * First Change * * * *

- 7.1.2	MSGin5G inbound messages into the MSGin5G Server

Figure 7.1.2-1 shows the procedure for a MSGin5G UE that initiates a MSGin5G message request.

Figure 7.1.2-1: New MSGin5G Message Request from UE

Figure 7.1.2-2 shows the procedure for an Application Server that initiates an API request for sending a MSGin5G Message to UE.

Editor's note:	The APIs provided by MSGin5G Server to App Server is to be specified in another section of the TS.

Figure 7.1.2-2: Application Server initiates a request for sending a MSGin5G message

Figure 7.1.2-3 shows the procedure for a Legacy 3GPP Message Gateway or a non-3GPP Message Gateway that sends a new MSGin5G message request to the MSGin5G Server on behalf of a Legacy 3GPP UE or Non-3GPP UE.

Figure 7.1.2-3: New MSGin5G Message Request sending from Message Gateway

The following procedure applies to the above figures 7.1.2-1, 7.1.2-2 and 7.1.2-3 with the exception that step 1 only applies to figure 7.1.2-1.

1.	The Application Client-1 in the UE sends a request to the MSGin5G Client for invoking the MSGin5G Client to send a new MSGin5G Message to a recipient or to multiple recipients.

Editor's note:	The APIs provided by the MSGin5G Client to the Application Client is to be specified in another section of the TS.

2.	As shown in figure 7.1.2-1 or 7.1.2-3, the MSGin5G Client or Message Gateway sends the MSGin5G Message Request to the MSGin5G Server and includes the IEs as listed in table 7.1.2-1 in the request; or as shown in figure 7.1.2-2, the Application Server sends an API request to the MSGin5G Server for sending a MSGin5G message, the API request includes the IEs as listed in table 7.1.2-1.

Table 7.1.2-1: Request to MSGin5G Server for sending MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Originating MSGin5G Service ID | M | The service identity of the sending MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |
| Recipient MSGSin5G Service ID | O | The service identity of the receiving MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server. This IE is mandatory for point-to-point messaging, application-to-point messaging, AOMT messaging and MOAT messaging and is not present in other message scenarios. |
| Recipient Group ID | O | The identifier of the target MSGin5G Group. This IE is mandatory for a Group Message and is not present in other message scenarios. |
| Recipient Service Area ID | O | The identifier of the Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. |
| Messaging Topic | O | Indicates which messaging topic this message is related to. This IE is mandatory for a message distribution based on topic and is not present in other message scenarios. |
| Application ID | O | Identifies the application for which the payload is intended. This IE is required when the message is sent to one of multiple Application Clients served by same MSGin5G Client. This IE may be included when the message is sent to an Application Server or to an Application Client. MSGin5G Server is unaware of the content. |
| Message ID | M | Unique identifier of this message. If the request is a Delivery Report (as shown in step 6) the Message ID IE is copied from the MSGin5G message that is being acknowledged. |
| Security Credentials | O | Security information required by the MSGin5G Server. This is a placeholder for SA3 security information. |
| Delivery Status Required | O | Indicates if delivery acknowledgement from the recipient is requested. This IE is not present if the MSGin5G message request is a Delivery Report (as shown in step 6). |
| Failure Cause | O | If the request is a Delivery Report (as shown in step 6) the Failure Cause indicates the failure reason, if applicable. |
| Payload | O | Payload of the message. MSGin5G Server/Client is unaware of the content. If the Request is a Delivery Report (as shown in step 6) the Payload IE is not included. The maximum size of this IE is a configurable value that shall not exceed 2048 octets. |
| Priority Type | O | Application priority level requested for this message If the Request is a Delivery Report (as shown in step 6) the Priority Type IE is copied from the message that is being acknowledged. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. |
| Message segment number | O | The message segment number for the segmented message |
| Store-and-forward indicator | M | Boolean indicating whether store-and-forward services are requested (NOTE 1) |
| Storage information | O | Present if store-and-forward services are requested via the Store-and-forward indicator. The value indicates when the message should be discarded, if stored (NOTE 1) |
| NOTE 1: MSGin5G Service Provider policies may indicate whether the store-and-forward feature is enabled or not. If the feature is disabled, this information element is ignored, if present. |  |  |

3.	The MSGin5G Server determines if the recipients are available to receive the message or if the message needs to be stored for deferred delivery.

4.	If the message needs to be stored for deferred delivery, the MSGin5G Server stores the message request with the relevant delivery information.

5.	The MSGin5G Server may return a response to the originating entity and includes the IEs as listed in table 7.1.2-2 in the response.

- Table 7.1.2-2: Information Elements in Response

| Information element | Status | Description |
| --- | --- | --- |
| Originating MSGin5G service ID | M | The identity of the MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the identity of the Application Server that initiated the previous Request. |
| Message ID | M | Identifier of the initiating Request. |
| Delivery Status | O | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |
| Failure Cause | O | The reason for failure |