3GPP TSG-SA WG6 Meeting #41-bis-e	S6-210834

e-meeting, 12th – 20th April 2021	()

Source:	Intel, Samsung, Convida Wireless, AT&T, Verizon, Qualcomm, InterDigital, Apple, Deutsche Telekom, Ericsson, Softil, ETRI

Title:	Architecture - modification to SBA figure

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Danny Moses <danny.moses@intel.com>

1. Introduction

This pCR re-introduces EDGEAPP SBA to the architecture clause 6.2.

2. Reason for Change

The motivation for this change is to include a service based representation of EDGEAPP architecture.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0.

* * * First Change * * * *

## 6.2	Architecture

This clause describes the architecture for enabling edge applications in the following representations:

-	A service-based representation, where the Edge Enabler Layer functions (e.g. ECS) enable other authorized Edge Enabler Layer functions (e.g. EES) to access their services;

-	A service-based representation as specified in 3GPP TS 23.501 [2], where the Network Functions (e.g. NEF) enable authorized Edge Enabler Layer functions (e.g. ECS), to access their services;

-	A service-based representation, where the Core Network Northbound APIs as specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], are utilized by authorized Edge Enabler Layer functions via the CAPIF core function as specified in 3GPP TS 23.222 [6]; and

-	A reference point representation, where existing interactions between any two functions (e.g. EES, ECS) is shown by an appropriate point-to-point reference point (e.g. EDGE-6, EDGE-7).

Edge Enabler Layer functions shown in the service-based representation of the edge architecture shall only use service-based interfaces for their interactions.

Figure 6.2-1 illustrates the service based representation of the architecture for enabling edge applications

Figure 6.2-1: Architecture for enabling edge applications - service-based representation

NOTE 1:	The EEC function and EAS function in figure 6.2-1 do not expose any service to the other functions.

The mechanisms for service discovery in the SBA representation by figure 6.2-1 are as follows:

-	The EES discovers the ECS via pre-configuration or by using CAPIF as specified in 3GPP TS 23.222 [6]

-	The EAS discovers the EES by means which are out of scope of this specification

-	The EAS discovers the other EAS(s) as specified in clause 8.8.3.2

-	The EEC discovers the ECS as specified in clause 8.3.2

-	The ECS discovers the EES via service provisioning as specified in clause 8.3.3

Figure 6.2-2 illustrates the service-based representation of utilizing of the 5GS network services based on 5GS SBA as specified in 3GPP TS 23.501 [2]

Figure 6.2-2: Utilization of 5GS network services based on 5GS SBA – service based representation

The ECS, EES and EAS act as AFs for consuming network services directly from the 3GPP 5G Core Network entities over the service based architecture specified in 3GPP TS 23.501 [2]

The ECS, EES and EAS can simultaneously consume network services from the 5GS SBA as represented in figure 6.2-2 while interacting in the SBA representation in figure 6.2-1

Figure 6.2-3 illustrates the service-based representation for utilizing the Core Network (5GC, EPS) northbound APIs via CAPIF.

Figure 6.2-3: Utilization of Core Network Northbound APIs via CAPIF – service based representation

The ECS, EES and EAS act as authorized API invokers to consume services from the Core Network (5GC, EPS) northbound API entities such as SCEF, NEF SCEF+NEF which act as API Exposing Functions as specified in 3GPP TS 23.222 [6]

The mechanism for northbound APIs discovery in the SBA represented in figure 6.2-3 is as specified in 3GPP TS 23.222 [6]

The ECS, EES and EAS can simultaneously consume Core Network northbound APIs via CAPIF as represented in figure 6.2-3 while interacting in the SBA represented in figure 6.2-1

The Figure 6.2-4 illustrates the reference point representation of the architecture for enabling edge applications.

Figure 6.2-4: Architecture for enabling edge applications – reference points representation

The EDN is a local Data Network. EAS(s) and the EES are contained within the EDN. The ECS provides configurations related to the EES, including details of the EDN hosting the EES. The UE contains AC(s) and the EEC. The EAS(s), the EES and the ECS can interact with the 3GPP Core Network.