3GPP TSG-SA WG6 Meeting #42-BIS-e meeting	S6-210985 Rev1

12th – 20th April 2021, Online	(revision of S6-210815 Rev3)

Source:	China Mobile

Title:	Pesudo-CR on MSGin5G UE Configuration

Spec:	3GPP TS 23.554 v0.2.0

Agenda item:	7.9

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

- 1. Introduction

Contribute new text addressing MSGin5G UE Configuration in MSGin5G.

- 2. Reason for Change

Contribute new text addressing MSGin5G UE Configuration in MSGin5G.

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v0.2.0.

* * * First Change * * * *

## 8.1	Configuration

### 8.1.1	General

The configuration procedure is used to get the MSGin5G Service configuration information (e.g. UE Service ID), It is used by the MSGin5G UE or used by Message Gateway on behalf of the non-MSGin5G UE. The MSGin5G Service configuration information is used in the future messaging communication.

The VAL UE configuration data specified in TS 23.434 [5] is used in these configuration procedures. After the configuration procedure, the MSGin5G UE, or the Message Gateway on behalf of the Non-MSGin5G UE can register to MSGin5G Server.

NOTE:  The configuration on the Message Gateway to support the non-MSGin5G UE for MSGin5G service can also be done without using the SEAL configuration procedures and is implementation specific.

Editor's Note:	 Alternative configuration procedures are FFS.

### 8.1.2	MSGin5G UE Configuration

In the MSGin5G UE configuration procedure, the MSGin5G UE acts as Configuration management client specified in 3GPP TS 23.434 [5].

The following steps of configuration management service apply for the MSGin5G UE:

-	Send the Get VAL UE configuration request specified in subclause 11.3.2.1 of 3GPP TS 23.434 [5];

-	Receive the related Get VAL UE configuration response specified in subclause 11.3.2.2 of 3GPP TS 23.434 [5];

The usage of the above information flows is clarified as below:

-	The MSGin5G UE ID works as VAL UE ID which is mandatory in the Get VAL UE configuration request;

-	The UE service ID works as VAL user ID;

-	The service identifier of MSGin5G Service works as VAL service ID;

Besides the IEs specified in subclause 11.3.2.1 of 3GPP TS 23.434 [5], the information in table 8.1.2-1 is also included in the Get VAL UE configuration request.

Table 8.1.2-1: Additional Information in the  Get VAL UE configuration request

| Information element | Status | Description |
| --- | --- | --- |
| MSGin5G UE information | O | Other information needed by the configuration procedure.(NOTE 1) |

NOTE 1:	The information can be the device type, device Vendor, etc. It is specified by application provider or MSGin5G service provider and is out of scope of this document. The MSGin5G service provider can configure the MSGin5G UE with different configuration data based on this IE. E.g. all sensors can be configured to a same MSGin5G Server.

Editor's Note:	 It is recommended that a new generic container IE is needed to be added in subclause 11.3.2.1 of 3GPP TS 23.434 to cover the MSGin5G UE information

The information in table 8.1.2-2 is included in the Get VAL UE configuration response as a part of VAL UE configuration data.

Table 8.1.2-2: Information in the Get VAL UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| UE service ID | M | UE service ID assigned to the requesting MSGin5G UE. |
| MSGin5G Server | M | The MSGin5G Server which serves this MSGin5G UE. |
| MSGin5G service specified information | O | The specific information of the MSGin5G Service specified by the MSGin5G Service Provider. (NOTE 2) |

NOTE 2:	e.g. the segment size of MSGin5G Message in this Service Provider, The detailed definition is out of scope of this document.

Besides the functionalities of Configuration Management Server specified in 3GPP TS 23.434 [5], the MSGin5G Configuration Function should also checks whether the MSGin5G UE ID (i.e. VAL UE ID) is included in a former Get VAL UE configuration request.

-	If so, the MSGin5G Configuration Function included the UE Service ID assigned to the MSGin5G UE in the former configuration procedure as a part of VAL UE configuration data,

-	Otherwise a new UE Service ID is assigned to the MSGin5G UE, and included in the VAL UE configuration data.

Then the MSGin5G Configuration Function processes the configuration request according to the service policy.

* * * Next Change * * * *

<Proposed change in revision marks>