3GPP TSG-SA WG6 Meeting #42-bis-e	S6-211006r1

e-meeting, 12th – 20th April 2021	(revision of S6-210858, S6-211006)

Source:	InterDigital

Title:	Pseudo-CR on removing duplicated tables on EAS Discovery Request

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Atle Monrad <atle.monrad@interdigital.com>

1. Introduction

This PCR proposes to remove duplication of EAS Discovery Request & fix an invalid reference.

2. Reason for Change

Clauses 8.8.4.2 & 8.5.3.2 both define “EAS Discovery Request”

- 8.5.3.2 from EEC to EES

- 8.8.4.2 from EAS to EES or S-EES to T-EES (extra optional parameters)

It is proposed to have a single EAS Discovery Request definition that covers both.

Clause 8.8.4.2 refers to inexistent table 8.8.4.2-2

It is proposed to refer to Table 8.5.3.2-2: EAS discovery filters.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0.

* * * * First Change * * * *

#### 8.5.3.2	EAS discovery request

Table 8.5.3.2-1 describes information elements for the EAS discovery request. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element.

Table 8.5.3.2-1: EAS discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The ID of the requestor (e.g. EECID) |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| Target DNAI (NOTE) | O | Target DNAI information which can be associated with potential T-EAS(s) |
| NOTE: This IE shall be included only when the request originates from the EAS or the EES. |  |  |

Table 8.5.3.2-2: EAS discovery filters

| Information element | Status | Description |
| --- | --- | --- |
| List of AC characteristics (NOTE 1) | O | Describes the ACs for which a matching EAS is needed. |
| > AC profile (NOTE 2) | M | AC profile containing parameters used to determine matching EAS. AC profiles are further described in Table 8.2.2-1. |
| List of EAS characteristics (NOTE 1, NOTE 3) | O | Describes the characteristic of required EASs. |
| > EASID | O | Identifier of the required EAS. |
| > EAS provider identifier | O | Identifier of the required EAS provider |
| > EAS type | O | The category or type of required EAS (e.g. V2X) |
| > EAS schedule | O | Required availability schedule of the EAS (e.g. time windows) |
| > EAS Geographical Service Area | O | Location(s) (e.g. geographical area, route) where the EAS service should be available. |
| > EAS Topological Service Area | O | Topological area (e.g. cell ID, TAI) for which the EAS service should be available. See possible formats in Table 8.2.7-1. |
| > Service continuity support | O | Indicates if the service continuity support is required or not. |
| > EAS status | O | Required status of the EAS (e.g. enabled, disabled, etc.) |
| > Service permission level | O | Required level of service permissions e.g. trial, gold-class |
| > Service feature(s) | O | Required service features e.g. single vs. multi-player gaming service |
| NOTE 1: Only one of the information elements shall be present. NOTE 2: "Preferred ECSP list" IE shall not be present. NOTE 3: The "List of EAS characteristics" IE must include at least one optional IE, if used as an EAS discovery filter. |  |  |

* * * * Next Change * * * *

#### 8.8.4.2	EAS discovery request

Table 8.5.3.2-1 describes information elements for the EAS discovery request from the EAS to the EES and from the S-EES to the T-EES. Table 8.5.3.2-2 provides further detail about the EAS Discovery Filter information element.

discovery request

Editor's Note: [SA3] Whether and how security credentials corresponding to different Requestors considering different deployments are handled is FFS and in scope of SA3.

* * * * End of Changes * * * *