3GPP TSG-SA WG6 Meeting #42-BIS-e	S6-211015

e-meeting, 12th – 20th April 2021	(Revision of S6-210888)

Source:	Huawei, Hisilicon

Title:	Clarification on ECS subscribing the UPPM events using the DNN/Slice of the EES(s)

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Cuili Ge (gecuili@huawei.com)

1. Introduction

The contribution is introduced to clarify on ECS subscribing the UPPM events using the DNN/Slice of the EES(s).

2. Reason for Change

The ECS should use the DNN/Slice of the EES(s) to subscribe the user plane path management event notifications with the 3GPP core network, since the ECS is not limited in the EDN, hence, if the ECS uses the ECS DNAI(s) to subscribe the UPPM event notifications with the 3GPP core network, then the ECS will not receive the UPPM event notification from the 3GPP core network.

So to deal with the above issue, this contribution is provided to add DNN/Slice of the EES(s) for ECS to subscribe the user plane path management event notifications with the 3GPP core network on service provisioning subscription procedure.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0.

* * * First Change * * * *

###### 8.3.3.2.3.2	Subscribe

Figure 8.3.3.2.3.2-1 illustrates the service provisioning subscription procedure between the EEC and the ECS.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS as specified in clause 8.10;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization;

4.	The ECS is configured with ECSP's policy for service provisioning; and

5.	The EEC has optionally acquired a Notification Target Address to be used in its subscriptions to notifications.

NOTE 1:	Details of ECSP's policy are out of scope.

NOTE 2:	How the EEC acquires the notification target address or a notification channel URI to receive notifications is out of scope of this release.

Figure 8.3.3.2.3.2-1: Service provisioning subscription

1.	The EEC sends a service provisioning subscription request to the ECS. The service provisioning subscription request includes the security credentials of the EEC received during EEC authorization procedure and Notification Target Address (e.g. URL) and may include the UE identifier such as GPSI, connectivity information, proposed expiration time and AC Profile information.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. If required, the ECS may utilize the capabilities (e.g. UE location or user plane management event notification service if available) of the 3GPP core network as specified in clause 8.9.2 for further EES(s) determination. If the request is authorized, the ECS creates and stores the subscription for provisioning.

NOTE 3: The ECS can use available EDN configuration information (e.g. DNN, S-NSSAI, etc) to subscribe the user plane management event notification as specified in clause 3GPP TS 23.501 [2]

3.	If the processing of the request was successful, the ECS responds with a service provisioning subscription response, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EEC shall send a Service provisioning subscription update request prior to the expiration time. If a Service provisioning subscription update request is not received prior to the expiration time, the ECS shall treat the EEC as implicitly unsubscribed.

If the ECS is unable to determine the EES information using the inputs in service provisioning subscription request, UE-specific service information at the ECS or the ECSP policy, the ECS shall reject the service provisioning subscription request and respond with an appropriate failure cause.

NOTE 4:	If the service provisioning subscription request fails, the EEC can resend the service provisioning subscription request again, taking into account the received failure cause.

* * * End of Change * * * *