3GPP TSG-SA WG6 Meeting #42-BIS-e	S6-211021

e-meeting, 12th – 20th April 2021	(Revision of S6-210898)

Source:	Huawei, Hisilicon

Title:	Negotiation of notification mode between EEC and EES/ECS

Spec:	3GPP TS 23.558 v2.0.0

Agenda item:	7.6

Document for:	Approval

Contact:	Cuili Ge (gecuili@huawei.com)

1. Introduction

The contribution is introduced to provide Negotiation mechanism of notification mode between EEC and EES/ECS.

2. Reason for Change

In SA6#42 e-meeting, it is consensus that the propagation mechanism of nofifications to EEC from EES and ECS is underspecified, that is, the EEC may not be aware of the capability of the ECS and EES, e.g. supporting which kind of notification mechanism, so that the EEC may not support to receive the notification from ECS and EES.

Besides, it is discussed that Notification Target Address should be added in service provisioning subscription request and EAS discovery subscription request, however, it is still insufficient to address this issue, the reasons are as follows:

-	If the IP address is an private, then providing such “Notification Target Address” URL makes no sense

-	If the IP address is public, and if the server does not act as the role of server, then only providing the “Notification Target Address” URL does not work. I.e., only public IP addr & UE acting as the server role, the ECS can addressing the EEC via the “Notification Target Address” URL. With this option, if the UE has the server capability, then the resource/power consumption at the UE must be considered.

So, to resolve this issue, the EEC should provide its capabilities indicating which kind of notification mechanism it supports, such as subscription type (event, or periodical information), then the ECS and EES can determine or select one of the notification mechanism to be used based on subscription information.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v2.0.0.

* * * First Change * * * *

- 8.3.3.2.3.2	Subscribe

Figure 8.3.3.2.3.2-1 illustrates the service provisioning subscription procedure between the EEC and the ECS.

Pre-conditions:

1.	The EEC has been pre-configured or has discovered the address (e.g. URI) of the ECS;

2.	The EEC has been authorized to communicate with the ECS as specified in clause 8.10;

3.	The UE Identifier is either preconfigured or resulted from a successful authorization;

4.	The ECS is configured with ECSP's policy for service provisioning; and

5.	The EEC has optionally acquired a Notification Target Address to be used in its subscriptions to notifications.

NOTE 1:	Details of ECSP's policy are out of scope.

NOTE 2:	How the EEC acquires the notification target address or a notification channel URI to receive notifications is out of scope of this release.

Figure 8.3.3.2.3.2-1: Service provisioning subscription

1.	The EEC sends a service provisioning subscription request to the ECS. The service provisioning subscription request includes the security credentials of the EEC received during EEC authorization procedure and Notification Target Address (e.g. URL) and may include the UE identifier such as GPSI, Event ID, connectivity information, proposed expiration time and AC Profile information.

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS determines the notification method is used for sending service provisioning notification to EEC based on Event ID received from step1. If required, the ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.2. If the request is authorized, the ECS creates and stores the subscription for provisioning.

3.	If the processing of the request was successful, the ECS responds with a service provisioning subscription response, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EEC shall send a Service provisioning subscription update request prior to the expiration time. If a Service provisioning subscription update request is not received prior to the expiration time, the ECS shall treat the EEC as implicitly unsubscribed.

If the ECS is unable to determine the EES information using the inputs in service provisioning subscription request, UE-specific service information at the ECS or the ECSP policy, the ECS shall reject the service provisioning subscription request and respond with an appropriate failure cause.

NOTE 2:	If the service provisioning subscription request fails, the EEC can resend the service provisioning subscription request again, taking into account the received failure cause.

###### 8.3.3.2.3.4	Subscription update

Figure 8.3.3.2.3.4-1 illustrates the service provisioning subscription update procedure between the EEC and the ECS.

Pre-conditions:

1.	The EEC has subscribed with the ECS for the provisioning information as specified in clause 8.3.3.2.3.2.

Figure 8.3.3.2.3.4-1: Service provisioning subscription update

1.	The EEC sends a service provisioning subscription update request to the ECS. The service provisioning subscription update request includes the security credentials of the EEC received during EEC authorization procedure along with the subscription identifier and may include the UE identifier such as GPSI, Event ID, connectivity information, proposed expiration time for the updated subscription and AC profile(s).

2.	Upon receiving the request, the ECS performs an authorization check to verify whether the EEC has authorization to perform the operation. The ECS determines the notification method is used for sending service provisioning notification to EEC based on Event ID received from step1. If required, the ECS may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.2. If authorized, the ECS updates the stored subscription for provisioning as requested in step 1.

3.	The ECS responds with a service provisioning subscription update response, which may include the expiration time, indicating when the updated subscription will automatically expire. To maintain the subscription, the EEC shall send a Service provisioning subscription update request prior to the expiration time. If a Service provisioning subscription update request is not received prior to the expiration time, the ECS shall treat the EEC as implicitly unsubscribed.

* * * Next Change * * * *

##### 8.3.3.3.4	Service provisioning subscription request

Table 8.3.3.3.4-1 describes the information elements for service provisioning subscription request from the EEC to the ECS.

Table 8.3.3.3.4-1: Service provisioning subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Event ID | M | The trigger event that ECS send service provisioning notification to EEC which has subscribed the service provisioning notification. Event ID: - New EES(s) is registered; - EES(s) serving UE location; |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the EEC should be sent to. |
| AC profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Connectivity information | O | List of connectivity information for the UE, e.g. PLMN ID, SSID. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

##### 8.3.3.3.5	Service provisioning subscription response

Table 8.3.3.3.5-1 describes the information elements for service provisioning subscription response from the ECS to the EEC.

Table 8.3.3.3.5.-1: Service provisioning subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Notification method | M | Indicates the notification method is used for sending service provisioning notification to EEC, (e.g. SIP, SSE, Websocket, long-polling and so on). |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

* * * Next Change * * * *

- 8.3.3.3.7	Service provisioning subscription update request

Table 8.3.3.3.7-1 describes the information elements for service provisioning subscription update request from the EEC to the ECS.

Table 8.3.3.3.7-1: Service provisioning subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Event ID | M | The trigger event that ECS send service provisioning notification to EEC which has subscribed the service provisioning notification. Event ID: - New EES(s) is registered; - EES(s) serving UE location; |
| Connectivity information | O | Details about the connectivity of the UE with the PLMN, such as type of the 3GPP network. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| AC profile(s) | O | Information about services the EEC wants to connect to, as described in Table 8.2.2-1. |

##### 8.3.3.3.8	Service provisioning subscription update response

Table 8.3.3.3.8-1 describes the information elements for service provisioning subscription update response from the ECS to the EEC.

Table 8.3.3.3.8.-1: Service provisioning subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Notification method | M | Indicate the notification method is used for sending service provisioning notification to EEC, (e.g. SIP, SSE, Websocket, long-polling and so on). |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

* * * Next Change * * * *

##### 8.5.2.3.2	Subscribe

Figure 8.5.2.3.2-1 illustrates the EAS discovery subscription procedure between the EEC and the EES. This subscription enables EES to inform EEC of various EAS discovery related events of interest to EEC (e.g. EAS discovery notification and EAS dynamic information).

Pre-conditions:

1.	The EEC has received information (e.g. URI, IP address) related to the EES;

2.	The EEC has received appropriate security credentials authorizing it to communicate with the EES as specified in clause 8.10; and

3.	The EES is configured with ECSP's policy for EAS discovery.

NOTE 1:	Details of ECSP's policy are out of scope.

Figure 8.5.2.3.2-1: EAS discovery subscription

1.	The EEC sends an EAS discovery subscription request to the EES. The EAS discovery subscription request includes the EECID along with the security credentials and may include Event ID, EAS discovery filters and EAS dynamic information filters to subscribe to information about particular EAS(s) or a category of EASs (e.g. gaming applications) or dynamic information about EAS(s).

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to subscribe for information of the requested EAS(s). The authorization check may apply to an individual EAS, a category of EASs or to the EDN, i.e. to all the EASs. The EES determines the notification method is used for sending EAS discovery subscription notification to EEC based on Event ID received from step1. The EES may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.3. If the request is authorized, the EES creates and stores the subscription for EAS discovery.

3.	If the processing of the request was successful, the EES sends an EAS discovery subscription response to the EEC, which includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EEC shall send an EAS discovery subscription update request prior to the expiration time. If an EAS discovery subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed.

If the EES is unable to determine the EAS information using the inputs in the EAS discovery request, UE-specific service information at the EES or the ECSP policy, the EES shall reject the EAS discovery request and respond with an appropriate failure cause.

NOTE 2:	If the EAS discovery subscription request fails, the EEC can resend the EAS discovery subscription request again, taking into account the received failure cause.

##### 8.5.2.3.3	Notify

Figure 8.5.2.3.3-1 illustrates the EAS discovery notification procedure between the EEC and the EES.

Pre-conditions:

1.	The EEC has subscribed with the EES for the EAS discovery information as specified in clause 8.5.2.3.2.

Figure 8.5.2.3.3-1: EAS discovery notification

1.	An event occurs at the EES that satisfies trigger conditions or event ID for notifying (e.g. to provide EAS discovery information or EAS dynamic information) a subscribed EEC. If UE's location information is not available, the EES may obtain the UE location by utilizing the capabilities of the 3GPP core network as specified in clause 8.9.2. If EAS discovery filters were provided by the EEC during subscription creation, the EES identifies the EAS(s) based on the provided EAS discovery filters and the UE location. If EAS discovery filters were not provided, then:

-	if available, the EES identifies the EAS(s) based on the UE-specific service information at the EES and the UE location;

-	EES identifies the EAS(s) by applying the ECSP policy (e.g. based only on the UE location);

NOTE 1:	Details of the UE-specific service information and how it is available at the EES is out of scope.

NOTE 2:	Both steps are evaluated prior to sending a response.

If the UE is located outside the Geographical or Topological Service Area of an EAS, then the EES shall not include this EAS in the EAS discovery notification.

2.	The EES sends an EAS discovery notification to the EEC with the EAS information determined in step 1.

##### 8.5.2.3.4	Subscription update

Figure 8.5.2.3.4-1 illustrates the EAS discovery subscription update procedure between the EEC and the EES.

Pre-conditions:

1.	The EEC has subscribed with the EES for the EAS discovery information as specified in clause 8.5.2.3.2

Figure 8.5.2.3.4-1: EAS discovery subscription update

1.	The EEC sends an EAS discovery subscription update request to the EES. The EAS discovery subscription update request includes the security credentials and the subscription identifier. It may also include Event ID, EAS discovery filters, EAS dynamic information filters and/or proposed expiration time for the updated subscription.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to update the subscription information. The EES determines the notification method is used for sending EAS discovery subscription notification to EEC based on Event ID received from step1. The EES may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.3. If the request is authorized, the EES updated the stored subscription for EAS discovery.

3.	The EES sends an EAS discovery subscription update response to the EEC, which may include the expiration time, indicating when the updated subscription will automatically expire. To maintain the subscription, the EEC shall send an EAS discovery subscription update request prior to the expiration time. If an EAS discovery subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed.

* * * Next Change * * * *

#### 8.5.3.4	EAS discovery subscription request

Table 8.5.3.4-1 describes the information elements for EAS discovery subscription request from the EEC to the EES.

Table 8.5.3.4-1: EAS discovery subscription request

| Information element | Status | Description |
| --- | --- | --- |
| EECID | M | Unique identifier of the EEC. |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Event ID | M | The trigger event that EES send EAS discovery notification or EAS dynamic information notification to subscribed EEC. Event ID: - New EES(s) is registered; - EES(s) serving UE location; |
| EAS discovery filters | O | Set of characteristics to determine matching EASs (as detailed in Table 8.5.3.2-2). |
| EAS dynamic information filters | O | List of dynamic information changes (as detailed in Table 8.5.3.4-3) about EAS, the EEC is interested in. |
| EAS discovery notification triggers | M | EEC provided notification triggers for EAS discovery notification, as described in Table 8.5.3.4-2. |
| Proposed expiration time | O | Proposed expiration time for the subscription |

Table 8.5.3.4-2: EAS discovery notification triggers

| Information element | Status | Description |
| --- | --- | --- |
| EAS availability change | O | Change in availability of an EAS for e.g. registration of a new EAS, de-registration of an existing EAS |
| EAS dynamic information change | O | Change in dynamic information of an EAS |
| NOTE: At least one notification trigger shall be included. |  |  |

Editor's note:	Restructuring the IEs with relevant Event ID description as well as whether the EAS discovery notification filters can be merged into the EAS discovery filters and EAS dynamic information filters is FFS.

Table 8.5.3.4-3: EAS dynamic information filters

| Information element | Status | Description |
| --- | --- | --- |
| List of dynamic information filters | M | List of EAS dynamic information required by the EEC per EAS. |
| > EASID | M | Identifier of the EAS |
| > ACIDs | O | Flag to notify change in list of ACIDs served by the EAS |
| > EAS Description | O | Flag to notify change in description of the EAS. |
| > EAS Endpoint | O | Flag to notify change in EAS endpoint |
| > EAS Features | O | Flag to notify any change in features provided by the EAS |
| > EAS Schedule | O | Flag to notify change in availability schedule of the EAS (e.g. time windows) |
| > EAS Service Area | O | Flag to notify change in change in geographical service area that the EAS serves |
| > EAS Service KPIs | O | Flag to notify change in characteristics of the EAS. |
| > EAS Status | O | Flag to notify change in the status of the EAS (e.g. enabled, disabled, etc.) |
| > Service continuity support | O | Flag to notify change in EAS support for service continuity. |

#### 8.5.3.5	EAS discovery subscription response

Table 8.5.3.5-1 describes the information elements for EAS discovery subscription response from the EES to the EEC.

Table 8.5.3.5.-1: EAS discovery subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Notification method | M | Indicate the notification method is used for EAS discovery notification to EEC, (e.g. SIP, SSE, Websocket, long-polling and so on). |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

* * * Next Change * * * *

#### 8.5.3.7	EAS discovery subscription update request

Table 8.5.3.7-1 describes the information elements for EAS discovery subscription update request from the EEC to the EES.

Table 8.5.3.7-1: EAS discovery subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Event ID | M | The trigger event that EES send EAS discovery notification or EAS dynamic information notification to subscribed EEC. Event ID: - New EES(s) is registered; - EES(s) serving UE location; |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| EAS discovery filters | O | Set of characteristics to determine required EASs, as detailed in Table 8.5.3.2-2. |
| EAS dynamic information filters | O | List of dynamic information about EAS, the EEC is interested in, as detailed in Table 8.5.3.4-3. |
| EAS discovery notification triggers | O | EEC provided notification triggers for EAS discovery notification, as described in Table 8.5.3.4-2. |

- 8.5.3.8	EAS discovery subscription update response

Table 8.5.3.8-1 describes the information elements for EAS discovery subscription update response from the EES to the EEC.

Table 8.5.3.8.-1: EAS discovery subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time | O | Indicates the expiration time of the updated subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| > Notification method | M | Indicate the notification method is used for EAS discovery notification to EEC, (e.g. SIP, SSE, Websocket, long-polling and so on). |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |

* * * End of Change * * * *