3GPP TSG-SA WG6 Meeting #42-e	S6-210458

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

Source:	Ericsson

Title:	ACR events consolidation

Spec:	3GPP TS 23.558 v1.3.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution solves the ENs for AF traffic influence delegation and consolidates the events to support ACR.

2. Reason for Change

Clause 8.6.1 has one EN about the AF traffic request delegation:

Editor's Note:	It is FFS whether a new API for delegating AF Request is necessary or the existing API in clause 8.6.3 can be enhanced to support the delegation of AF Request to influence traffic routing.

Another EN related to this is in clause 8.6.3.2.3:

Editor's note:	With the "application context relocation monitoring" Event, the AF traffic influence executed by EAS/EES is for FFS.

As discussed for 0244/0088 in the previous meeting, the EES can offer different services to the EAS in relation to :

- Monitor the UP path change

- Discover target EAS(s)

- Select target EAS

- Influence the traffic.

Initially, UPPM event API only offers [UP path change] event. Later, another event [ACR monitoring] is added to further enhance the same API.

By following the same approach, the 3rd event [ACR facilitation] which is proposed in this CR can offer further facilitation to the EAS as shown above.

Those 3 events can be selected by EAS for the purpose of ACR based on EAS need, therefore it is proposed also to change the API name from UPPM event to ACR management event.

Because the 3rd event can be used in the EES decided ACR scenario, this CR also includes the related description for the e2e case in cl.8.8.2.5, which is similar to the trigger description in cl.8.8.2.4.

3. Proposal

It is proposed to agree the following changes in TS 23.558.

* * * First Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

ACID	Application Client Identification

ACR		Application Context Relocation

AF	Application Function

ASP	Application Service Provider

DN	Data Network

DNAI	Data Network Access Identifier

DNN	Data Network Name

EAS	Edge Application Server

EASID	Edge Application Server Identification

ECS	Edge Configuration Server

ECSP	Edge Computing Service Provider

EDN	Edge Data Network

EEC	Edge Enabler Client

EECID	Edge Enabler Client Identification

EES	Edge Enabler Server

EESID	Edge Enabler Server Identification

FQDN	Fully Qualified Domain Name

GPSI	Generic Public Subscription Identifier

LADN	Local Area Data Network

NEF	Network Exposure Function

SCEF	Service Capability Exposure Function

SSID	Service Set Identifier

TAI	Tracking Area Identity

* * * Next Change * * * *

## 6.6	Capability exposure for enabling edge applications

The Figure 6.6-1 shows the capability exposure for enabling edge applications.

Figure 6.6-1: Capability exposure for enabling edge applications

Capability exposure includes the 3GPP core network (i.e. 5GC, EPC) , Edge Configuration Server and the Edge Enabler Server capability exposure, to fulfil the needs of the edge service operations. The capability exposure functionality is utilized by the functional entities (i.e. Edge Enabler Server, Edge Application Server and Edge Configuration Server) depicted in the architecture for enabling the edge applications.

Table 6.6-1 summarizes the APIs exposed by the ECS.

Table 6.6-1: APIs provided by the ECS

| API Name | References |
| --- | --- |
| Eecs_ServiceProvisioning | 8.3 |
| Eecs_EESRegistration | 8.4.4 |

Table 6.6-2 summarizes the APIs exposed the EES.

Table 6.6-2: APIs provided by the EES

| API Name | References |
| --- | --- |
| Eees_EECRegistration | 8.4.2 |
| Eees_EASRegistration | 8.4.3 |
| Eees_EASDiscovery | 8.5 |
| Eees_UELocation | 8.6.2 |
| Eees_ACRManagementEvent | 8.6.3 |
| Eees_AppClientInformation | 8.6.4 |
| Eees_UEIdentifier | 8.6.5 |
| Eees_SessionWithQoS | 8.6.6 |
| Eees_TargetEASDiscovery | 8.8.3.2 |

* * * Next Change * * * *

### 8.6.1	General

This clause describes service capability APIs exposed by the Edge Enabler Server to the Edge Application Server(s). The service capability APIs exposed include EES capabilities and exposed 3GPP Core Network capabilities. The 3GPP Core Network capabilities may be exposed from EES to the Edge Application Server(s).

The 3GPP Core Network capabilities APIs which are enhanced for exposure are also specified in this clause.

* * * Next Change * * * *

### 8.6.3	Application Context Relocation management events

#### 8.6.3.1	General

The Edge Enabler Server exposes ACR management event notifications of one or more UEs to an Edge Application Server (e.g. in order to trigger the application context relocation). ACR management event notifications exposed by the Edge Enabler Server may rely on the NEF northbound API for monitoring event of user plane path management event.

This capability exposed by the EES supports:

-	"User plane path change". This event supports to detect user plane path change for the application traffic and report the corresponding notification with user plane path change to the EAS.

-	"ACR monitoring". This event supports to detect the need of ACR (e.g. user plane path change for the application traffic, ACR initiation request or ACR determination request from EEC in clause 8.8.3.4 and 8.8.3.5), discover the target EAS(s), and report the corresponding notification with the discovered target EAS(s).

-	"ACR facilitation". This event supports to detect the need of ACR (e.g. user plane path change for the application traffic or ACR initiation request or ACR determination request from EEC in clause 8.8.3.4 and 8.8.3.5), make the decision for application context relocation, discover the target EAS(s), influence the traffic for the selected target EAS and report the corresponding notification with the selected target EAS.

The availability of the ACR management event notifications relying on the user plane path management event notification service exposed by 5GC may change due to UE mobility between 5GC and EPC. If an Edge Enabler Server exposes ACR event notifications to Edge Application Server(s) for a UE by relying on the 3GPP Core Network northbound API for user plane path management notifications, the Edge Enabler Server monitors the availability of the northbound API for UE(s) served by the Edge Application Server (e.g. by utilizing Nnef_APISupportCapability as described in 3GPP TS 23.502 [3]) and provides the availability information to the Edge Application Server accordingly. If CAPIF is supported, the EES determines if the ACR management event API is available and able to be exposed to the Edge Application Server for a UE via the Availability of service APIs event notifications provided by the CAPIF core function as defined in 3GPP TS 23.222 [6].

##### 8.6.3.2.2	Subscribe

Figure 8.6.3.2.2-1 illustrates the subscribe operation between the Edge Application Server and the Edge Enabler Server for ACR management event notifications.

Figure 8.6.3.2.2-1: ACR management event API: Subscribe operation

1.	The Edge Application Server sends ACR management event subscribe request (e.g. tracking the UE's user plane path change continuously). The Edge Application Server shall include UE Identifier or UE group Identifier.

a.	The Edge Application Server may include the "user plane path change" event to indicate the EES to notify the Edge Application Server when the EES detects there is a user plane path change for the application traffic and the Edge Application Server may include Subscription Type (Early and/or Late notification defined in clause 5.6.7 of 3GPP TS 23.501 [2]) and/or Indication of EAS Acknowledgement in the event subscription.

b.	The Edge Application Server may include the "application context relocation monitoring" event to indicate the EES to notify the Edge Application Server when the EES detects there is a need for the application context relocation (e.g. when target EAS is available at the target DNAI). The Edge Application Server may also include the Event Filters to specify the conditions to match for notifying the event, e.g., inter-EDN mobility.

c.	The EAS may include the "ACR facilitation" event to indicate the EES to notify the source EAS of the selected target EAS after the EES detects and determines the need for ACR.

2.	The Edge Enabler Server checks if the Edge Application Server is authorized for this operation. If authorized, Edge Enabler Server may check if there exists a subscription with the 3GPP core network for the user plane path management event notifications corresponding to the UE information obtained in step 1 as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], which may be triggered by other Edge Application Server for the same UE. The Edge Enabler Server may check the availability of the user plane path management event service for the UE.

a.	if a subscription with 3GPP core network does not exist, then the Edge Enabler Server subscribes with the 3GPP core network (PCF, NEF or SCEF+NEF) for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] If the Edge Application Server provides Subscription Type and/or Indication of EAS Acknowledgement, the Edge Enabler Server include the type of subscription and/or the indication of "AF acknowledgement to be expected" as information on AF subscription to corresponding SMF events within the AF Request;

b.	if a subscription with 3GPP core network exists, then the Edge Enabler Server uses the locally cached user plane path management event notification information of the UE to respond to the Edge Application Server.

The Edge Enabler Server stores the subscription related to the Edge Application Server.

3.	If the event is "user plane path change", the Edge Enabler Server may subscribe to UE expected behaviour analytics (UE mobility and UE communication) for the group of UEs as described in 3GPP TS 23.288 [18].

4.	If Edge Application Server is authorized, the Edge Enabler Server responds with ACR path management event subscribe response. If Edge Application Server is not authorized, the Edge Enabler Server provides a rejection response with cause information.

If the target UE and the 3GPP network support mobility between 5GC and EPC, the Edge Enabler Server monitors the availability of the user plane path management event notification from the 3GPP network by utilizing Nnef_APISupportCapability or Availability of service APIs event notifications provided by the CAPIF core function.

* * * Next Change * * * *

##### 8.6.3.2.3	Notify

Figure 8.6.3.2.3-1 illustrates the notify operation between the Edge Enabler Server and the Edge Application Server for continuous ACR management event notifications.

Figure 8.6.3.2.3-1: ACR management event API: Notify operation

1.	The Edge Enabler Server detects the ACR management event of the UE (e.g. receiving User plane path management event notification for the UE from the 3GPP core network).

a.	If "user plane path change" Event is subscribed, the Edge Enabler Server may cache the detected User plane path management event notification locally with timestamp as the latest information of the UE(s) and start the notification aggregation for a group of UEs. The Edge Enabler Server decides whether to aggregate and the aggregation period based on the analytics result received from the 3GPP Core Network, local policy and User Plane path management subscription information received from the Edge Application Server. The Edge Enabler Server determines to notify the user plane path management event notification information (e.g., DNAI) to the Edge Application Servers which has subscribed for the "user plane path management" event.

b.	If "ACR monitoring" Event is subscribed, based on the detected ACR report sent from the 3GPP core network or EEC, the EES checks if a target EAS is available. If the target DNAI is included in the recieved report, the Edge Enabler Server checks whether the target DNAI is in the EAS profile of the subscribing EAS, if not it further checks whether a target Edge Application Server is available at the target DNAI as described in steps 2-4 of clause 8.8.3.2. If a target Edge Application Server is available, the Edge Enabler Server notifies the Edge Application Server with target EAS endpoint.

c.	If "ACR facilitation" Event is subscribed, based on the detected user plane path change report sent from the 3GPP core network, the EES checks whether the target DNAI is in the EAS profile of the subscribing EAS, if not it further checks whether a target EAS is available at the target DNAI as described in steps 2-4 of clause 8.8.3.2.  If a target EAS is available, the EES selects the target EAS from the discovered EAS list and applies the AF traffic influence with the N6 routing information of the selected target EAS in the 3GPP Core Network. The EES also notifies the source EAS with the selected target EAS endpoint.

2.	The Edge Enabler Server sends ACR management event notification to the Edge Application Server. The Edge Enabler Server includes the ACR management event notification information of the UE(s) and optionally the timestamp. If the event triggering the notification is DNAI change, the timestamp can be included to indicate the age of the user plane path management event notification information. The Edge Enabler Server may only provide part of information included in the user plane path management event notification from 3GPP network (e.g. target DNAI). If the Edge Application Server had provided "Indication of EAS acknowledgement", the Edge Enabler Server waits for acknowledgement from the EAS before it sends AF acknowledgement to the 3GPP core network.

3.	If the Edge Application Server had included Indication of EAS Acknowledgement within the ACR management event subscribe request described in clause 8.6.3.2.1, the Edge Application Server sends EAS Acknowledgement as a response to ACR management event notification to the Edge Enabler Server either immediately or after the required application context transfer is completed. The Edge Application Server may reply in negative, e.g., the Edge Application Server may determine not to perform application context relocation. Then, the Edge Enabler Server sends the AF acknowledgement to the 3GPP core network.

Editor's Note:	It is FFS whether the EAS can provide N6 traffic routing details, target EAS address, and notification target address of the target EAS to the EES.

Editor's Note:	The relationship between the step 3 and the application relocation notify in clause 8.8.2.3 and 8.8.2.5 is FFS.

* * * Next Change * * * *

##### 8.6.3.2.4	Subscription update

Figure 8.6.3.2.4-1 illustrates the subscription update operation between the Edge Application Server and the Edge Enabler Server for ACR management event notifications.

Figure 8.6.3.2.4-1: ACR management event API: Subscription update operation

1.	The Edge Application Server sends ACR management event subscription update request to update an existing subscription. The subscription update request may include Event ID, Event Filter, Event Report, Subscription Type (Early and/or Late notification defined in clause 5.6.7 of 3GPP TS 23.501 [2]) and/or Indication of EAS Acknowledgement.

2.	The Edge Enabler Server checks if the Edge Application Server is authorized for the operation. If authorized, Edge Enabler Server checks if there exists a subscription with the 3GPP core network for the user plane path management event notifications corresponding to the updated information obtained in step 1 as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3], which may be triggered by other Edge Application Server for the same UE.

a.	if a subscription with 3GPP core network does not exist corresponding to the updated information, then the Edge Enabler Server subscribes with the 3GPP core network (PCF, NEF or SCEF+NEF) for the user plane path management event notifications of the UE as described in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3] If the Edge Application Server provides Subscription Type and/or Indication of EAS Acknowledgement, the Edge Enabler Server includes the type of subscription and/or the indication of "AF acknowledgement to be expected" as information on AF subscription to corresponding SMF events within the AF Request;

b.	if a subscription with 3GPP core network exists corresponding to the updated information, then the Edge Enabler Server uses the locally cached user plane path management event notification information of the UE to respond to the Edge Application Server.

The Edge Enabler Server stores the updated subscription related to the Edge Application Server.

3.	The Edge Enabler Server may update the subscription to the UE expected behaviour analytics.

4.	The Edge Enabler Server responds with ACR management event subscription response.

* * * Next Change * * * *

##### 8.6.3.2.5	Unsubscribe

Figure 8.6.3.2.5-1 illustrates the unsubscribe operation between the Edge Application Server and the Edge Enabler Server for ACR management event notifications.

Figure 8.6.3.2.5-1: ACR management event API: Unsubscribe operation

1.	The Edge Application Server sends ACR management event unsubscribe request to the Edge Enabler Server to cancel the ACR management event subscription.

2.	The Edge Enabler Server checks if another EAS requires to track the UE's user plane path change or not. If not, the Edge Enabler Server unsubscribes the user plane path management event notifications from the 3GPP core network for the corresponding UE.

3.	The Edge Enabler Server unsubscribes from the UE expected behaviour analytics, if applicable.

4.	The Edge Enabler Server checks if the Edge Application Server is authorized for the operation. If authorized, the Edge Enabler Server removes the subscription related to the Edge Application Server and sends ACR management event unsubscribe response to the Edge Application Server.

* * * Next Change * * * *

##### 8.6.3.3.1	ACR management event subscribe request

Table 8.6.3.3.1-1 describes the information elements for a ACR management event subscribe request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.3.3.1-1: ACR management event subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID (NOTE) | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE Group ID (NOTE) | O | Identifies a group of UEs (i.e. internal group ID or external group ID) |
| Event ID(s) | M | Event ID: - user plane path change - ACR monitoring - ACR facilitation |
| Event Report | M | Event Reporting Information as specified in 3GPP TS 23.502 [3] |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Type of subscription | O | Indicates Early and/or Late notification to inform if the notification needs to be received before and/or after UP path configuration. Applicable for the "user plane path change" event. |
| Indication of EAS acknowledgement | O | This IE indicates the EES to include indication of "AF acknowledgement to be expected" within the AF request for subscribing UP path management events to 3GPP network and that the EAS will provide an acknowledgement as a response for the notifications of UP path management events to the EES. Applicable for the "user plane path change" event. |
| Event Filter | O | Event filter as specified in 3GPP TS 23.501 [2] |
| EAS characteristics for ACR | O | Set of characteristics to determine required EAS as detailed in Table 8.5.3.1-2. Applicable for the "ACR monitoring" event and "ACR facilitation" event. |
| NOTE: Either UE ID or UE Group ID shall be provided. |  |  |

* * * Next Change * * * *

##### 8.6.3.3.2	ACR management event subscribe response

Table 8.6.3.3.2-1 describes the information elements for a ACR management event subscribe response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.2-1: ACR management event API subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| > Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the subscription never expires. |
| Failure response | O | Indicates that the subscription request failed. |
| > Cause | O | Indicates the cause of subscription request failure |

* * * Next Change * * * *

##### 8.6.3.3.3	ACR management event notification

Table 8.6.3.3.3-1 describes the information elements for a ACR management event notification from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.3-1: ACR management event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |
| List of event notifications | M | A list of event notifications for one or more UEs. |
| > Event report | M | Event reporting information as specified in clause 5.2.8.3.1 of 3GPP TS 23.502 [3] |
| > Timestamp | O | The timestamp of each event report. |
| > Target EAS endpoint | O | The target EAS endpoint. Applicable for the "ACR monitoring" event and "ACR facilitation" event. |

* * * Next Change * * * *

##### 8.6.3.3.4	ACR management event subscription update request

Table 8.6.3.3.4-1 describes the information elements for a ACR management event subscribe request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.3.3.4-1: ACR management event subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | The identifier of the subscription to be updated. |
| Event ID(s) | O | Event ID(s) as specified in 3GPP TS 23.501 [2] |
| Event Report | O | Event Reporting Information as specified in 3GPP TS 23.502 [3] |
| Type of subscription | O | Indicates Early and/or Late notification to inform if the notification needs to be received before and/or after UP path configuration. |
| Indication of EAS acknowledgement | O | This IE indicates the EES to include indication of "AF acknowledgement to be expected" within the AF request for subscribing UP path management events to 3GPP network and that the EAS will provide an acknowledgement as a response for the notifications of UP path management events to the EES. |
| Event Filter | O | Event filter as specified in 3GPP TS 23.501 [2] |
| EAS characteristics for ACR | O | Set of characteristics to determine required EAS as detailed in Table 8.5.3.1-2. Applicable for the "ACR monitoring" event and "ACR facilitation" event. |

* * * Next Change * * * *

##### 8.6.3.3.5	ACR management event subscription update response

Table 8.6.3.3.5-1 describes the information elements for a ACR management event subscription update response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.5-1: ACR management event subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the subscription update request was successful. |
| > Expiration time (NOTE) | O | Indicates the expiration time of the updated subscription. To maintain an active subscription, a subscription update is required before the expiration time. |
| Failure response | O | Indicates that the subscription update request failed. |
| > Cause | O | Indicates the cause of subscription update request failure |
| NOTE: If the Expiry time IE is not included, it indicates that the subscription never expires. |  |  |

* * * Next Change * * * *

##### 8.6.3.3.6	ACR management event unsubscribe request

Table 8.6.3.3.6-1 describes the information elements for a ACR management event unsubscribe request from the Edge Application Server to the Edge Enabler Server.

Table 8.6.3.3.6-1: ACR management event unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription stored in the EES for the request |

* * * Next Change * * * *

##### 8.6.3.3.7	ACR management event unsubscribe response

Table 8.6.3.3.7-1 describes the information elements for a ACR management event unsubscribe response from the Edge Enabler Server to the Edge Application Server.

Table 8.6.3.3.7-1: ACR management event unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the unsubscribe request was successful. |
| Failure response | O | Indicates that the unsubscribe request failed. |
| > Cause | O | Indicates the cause of unsubscribe request failure |

* * * Next Change * * * *

##### 8.6.3.4.1	General

Table 8.6.3.4.1-1 illustrates the API for ACR management event.

Table 8.6.3.4.1-1: Eees_ACRManagementEvent API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_ACRManagementEvent | Subscribe | Subscribe/Notify | EAS |
|  | Notify |  |  |
|  | UpdateSubscription |  |  |
|  | Unsubscribe |  |  |

* * * Next Change * * * *

##### 8.6.3.4.2	Eees_ACRManagementEvent_Subscribe operation

API operation name: Eees_ACRManagementEvent_subscribe

Description: The consumer subscribes to receive a ACR management event.

Inputs: See clause 8.6.3.3.1.

Outputs: See clause 8.6.3.3.2.

See clause 8.6.3.2.2 for details of usage of this operation.

* * * Next Change * * * *

##### 8.6.3.4.3	Eees_ACRManagementEvent_Notify operation

API operation name: Eees_ACRManagementEvent_Notify

Description: The consumer is notified of an event by the Edge Enabler Server.

Inputs: See clause 8.6.3.3.3.

Outputs: None.

See clause 8.6.3.2.3 for details of usage of this operation.

* * * Next Change * * * *

##### 8.6.3.4.4	Eees_ACRManagementEvent_UpdateSubscription operation

API operation name: Eees_ACRManagementEvent_UpdateSubscription

Description: The consumer updates an existing subscription for a ACR management event.

Inputs: See clause 8.6.2.3.4.

Outputs: See clause 8.6.2.3.5.

See clause 8.6.3.2.4 for details of usage of this operation.

* * * Next Change * * * *

##### 8.6.3.4.5	Eees_ACRManagementEvent_Unsubscribe operation

API operation name: Eees_ACRManagementEvent_Unsubscribe

Description: The consumer unsubscribes for the previously subscribed events.

Inputs: See clause 8.6.3.3.6.

Outputs: See clause 8.6.3.3.7.

See clause 8.6.3.2.5 for details of usage of this operation.

* * * Next Change * * * *

#### 8.8.2.5	Source EES executed application context relocation

Figure 8.8.2.5-1 illustrates the procedure for the S-EES to decide and execute the application context relocation from the S-EAS to the T-EAS

Pre-condition:

1.	The AC at the UE already has a connection to the S-EAS; and

2.	The EEC is able to communicate with the S-EES.

Figure 8.8.2.5-1: S-EES executed application context relocation procedure

Phase I: ACR Detection

1.	Detection entities (S-EAS, S-EES, EEC) detects that application context relocation may be required as described in clause 8.8.1. The detection by the S-EES may be triggered by the User Plane path change notification received from the 3GPP Core Network due to S-EAS request for "ACR facilitation" event (see clause 8.6.3).

2.	The detection entity informs the S-EES that application context relocation is required as in clause 8.8.3.5.

Phase II: ACR Decision

3.	The S-EES authorises the received message from step 2. The S-EES decides to execute application context relocation based on the information received in step 2 and the information of EEC context or EAS profile, and then proceed the below steps.

Phase III:	ACR Execution

4.	The S-EES determines T-EES and T-EAS via the Discover target EAS procedure in clause 8.8.3.2 of the present document. The S-EES may decide not to perform application context relocation if T-EAS is not available.

5.	The source EES sends the target information notification to the EEC as described in clause 8.8.3.6.

6.	The S-EES may apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable) and sends the Application Context Relocation Notify message (e.g. as notification for "ACR facilitation" event) to the S-EAS to initiate application context transfer between the S-EAS and the T-EAS.

7.	The S-EAS transfers the application context to the T-EAS at implementation specific time. This process is out of scope of the present specification.

Phase IV:	Post-ACR Clean up

8.	The T-EAS sends the Application Context Relocation Complete message to the T-EES to confirm that the application context relocation has completed.

9.	The S-EAS sends the Application Context Relocation Complete message to the S-EES to confirm that the application context relocation has completed.

10.	The S-EES sends the Application Context Relocation Complete message to the EEC to confirm that the application context relocation has completed.

Editor's note: The relationship of scenario specified in clause 8.8.2.5 and building block procedure specified in clause 8.8.3.4 and its representation is FFS

* * * End of Changes * * * *