3GPP TSG-SA WG6 Meeting #42-e	S6-210495

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

Source:	Samsung, AT&T

Title:	Edge Enabler Layer definition

Spec:	3GPP TS 23.558 v1.3.0

Agenda item:	7.6

Document for:	Approval

Contact:	Nishant Gupta <nishant.gup@samsung.com>

1. Introduction

This contribution provides a definition of Edge Enabler Layer.

2. Reason for Change

The term is used frequently in the TS without appropriate definition.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.558 v1.3.0.

* * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application Context: A set of data about the Application Client that resides in the Edge Application Server.

Application Context Relocation: Refers to the end-to-end service continuity procedure described in clause 8.8.

Application Context Transfer: Refers to the transfer of the Application Context between the source Edge Application Server and the target Edge Application Server, which is a part of the service continuity procedure described in clause 8.8.

Application Server: Application software resident in the cloud performing the server function.

Edge Computing: A concept, as described in 3GPP TS 23.501 [2], that enables operator and 3rd party services to be hosted close to the UE's access point of attachment, to achieve an efficient service delivery through the reduced end-to-end latency and load on the transport network.

Edge Computing Service Provider: A mobile network operator or a 3rd party service provider offering Edge Computing service.

Edge Data Network: A local Data Network that supports the architecture for enabling edge applications.

EEC Context: A set of data about the Edge Enabler Client that resides in the Edge Enabler Server.

Edge Enabler Layer: Refers to the overall functionality provided by entities such as Edge Enabler Client, Edge Enabler Server and Edge Configuration Server (as per the architecture for enabling edge applications defined in clause 6) in support of Application Clients requiring to locate and connect with the most suitable Edge Application Servers.

Edge Hosting Environment: An environment providing support required for Edge Application Server's execution.