3GPP TSG-SA WG6 Meeting #42-e	S6-210465

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

Source:	Ericsson

Title:	Fix inconsistency

Spec:	3GPP TS 23.558 v1.3.0

Agenda item:	7.6

Document for:	Approval

Contact:	Wenliang Xu; wenliang.xu@ericsson.com

1. Introduction

This contribution fixes inconsistencies.

2. Reason for Change

1) cl.8.5.2.3.4 subscription update mentions UE ID as part of the updated information. But information table 8.5.3.7 doesn’t include it and the UE ID should not be changed during the subscription lifetime.

2) The replacement from location accurarcy to location QoS is not fully done for the location API.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes in TS 23.558.

Revision history:

R1: remove EEC ID in 8.5.2.3.4 update operation and do the same for 8.5.2.4.4 update operation

* * * First Change * * * *

##### 8.5.2.3.4	Subscription update

Figure 8.5.2.3.4-1 illustrates the EAS discovery subscription update procedure between the EEC and the EES.

Pre-conditions:

1.	The EEC has subscribed with the EES for the EAS discovery information as specified in clause 8.5.2.3.2

Figure 8.5.2.3.4-1: EAS discovery subscription update

1.	The EEC sends an EAS discovery subscription update request to the EES. The EAS discovery subscription update request includes the security credentials and the subscription identifier. It may also include EAS discovery filters and/or proposed expiration time for the updated subscription.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to update the subscription information. The EES may utilize the capabilities (e.g. UE location) of the 3GPP core network as specified in clause 8.9.3. If the request is authorized, the EES updated the stored subscription for EAS discovery.

3.	The EES sends an EAS discovery subscription update response to the EEC, which may include the expiration time, indicating when the updated subscription will automatically expire. To maintain the subscription, the EEC shall send an EAS discovery subscription update request prior to the expiration time. If an EAS discovery subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed.

* * * Next Change * * * *

##### 8.5.2.4.4	Subscription update

Figure 8.5.2.4.4-1 illustrates the EAS dynamic information subscription update procedure between the EEC and the EES.

Pre-conditions:

1.	The EEC has subscribed with the EES for the EAS dynamic information as specified in clause 8.5.2.4.2.

Figure 8.5.2.4.4-1: EAS dynamic information subscription update

1.	The EEC sends an EAS dynamic information subscription update request to the EES. The EAS dynamic information subscription update request includes the security credentials and the subscription identifier. It may also include EAS dynamic information filters and proposed expiration time for the updated subscription.

2.	Upon receiving the request from the EEC, the EES checks if the EEC is authorized to update the subscription information. If the request is authorized, the EES updates the stored subscription for EAS dynamic information.

3.	The EES sends an EAS dynamic information subscription update response to the EEC, which may include the expiration time, indicating when the updated subscription will automatically expire. To maintain the subscription, the EEC shall send an EAS dynamic information subscription update request prior to the expiration time. If an EAS dynamic information subscription update request is not received prior to the expiration time, the EES shall treat the EEC as implicitly unsubscribed.

* * * Next Change * * * *

###### 8.6.2.2.3.2	Subscribe

Figure 8.6.2.2.3.2-1 illustrates the subscribe operation between the EAS and the EES for continuous UE location reporting.

Pre-conditions:

1.	The EAS is authorized to discover and to use UE location API provided by the EES;

2.	The EES is authorized to use 3GPP network exposure capability (e.g. Nnef Event Exposure API) for UE location, based on service level agreement with MNO; and

3.	UE Identifier between EAS and the EES is authorized for the UE location API (e.g. appropriate access token is received by EAS based on user's consent).

Figure 8.6.2.2.3.2-1: UE location API: Subscribe operation

1.	The EAS sends UE location subscribe request to the EES for tracking a UE's location continuously. The UE location subscribe request shall include the UE Identifier or UE group Identifier and may include proposed expiration time. It may also include location granularity to indicate requested format of the location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.), which can be understood by the EAS and location QoS.

2.	The EES checks if the request from the EAS is authorized. If authorized, the EES sends success response to the EAS and utilizes the UE location exposure capability of the 3GPP core network as specified in clause 8.9.3. The EES requests continuous location reporting from the 3GPP system to have up to date location information of the UE.

3.	The EES may subscribe to UE expected behaviour analytics (UE mobility) as described in 3GPP TS 23.288 [18].

4.	The response includes the subscription identifier and may include the expiration time, indicating when the subscription will automatically expire. To maintain the subscription, the EAS shall send a UE location subscription update request prior to the expiration time. If a UE location subscription update request is not received prior to the expiration time, the EES shall treat the EAS as implicitly unsubscribed. If it is not authorized, the EES sends failure response with rejection cause.

* * * Next Change * * * *

###### 8.6.2.2.3.4	Subscription update

Figure 8.6.2.2.3.4-1 illustrates the subscription update operation between the EAS and the EES.

Pre-conditions:

1.	The EAS has subscribed to UE location API provided by the EES.

Figure 8.6.2.2.3.4-1: UE location API: Subscription update operation

1.	The EAS sends UE location subscription update request to the EES. The request includes the subscription identifier of the subscription to be. The UE location subscription update request may also include proposed expiration time for the updated subscription, updated Location granularity and Location QoS.

2.	The EES checks if the request from the EAS is authorized. If authorized, the EES updates the subscription request and may update the UE location subscription in the 3GPP Core Network.

3.	The EES may update the subscription to the UE expected behaviour analytics.

4.	The EES sends the UE location subscription update response to the EAS.

* * * Next Change * * * *

##### 8.6.2.3.2	UE location request

Table 8.6.2.3.2-1: UE location request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID | M | Identifies the UE for which location will be reported (e.g. GPSI, or an access token which not only identifies the UE, but also provides the end user's consent for reporting UE's location to a given EAS) |
| Security credentials | M | Security credentials of the EAS |
| Location granularity | O | indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the EAS |
| Location QoS | O | Indicate the location quality of service as described in clause 4.1b of 3GPP TS 23.273 [9]. |

* * * Next Change * * * *

##### 8.6.2.3.4	UE location subscribe request

Table 8.6.2.3.4-1: UE location subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| UE ID (NOTE) | O | Identifies the UE for which location will be reported (i.e GPSI, or an access token which not only identifies the UE, but also provides the end user's consent for reporting UE's location to a given EAS) |
| UE Group ID (NOTE) | O | Identifies a group of UEs (i.e. internal group ID or external group ID) |
| Security credentials | M | Security credentials of the EAS |
| Notification Target Address | M | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Event reporting information | O | Event Reporting Information as specified in Table 4.15.1-1 of 3GPP TS 23.502 [3], e.g. event reporting mode. |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| Location Granularity | O | indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the EAS |
| Location QoS | O | Indicate the location quality of service as described in clause 4.1b of 3GPP TS 23.273 [9]. |
| NOTE: Either UE ID or UE Group ID shall be provided. |  |  |

* * * Next Change * * * *

##### 8.6.2.3.7	UE location subscription update request

Table 8.6.2.3.7-1: UE location subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |
| Security credentials | M | Security credentials of the EAS |
| Event reporting information (NOTE) | O | Event Reporting Information as specified in Table 4.15.1-1 of 3GPP TS 23.502 [3], e.g. event reporting mode. |
| Proposed expiration time (NOTE) | O | Proposed expiration time for the subscription |
| Notification Target Address (NOTE) | O | Notification Target Address of the EAS where the notification is to be sent by the EES |
| Location Granularity (NOTE) | O | indicates format of location e.g. GPS Coordinates, Cell ID, Tracking Area ID, or civic addresses (e.g. streets, districts, etc.) that can be understood by the EAS |
| Location QoS (NOTE) | O | Indicate the location quality of service as described in clause 4.1b of 3GPP TS 23.273 [9]. |
| NOTE: At least one of these IEs is present. |  |  |

* * * End of Changes * * * *