3GPP TSG-SA WG6 Meeting #42-e	S6-210484 Rev1

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

Source:	China Mobile

Title:	Pesudo-CR on MSGin5G UE Configuration

Spec:	3GPP TR 23.700-24 v1.3.0

Agenda item:	8.4

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

- 1. Introduction

How to assign the MSGin5G Service ID to the MSGin5G UE is not described yet in this TR. There are two choices for UE to get MSGin5G Service before registration, pre-defined or by configuration. It is difficult for every  Smart Devices Vendor / MIoT Application / Device Vendor to know the rule of every MSGin5G Service Provider to ensure the MIoT devices/Applications can work in every MSGin5G Service Provider’s system, so pre-defined will not work. In this contribution, configuration procedures for MSGin5G UEis described.

- 2. Reason for Change

Contribute a new solution addressing configuration.

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-24 v1.3.0.

* * * First Change * * * *

### 6.8.5a	MSGin5G UE identity (MSGin5G UE ID)

The MSGin5G UE ID is a unique identifier that represents the MSGin5G UE. A MSGin5G UE ID is associated with a specific MSGin5G UE. The format of MSGin5G UE ID may differ among different VALs. The MSGin5G UE ID may be pre-configured to the MSGin5G UE by its vendor.

* * * Next Change * * * *

## 6.X	Solution X: MSGin5G UE Configuration

### 6.X.1	Description

#### 6.X.1.1	General

When a MSGin5G Client is deployed in a PLMN which provides MSGin5G Service first time, a configuration procedure is needed for it to get the MSGin5G Service configuration information (e.g. MSGin5G Service ID) to use in the future messaging communication, if this ID is not pre-configured.. The configuration procedures may use VAL UE configuration data specified in TS 23.434 [9] as described in this solution. After the configuration procedure, the MSGin5G UE can register to MSGin5G Server with the MSGin5G Service ID got from the configuration procedure.

#### 6.X.1.2	The MSGin5G UE Configuration Function.

A MSGin5G Configuration Function is logical function which is used for configuring MSGin5G functional entities, that is implemented based on Configuration Management aspects from SEAL configuration management Server as specified in 3GPP TS 23.434 [9].

The MSGin5G Configuration Function is utilized for offline configuration (before the MSGin5G UE is sending and receiving messages from the MSGin5G Server) and online configuration (during the MSGin5G UE is sending and receiving messages from the MSGin5G Server).

#### 6.X.1.3	MSGin5G UE configuration

In the MSGin5G UE configuration procedure, the MSGin5G UE acts as Configuration management client specified in 3GPP TS 23.434 [9].

The following steps of configuration management service may apply for the MSGin5G UE:

-	Get VAL UE configuration request specified in subclause 11.3.2.1 of 3GPP TS 23.434 [9];

-	Get VAL UE configuration response specified in subclause 11.3.2.2 of 3GPP TS 23.434 [9];

The usage of the above information flows is clarified as below:

-	The VAL UE ID is the MSGin5G UE ID;

-	The VAL user ID is the MSGin5G service ID specified in clause 6.8;

-	The VAL service ID is the service identifier of MSGin5G Service;

Besides the IEs specified in subclause 11.3.2.1 of 3GPP TS 23.434 [9], the information in table 6.X.1.3-1 is also included in the Get VAL UE configuration request.

Editor’s note:     IEs in this solution are for FFS during normative phase.

Table 6.X.1.3-1: Additional Information in the  Get VAL UE configuration request

| Information element | Status | Description |
| --- | --- | --- |
| MSGin5G UE information | O | Other information needed by the configuration procedure.(NOTE) |
| NOTE: The information can be the device type, device Vendor, etc. It is specified by application provider or MSGin5G service provider and is out of scope of this document. The MSGin5G service provider can configure the MSGin5G UE with different configuration data based on this IE. E.g. all sensors can be configured to a same MSGin5G Server. |  |  |

-	Besides the IEs specified in subclause 11.3.2.2 of 3GPP TS 23.434 [9], the information in table 6.X.1.3-2 is also included in the Get VAL UE configuration response as a part of VAL UE configuration data.

- Table 6.X.1.3-2: Additional information in the Get VAL UE configuration response

| Information element | Status | Description |
| --- | --- | --- |
| MSGin5G service ID | M | MSGin5G service ID assigned to the requesting MSGin5G UE. |
| MSGin5G Server | M | The MSGin5G Server which serves this MSGin5G UE. |
| MSGin5G service specified information | O | The specific information of the MSGin5G Service specified by the MSGin5G Service Provider. (NOTE) |
| NOTE1: e.g. the segment size of MSGin5G Message in this Service Provider, The detailed definition is out of scope of this document. |  |  |

The following additional functional requirement is needed to be added to the Configuration Management Server:

-	The MSGin5G Configuration Function checks whether the MSGin5G UE ID (i.e. VAL UE ID) is included in a former  Get VAL UE configuration request. If so, the MSGin5G Configuration Function included the MSGin5G Service ID assigned to the MSGin5G UE in the former configuration procedure as a part of VAL UE configuration data, otherwise a new MSGin5G Service ID is assigned to the MSGin5G UE, and included in the  VAL UE configuration data. Then the MSGin5G Configuration Function processes the configuration request according to the service policy.

### 6.X.2	Solution Evaluation

This solution provides a MSGin5G UE configuration procedures. The MSGin5G UE may use this solution to obtain MSGin5G Service ID which is mandatory for the future messaging communication.

* * * Next Change * * * *

<Proposed change in revision marks>