3GPP TSG-SA WG6 Meeting #42-BIS-e	S6-210966

- 12th-20th Apr 2021, online															(revision of S6-210757)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.282 | CR | 0270 | rev | 1 | Current version: | 17.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct misuse of the term “transmission control” |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | AT&T |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMCData3 |  |  |  |  |  | Date: |  |  | 2021-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is some confusion In sections 7.3.2 and 7.3.3 (dealing with MBMS) regarding the use of “transmission control messages”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | In sections 7.3.2 and 7.3.3 of 23.282, use the term “transmission control messages” as an example of “application level control signaling”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion and misunderstanding of the spec may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3.2, 7.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Changed CR cat from D to F Revised the changed text |  |  |  |  |  |  |  |  |

**** First change ****

##### 6.6.3.1.4	Media storage function

The media storage function is responsible for the storing of media uploaded by the media storage client of the MCData UE in case of MCData file distribution. It also supports download of stored media by the MCData UE in case of file distribution via media storage client.

The media storage function supports partial download requests of stored media by the MCData UE via media storage client.

**** Next change ****

##### 7.5.2.1.21	MCData group standalone FD over MBMS request

Table 7.5.2.1.21-1 describes the information flow for the MCData group standalone FD request (in subclause 7.5.2.6.2) sent from from the MCData server to another MCData client.

Table 7.5.2.1.21-1: MCData group standalone FD over MBMS request

| Information element | Status | Description |
| --- | --- | --- |
| MCData ID | M | The identity of the MCData user sending file |
| MCData group ID | M | The MCData group ID to which the file is to be sent |
| Conversation Identifier | M | Identifies the conversation |
| Transaction Identifier | M | Identifies the MCData transaction |
| Reply Identifier | O | Identifies the original MCData transaction to which the current transaction is a reply to |
| Disposition indication | O | Indicates whether file download completed reported is expected or not |
| Download indication | M | Indicates mandatory download |
| Content reference | M | URL reference to the content and file metadata information |
| MBMS user service id | M | Id of the MBMS user service delivering the file |
| MBMS content URI | M | URI upon which the content is delivered in the MBMS user service |

**** Next change ****

##### 7.5.2.3.2	Procedure

The procedure in figure 7.5.2.3.2-1 describes the case where an MCData user is downloading a file from the media storage function of the MCData content server.

Pre-conditions:

1.	The MCData user on the media storage client is registered for receiving MCData service.

Figure 7.5.2.3.2-1: File download using HTTP

1.	The user at the media storage client initiates a file download request available at the indicated URL.

2.	The file available at the URL (received in MCData FD request or MCData group standalone FD request) is requested to be downloaded by the media storage client from the media storage function on the MCData content server using a MCData download data request. If emergency indicator is set in received in MCData FD request or MCData group standalone FD request, the the media storage client sets emergency indicator in MCData download data request.

NOTE:	the Media storage client can perform partial download requests to complete the missing parts after an incomplete transmission.

3.	The media storage function on the MCData content server may apply reception control policy and provides a MCData download data response including the file to the media storage client.

**** Next change ****

##### 7.5.2.10.2	Procedure

The procedure in figure 7.5.2.10.2-1 describes the case where a MCData user is initiating group standalone data communication for sending file to multiple MCData users, with or without download completed report request.

Pre-conditions:

1.	The MCData users on the MCData client 1 to n belong to the same group and are already registered for receiving MCData service and affiliated.

2.	File to be distributed is uploaded to media storage function on MCData content server using the procedures defined in subclause 7.5.2.2.

Figure 7.5.2.10.2-1: Group standalone FD using the MBMS download delivery method

1-4.	Steps 1-4 are the same as in the procedure for Group standalone FD using HTTP (7.5.2.6).

5.	The MCData server executes the procedure described in subclause 7.3.5	Use of MBMS user services for file distribution. The MCData server provides, in the MBMS session properties (subclause 5.4 of 3GPP TS 26.348 [19]), the file location on the media storage function and sets the earlier fetch time with a long enough delay so that steps 6 to 8 are executed before the delivery over MBMS.

6.	MCData server initiates the MCData group standalone FD over MBMS request towards each MCData user determined in step 3. The request is sent over unicast or within an MBMS bearer for application level control signalling.

7.	The receiving MCData clients 2 to n notify the user about the incoming MCData group standalone FD request (including file metadata, if present).

8.	MCData client 2 to n automatically sends accepted MCData group standalone FD response when the incoming request included mandatory download indication.

NOTE 1:	When the UE is in idle mode, MCData clients may skip step 8.

9.	The MCData server forwards the MCData group standalone FD responses to the MCData client 1.

NOTE 2:	Step 8 can occur at any time following step 6, and prior to step 10 depending on the conditions to proceed with the file transmission.

10.	The MCData client receives the file delivered over MBMS.

11.	If losses occurred after the end of the file delivery over MBMS, the MCData client downloads the missing parts using the procedures defined in subclause 7.5.2.

Editor's note: whether any enhancements are required on MCData-FD5 are FFS.

12.	The MCData clients, after a successfully reception, initiate a MCData download completed reports for reporting file download completed, if requested by the user at MCData client 1.

13.	The MCData file download completed reports from MCData clients may be stored by the MCData server for download history interrogation from the authorized MCData users. The MCData file download completed report from each MCData user may be aggregated.

14.	Aggregated or individual MCData download completed report is sent by the MCData server to the MCData user at MCData client 1.