3GPP TSG-SA WG6 Meeting #44	S6- 211799

e-meeting, 12th – 20th July 2021	(revision of S6-211595)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0074 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unified support for TSC/TSN services procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 2021-07-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | TR 23.745 Solution #22 incorporates SEAL support for TSN and 5G native TSC services. It is applicable for other vertical applications as well. It is proposed to use that solution for enhancing TS 23.434 NRM function. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow NRM server to use TSN and 5G native TSC for resource management. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NRM does not support TSN and 5G-native TSC. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 14.2.2.2, 14.2.2.3, 14.3.2.X (new), 14.3.2.Y (new), 14.3.2.Z (new), 14.3.2.A (new), 14.3.2.B (new), 14.3.2.C (new), 14.3.2.D (new), 14.3.2.E (new), 14.3.2.F (new), 14.3.2.G (new), 14.4.1, 14.3.X (new), 14.3.X.1 (new), 14.3.X.2 (new), 14.3.X.3 (new), 14.3.X.4 (new), 14.3.Y (new), 14.3.Y.1 (new), 14.3.Y.2 (new), 14.3.Y.3 (new), 14.4.2.X (new), 14.4.2.Y (new), 14.4.2.Z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains".

[3]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[4]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[5]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.286: "Application layer support for V2X services; Functional architecture and information flows".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[10]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[11]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[12]	3GPP TS 23.303: "Proximity-based services (ProSe); Stage 2".

[13]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[14]	3GPP TS 23.002: "Network Architecture".

[15]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.246: "Multimedia Broadcast/Multicast Service (MBMS); Architecture and functional description".

[18]	3GPP TS 23.203: "Policy and charging control architecture".

[19]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[20]	3GPP TS 26.348: "Northbound Application Programming Interface (API) for Multimedia Broadcast/Multicast Service (MBMS) at the xMB reference point".

[21]	3GPP TS 29.214: "Policy and charging control over Rx reference point".

[22]	3GPP TS 29.468: "Group Communication System Enablers for LTE (GCSE_LTE); MB2 Reference Point; Stage 3".

[23]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[24]	IETF RFC 6733 (October 2012): "Diameter Base Protocol".

[25]	ETSI TS 102 894-2 (V1.2.1): "Intelligent Transport Systems (ITS); Users and applications requirements; Part 2: Applications and facilities layer common data dictionaryMultimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[26]	ETSI TS 102 965 (V1.4.1): "Intelligent Transport Systems (ITS); Application Object Identifier (ITS-AID); Registration".

[27]	ISO TS 17419: "Intelligent Transport Systems - Cooperative systems - Classification and management of ITS applications in a global context".

[28]	3GPP TS 26.346: "Multimedia Broadcast/Multicast Service (MBMS); Protocols and codecs".

[29]	3GPP TS 33.434: "Service Enabler Architecture Layer (SEAL); Security aspects for Verticals".

[30]	3GPP TS 29.549: "Service Enabler Architecture Layer for Verticals (SEAL); Application Programming Interface (API) specification; Stage3".

[31]	3GPP TS 23.285: "Architecture enhancements for V2X services".

[32]	IETF RFC 7252: "The Constrained Application Protocol (CoAP)".

[33]	IETF RFC 8323: "CoAP (Constrained Application Protocol) over TCP, TLS, and WebSockets".

[34]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[35]	IEEE Std 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

[36]	IEEE 802.1Q-2018: "IEEE Standard for Local and Metropolitan Area Networks—Bridges and Bridged Networks".

[IEEE8021CB]	IEEE Std 802.1CB-2017: "Frame Replication and Elimination for Reliability".

* * * Next Change * * * *

#### 14.2.2.2	On-network functional model for network resource management for TSN

The architecture for integration of the 5G with TSN [36] is depicted in Figure 14.2.2.2-1. The SEAL Network Resource Management (NRM) server acts as a TSN AF (defined in [10] in clause 5.28.1). Upon request from a VAL server acting as a TSN CNC (defined in [35]) via the NRM-S reference point it configures the TSN flows in the 5GS. In this case the NRM-S supports the IEEE 802.1Qcc management protocol. As a TSN AF the SEAL NRM server interacts with the 5GS PCF over the N5 reference point to configure the 5G QoS and TSCAI parameters in the 5GS.

Figure 14.2.2.2-1: On-network functional model for network resource management for TSN

NOTE: Whether DS-TT and UE are combined or are separate is up to implementation.

#### 14.2.2.3	On-network functional model for network resource management for 5G-native TSC

5G-native TSC refers to time sensitive communication service offered natively (i.e. without integration with a TSN system) by the 5GS for the UEs connected to the 5GS. The architecture for the 5G-native TSC is depicted in Figure 14.2.2.3-1. The SEAL NRM server acts as an AF (referred to as TSC AF) towards the 5G Core Network and performs coordination of QoS flows to fulfill the end-to-end QoS requirements for the UEs involved in the TSC communication.

Upon request from a VAL server via the NRM-S reference point it configures the TSC end-to-end QoS flows in the 5GS. In line with other SEAL service enablers the SEAL NRM server provides a RESTful interface on the NRM-S reference point. As a TSC AF the SEAL NRM server interacts with the NEF orTSCTSF via the N33/Nyx reference point to collect 5GS user plane node information and to configure the 5G QoS and TSCAI parameters in the 5GS. UE-TSC corresponds to DS-TT.

Figure 14.2.2.3-1: On-network functional model for network resource management for 5G-native TSC

* * * Next Change * * * *

#### 14.3.2.X	TSC stream discovery request

Table 14.3.2.X-1 describes the information flow TSC stream discovery request from the VAL server to the NRM server.

Table 14.3.2.X-1: TSC stream discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination UE-TSC ports as defined in IEEE 802.1CB [IEEE8021CB]. |

#### 14.3.2.Y	TSC stream discovery response

Table 14.3.2.Y-1 describes the information flow TSC stream discovery response from the NRM server to the VAL server.

Table 14.3.2.Y-1: TSC stream discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the TSC stream discovery with the underlying network. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination UE-TSC ports as defined in IEEE 802.1CB [IEEE8021CB]. |
| List of traffic specifications | M | The traffic classes supported by the UE-TSCs and available end-to-end latency values and Priority Code Point (PCP) values |

#### 14.3.2.Z	TSC stream creation request

Table 14.3.2.Z-1 describes the information flow TSC stream creation request from the VAL server to the NRM server.

Table 14.3.2.Z-1: TSC stream creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL Stream ID | M | It identifies the VAL stream. |
| Stream specification | M | Stream specification includes MAC addresses of the source and destination UE-TSC ports as defined in IEEE 802.1CB [IEEE8021CB]. |
| Traffic specification | M | It includes Priority Code Point (PCP), MaxFrameInterval, MaxFrameSize, MaxIntervalFrames, MaxLatency, etc., as described in IEEE 802.1Qcc [35] in clause 46.2. |

#### 14.3.2.A	TSC stream creation response

Table 14.3.2.A-1 describes the information flow TSC stream creation response from the NRM server to the VAL server.

Table 14.3.2. A-1: TSC stream creation response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the TSC stream creation. |
| VAL Stream ID | M | It identifies the VAL stream. |

#### 14.3.2.B	TSC stream deletion request

Table 14.3.2.B-1 describes the information flow TSC stream deletion request from the VAL server to the NRM server.

Table 14.3.2.B-1: TSC stream deletion request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the VAL server performing the request. |
| VAL Stream ID | M | It identifies the VAL stream. |

#### 14.3.2.C	TSC stream deletion response

Table 14.3.2.C-1 describes the information flow TSC stream deletion response from the NRM server to the VAL server.

Table 14.3.2.C-1: TSC stream deletion response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result includes success or failure of the network TSC stream deletion. Even in case of failure, the stream is deemed unusable. |
| VAL Stream ID | M | It identifies the VAL stream. |

#### 14.3.2.D	TSN bridge information report

The 5GS bridge management information is defined in TS 23.501 [10], clause 5.28.1 (5GS bridge management).

#### 14.3.2.E	TSN bridge information confirmation

The 5GS bridge management information is defined in TS 23.501 [10], clause 5.28.1 (5GS bridge management).

#### 14.3.2.F	TSN bridge configuration request

The configuration information of 5GS Bridge is defined in TS 23.501 [10], clause 5.28.2 (The configuration information of 5GS Bridge).

#### 14.3.2.G	TSN bridge configuration response

The configuration information of 5GS Bridge is defined in TS 23.501 [10], clause 5.28.2 (The configuration information of 5GS Bridge).

* * * Next Change * * * *

### 14.4.1	General

Table 14.4.1-1 illustrates the SEAL APIs for configuration management.

Table 14.4.1-1: List of SEAL APIs for network resource management

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_NetworkResourceAdaptation | Reserve_Network_Resource | VAL server | Request /Response |
|  | Request_Unicast_Resource | VAL server | Request /Response |
|  | Update_Unicast_Resource | VAL server | Request /Response |
|  | Request_Multicast_Resource | VAL server | Request /Response |
|  | Notify_UP_Delivery_Mode | VAL server | Subscribe/Notify |
|  | TSC_Stream_Discovery | VAL server | Request /Response |
|  | TSC_Stream_Creation | VAL server | Request /Response |
|  | TSC_Stream_Deletion | VAL server | Request /Response |

* * * Next Change * * * *

### 14.3.X 5G-native TSC resource management procedures

#### 14.3.X.1	General

The procedures related to the 5G-native TSC network resource management are described in the following subclauses.

#### 14.3.X.2	TSC stream discovery procedure

The TSC stream discovery procedure is used by the VAL server to discover the availability of resources for TSC communication for the given stream specification (i.e., between the target UEs) prior to creating the stream.

Pre-conditions:

1.	Each UE has an established Ethernet PDU session and UE-TSCs are assigned to VLANs by the NRM server. The traffic classes are configured on each UE-TSC, the same way as defined for DS-TT.

2.	The TSC AF has populated the NRM server management information base with the 5G bridge management and port management information. The latter is related to the Ethernet ports located in the UE-TSCs including bridge delay per UE-TSC Ethernet port pair per traffic class.

3.	NRM server acting as TSC AF has calculated the bridge delay for each port pair, i.e. composed of (ingress UE-TSC Ethernet port, egress UE-TSC Ethernet port) including the DS-TT residence time, packet delay budget (PDB) and propagation delay for both UL from sender UE and DL to receiver UE.

Figure 14.3.X.2-1: TSC stream discovery procedure

1.	The NRM server receives a request from a VAL server on NRM-S reference point to discover the connectivity and available QoS characteristics between UE-TSCs identified by the stream specification.

2.	The NRM server validates the connectivity between the UE-TSCs connected in the same VLAN based on the stored managed objects info base, identifies the traffic classes supported by the UE-TSCs and calculates the end-to-end latency (including the residence time of the DS-TTs, PDBs, and propagation delay).

3.	NRM server responds to the VAL server with the stream specification and a list of traffic specifications with the available end-to-end latency and the traffic classes supported by the UE-TSCs.

#### 14.3.X.3	TSC stream creation procedure

This procedure allows the VAL server to create a TSC stream. The TSC stream creation procedure enables the VAL server to establish TSC connectivity with the required QoS between the UEs connected to the 5GS after the stream discovery procedure.

Pre-conditions:

1.	Each UE has an established Ethernet PDU session for its UE-TSC port MAC address.

2.	Connectivity between the UE-TSCs has been validated by the TSC stream discovery procedure.

3.	NRM server maintains mapping from the traffic class to TSC QoS.

Figure 14.3.X.3-1: TSC stream creation procedure

- NRM server receives a TSC stream creation request from a VAL server to create a TSC stream identified by a VAL Stream ID, between UE-TSC ports in the stream specification and for the traffic class in the traffic specification.

NOTE: It is left for stage 3 to decide whether to combine the QoS monitoring subscription request with the TSC stream creation request.

2.	NRM server calculates the schedule for the VAL Stream ID based on the information collected earlier from the 5GS via N5. It provides per-stream filtering and policing parameters according to IEEE 802.1Q [36] used to derive the TSC QoS information and related flow information. NRM server also provides the forwarding rule according to IEEE 802.1Q [36] used to identify the UE-TSC MAC address of the corresponding PDU session. Based on the 5GS bridge delay information it determines the TSC QoS information and TSC Assistance information for the stream.

3.	NRM server triggers the TSCTSF via N33 request procedure for the TSC stream for both UL QoS flow (sender UE to UPF/bridge) and DL QoS flow (UPF/bridge to receiver UE). The AF request includes the UE-TSC port MAC address, TSC QoS information, TSC Assistance Information, flow bit rate, priority, Service Data Flow Filter containing flow description including Ethernet Packet Filters. The QoS flow will be assigned for the PDU session for the source MAC address for the UL direction and for the PDU session for the destination MAC address for the DL direction.

4.	NRM server sends TSC stream creation response to the VAL server with the result of TSC stream creation for the VAL Stream ID.

#### 14.3.X.4	TSC stream deletion procedure

This procedure allows the VAL server to delete a TSC stream.

Pre-conditions:

1.	The TSC stream is configured in the 5GS and the UE-TSCs.

Figure 14.3.X.4-1: TSC stream deletion procedure

1.	NRM server receives a request from VAL server to delete a TSC stream for with a VAL Stream ID.

2.	NRM server identifies the MAC addresses of the UE-TSCs involved in the stream based on the stored information for the VAL Stream ID.

3.	NRM server triggers the TSCTSF via N33 to remove the UL QoS flows (sender UE to UPF/bridge) and DL QoS flows (UPF/bridge to receiver UE) from the PDU sessions of the UEs referred to by the VAL Stream ID.

4.	NRM server sends TSC stream deletion response to the VAL server with the result of TSC stream deletion for the VAL Stream ID.

* * * Next Change * * * *

### 14.3.Y TSN resource management procedures

#### 14.3.Y.1	General

The procedures related to the TSN network resource management are described in the following subclauses.

#### 14.3.Y.2	5GS TSN Bridge information reporting

Pre-conditions:

1.	There is already an established session between the VAL server which is acting as CNC and the NRM-S acting as TSN-AF.

Figure 14.3.Y.2-1: TSN Bridge information reporting procedure

1.	Acting as the TSN AF the NRM server collects 5GS TSN Bridge information by interaction with the 5GS via the N5 reference point, as described in in TS 23.502 [11] Annex F.1. The NRM server stores the binding relationship between 5GS Bridge ID, MAC address of the DS-TT Ethernet port and also updates 5GS bridge delay as defined in clause 5.27.5 of TS 23.501 [10]. The NRM server retrieves txPropagationDelay and Traffic Class table from DS-TT and it also retrieves txPropagationDelay and Traffic Class table from NW-TT.

2.	Whenever there is a new or updated bridge information the NRM server interacts with the VAL server and reports the TSN Bridge information to register a new TSN Bridge or update an existing TSN Bridge. The VAL server stores the TSN Bridge information and confirms to the NRM server.

#### 14.3.Y.3	5GS TSN Bridge configuration procedure

Pre-conditions:

1.	VAL server acts as TSN CNC and it has stored the 5GS TSN Bridge information received from the NRM server acting as TSN AF.

2.	The NRM server acting as TSN AF has stored the 5GS TSN Bridge information collected from the 5GS, as described in clause 14.3.Y.2.

Figure 14.3.Y.3-1: TSN Bridge configuration procedure

1.	The NRM server receives from the VAL server acting as CNC per-stream filtering, policing parameters and related flow information according to IEEE 802.1Q [36] and it uses them to derive TSN QoS information and related flow information. The TSN AF uses this information to identify the DS-TT MAC address of the corresponding PDU session.

2.	NRM server triggers via N5 the AF request procedure as described in 3GPP TS 23.502 [11] Annex F.2. The AF request includes the DS-TT port MAC address, TSC QoS information, TSC Assistance Information, flow bit rate, priority, Service Data Flow Filter containing flow description including Ethernet Packet Filters.

3.	NRM server responds with a TSN Bridge configuration.

* * * Next Change * * * *

#### 14.4.2.X	TSC_Stream_Discovery

API operation name: TSC_Stream_Discovery

Description: Requesting to discover the connectivity and available QoS characteristics between UE-TSCs.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.X

Outputs: See subclause 14.3.2.Y

See subclause 14.3.X.2 for the details of usage of this API operation.

* * * Next Change * * * *

#### 14.4.2.Y	TSC_Stream_Creation

API operation name: TSC_Stream_Creation

Description: Requesting the NRM to create a TSC stream.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.Z

Outputs: See subclause 14.3.2.A

See subclause 14.3.X.3 for the details of usage of this API operation.

* * * Next Change * * * *

#### 14.4.2.Z	TSC_Stream_Deletion

API operation name: TSC_Stream_Deletion

Description: Requesting the NRM to delete the TSC stream.

Known Consumers: VAL server.

Inputs: See subclause 14.3.2.B

Outputs: See subclause 14.3.2.C

See subclause 14.3.X.4 for the details of usage of this API operation.

* * * End of Change * * * *