3GPP TSG-SA WG6 Meeting #44	S6-211839

e-meeting, 12th – 20th July 2021	(revision of S6-211786, S6-211641)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0078 | rev | 2 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Location based Group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 07-07-2021 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Resolving following EN: “Editor's Note: Group updates due to Users or UEs moving in or out of the location is FFS.” |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) To resolve the EN – added procedure in location management service where VAL server (or any SEAL server) can subscribe to location area of interest to received notification of all UEs within the area or UEs moving in/out of the area. 2) Added location based group upate procedure to upate the group based on notification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EN will remain unresolved in Rel-17. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.7, 10.3.x (NEW), 9.3.2.a (NEW), 9.3.2.b (NEW), 9.3.2.c (NEW), 9.3.2.d (NEW), 9.3.2.e (NEW), 9.3.2.f (NEW), 9.3.2.g (NEW), 9.3.x (New), 9.3.x.1 (New), 9.3.x.2 (New), 9.3.x.3 (New), 9.3.x.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information.

3.	The location management server composes the list of users or UEs within the requested location.

4.	The group management server receives the composed list of users or UEs from the location management server.

5.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group’s configuration information.

NOTE:	The exact policies are out of scope of the present document.

6.	The group management server provides a location-based group creation response to the group management client or the VAL server.

### 10.3.x	Location-based group update

Figure 10.3.x-1 below illustrates the location-based group update.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The location based group has been created as specified in clause 10.3.7.

3.	The group management server has subscribed to monitor UEs moving in or out of the fixed location area as specified in clause 9.3.x.

Figure 10.3.x-1: Location-based group update

1.	The group management server receives location area monitoring notification from location management server as specified in clause 9.3.x.4.

2.	The group management server updates the group members and sends notification as specified in clause 10.3.5.1.

* * * Next Change * * * *

#### 9.3.2.a	Location area monitoring subscription request

Table 9.3.2.a-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription request.

Table 9.3.2.a-1: Location information monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| Location Information criteria | M | Location information to be monitored |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Immediate Report Indicator | O | Indicates whether an immediate location report is required |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled, age of the location, etc) |

Editor’s note: Whether location information critieria supports reference UE location along with application defined proximity range is FFS.

#### 9.3.2.b	Location area monitoring subscription response

Table 9.3.2.b-1 describes the information flow from the location management server to the VAL server for location area monitoring subscription response.

Table 9.3.2.b-1: Location area monitoring subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |
| Subscription identity | O (NOTE) | If subscription is successful, identifies the subscription |
| NOTE: This IE shall be present if subscription status is set to success. |  |  |

#### 9.3.2.c	Location area monitoring notification

Table 9.3.2.c-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.c-1: Location area monitoring notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription |
| UEs currently present (NOTE) | O | List of the identities of all VAL UEs who are currently present in the given location area |
| UEs moved in/out (NOTE) | O | List of UEs either moved in to the location area or moved out of the location area |
| >> UEs moved in | O | List of the identities of the VAL UEs who moved in to the given location area since previous notification. |
| >> UEs moved out | O | List of the identities of the VAL UEs who moved out of the given location area since previous notification. |
| Triggering event | O | Identity of the event that triggered the sending of the notification |
| NOTE: Any one of these shall be present. |  |  |

#### 9.3.2.d	Location area monitoring subscription modify request

Table 9.3.2.d-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription modify request.

Table 9.3.2.d-1: Location information monitoring subscription modify request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| Subscription identity | M | Identifies the subscription |
| Location Information criteria | M | Location information to be monitored |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled, age of the location, etc) |

#### 9.3.2.e	Location area monitoring subscription modify response

Table 9.3.2.e-1 describes the information flow from the location management server to the VAL server for location area monitoring subscription modify response.

Table 9.3.2.e-1: Location area monitoring subscription modify response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription modification status | M | It indicates the request result |

#### 9.3.2.f	Location area monitoring unsubscribe request

Table 9.3.2.f-1 describes the information flow from the VAL server to the location management server for location area monitoring unsubscribe request.

Table 9.3.2.f-1: Location information monitoring unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| Subscription identity | M | Identifies the subscription |

#### 9.3.2.g	Location area monitoring unsubscribe response

Table 9.3.2.g-1 describes the information flow from the location management server to the VAL server for location area monitoring unsubscribe response.

Table 9.3.2.g-1: Location area monitoring unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the request result |

* * * Next Change * * * *

### 9.3.x	Location area monitoring information procedure

#### 9.3.x.1	Location area monitoring subscribe procedure

Figure 9.3.x.1-1 illustrates the high level procedure of location area monitoring subscription request. The same procedure can be applied for location management client and other SEAL servers that would like to subscribe to the list of UEs moving in or moving out of the specific location area.

Figure 9.3.x.1-1: Location area monitoring subscription procedure

1.	The VAL server sends a location area monitoring subscription request to the location management server to subscribe to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.a-1. The location information criteria may include the geographic location information where the UEs moving in or moving out to be monitored

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription request.

3.	The location management server replies with a location area monitoring subscription response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.b-1.

#### 9.3.x.2	Location area monitoring subscribe modify procedure

Figure 9.3.x.2-1 illustrates the high level procedure of location area monitoring subscribe modify request. The same procedure can be applied for location management client and other SEAL servers that would like to modify the subscription to the list of UEs moving in or moving out of the specific location area.

Figure 9.3.x.2-1: Location area monitoring subscription modify procedure

1.	The VAL server sends a location area monitoring subscription modify request to the location management server to modify the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.d-1.

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription modification request.

3.	The location management server replies with a location area monitoring subscription modify response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.e-1.

#### 9.3.x.3	Location area monitoring unsubscribe procedure

Figure 9.3.x.3-1 illustrates the high level procedure of location area monitoring unsubscribe request. The same procedure can be applied for location management client and other SEAL servers that would like to unsubscribe to the list of UEs moving in or moving out of the specific location area.

Figure 9.3.x.3-1: Location area monitoring unsubscribe procedure

1.	The VAL server sends a location area monitoring unsubscrbe request to the location management server to unsubscribe to the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.f-1.

2.	The location management server replies with a location area monitoring unsubscrbe response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.g-1.

#### 9.3.x.4	Location area monitoring notification procedure

Figure 9.3.x.4-1 illustrates the high level procedure of location area monitoring notification. The same procedure can be applied for location management client and other SEAL servers who have subscribe to the list of the UEs moving in or moving out of the specific location area.

Figure 9.3.x.4-1: Location are monitoring notification procedure

1.	One of the event occurs at the location management server as specified in the subscribe request. The location management server identifies the UEs which are moved into the area or moved out of the area based on their location data and time stamp of the location. The location management server also determines that the age of the location of the UEs is not more than the age value specified in the subscribe request, if available. The LMS may report the list of all UEs in the given location or UEs moved in and moved out from the last reported UEs list.

2.	The location management server sends a location area monitoring notification to the VAL server. In the notification message, the location management server includes the information as specified in Table 9.3.2.c-1.

* * * End of Change * * * *