3GPP TSG-SA WG6 Meeting #44	S6-211571_rev1

e-meeting, 12th – 20th July 2021	(revision of S6-21xxxx)

Source:	Huawei, Hisilicon

Title:	AS registration and de-registration procedure

Spec:	3GPP TS 23.554

Agenda item:	8.2

Document for:	Approval

Contact:	Hongxia Hao, haohongxia@huawei.com

1. Introduction

This pCR is to add the procedure of AS registration/de-registration is to be added

2. Reason for Change

The EN that the procedure of AS registration/de-registration is to be added needs to be resolved.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v1.0.0

* * * First Change * * * *

##### 9.1.1.5.2	Registration operation

API operation name: Registration

Description: Do registration or update registration to a MSGin5G Server, by using this API, the AS provides/updates its informations, including the URL used for the message delivery from MSGin5G Server to AS.

Known Consumers: Application Server

Inputs: Refer subclause 9.1.2.3

Outputs: Refer subclause 9.1.2.4

* * * Next Change * * * *

##### 9.1.1.5.3	Deregistration operation

API operation name: Send_MSGin5G_Message

Description: Do deregistration with a MSGin5G Server.

Known Consumers: Application Server

Inputs: Refer subclause 9.1.2.5

Outputs: Refer subclause 9.1.2.6

* * * Next Change * * * *

#### 9.1.2.3	M5S AS registration request

The information flows from the Application Server to the MSGin5G server for registration request includes the information elements in Table 9.1.2.3-1.

Table 9.1.2.3-1: AS Registration request

| Information element | Status | Description |
| --- | --- | --- |
| AS service ID | M | The MSGin5G identifier of the Application Server. This ID is configured before registration. |
| Application ID | M | The identifier of the application specified by the application provider. |
| Authorization Infomation | O | The authorization information used to determine whether the AS is allowed to send the message |
| Security Credentials | O | Security information required for the Application Server to register to the MSGin5G Server. |
| Notification target URI | O | The URL for receiving message, message delivery status report, etc. The MSGin5G Server uses this URL to interact to AS. |
| Application Profile (NOTE) | O | The elements in Application Profile include the information of the Application Server, e.g. application name, application provider, application scenario description, application category, etc. This IE is used by MSGin5G Server to compare with application client information. |
| NOTE: The detailed definition of Application Profile is out of scope of this document. |  |  |

* * * Next Change * * * *

## 8.2	Registration

* * * Next Change * * * *

- 8.2.x	AS Registration

The signalling flow for AS registration is illustrated in figure 8.2.x-1. AS may use the procedure in this clause to do registration. The procedure assumes that the AS is responsible for triggering registration to the MSGin5G Server in order to establish association with the MSGin5G Server to receive MSGin5G services.

NOTE:	The procedure in this clause is optional for AS registration.

Pre-conditions:

1.	The AS has connected to the serving network successfully.

2.	A AS Service ID has been provisioned.

3.	The MSGin5G Server address has been provisioned on the AS.

4.	Both the AS and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.x-1: AS registration

1.	The AS sends a registration request to the MSGin5G Server. The request includes security credentials required for the AS to register to the MSGin5G Server.  The request includes the AS Service ID and AS Profile information as detailed in Table 9.1.2.3-1.

2.	Upon receiving the request, the MSGin5G Server validates the registration request and verifies the security credentials.

3.	The MSGin5G Server sends a registration response to the MSGin5G UE, the respone includes the information elements as specified in Table 9.1.2.4-1. If the registration is successful, the MSGin5G Server stores the AS Profile information.

- 8.2.y	AS De-registration

By de-registering, the AS informs the MSGin5G Server that it wishes to terminate its association with the MSGin5G Server.

The procedure assumes that the AS is responsible for triggering the de-registration from the MSGin5G Server. The signalling flow for AS de-registration is illustrated in figure 8.2.y-1.

Pre-conditions:

1.	The AS is registered to the MSGin5G Server.

Figure 8.2.y-1: AS de-registration

1.	The AS determines to de-register from the MSGin5G Server.

2.	The AS sends a de-registration Request to the MSGin5G Server that includes the AS Service ID, as detailed in Table 9.1.2.5-1.

3.	The MSGin5G Server deletes any applicable MSGin5G Client Profile information that it has stored and replies with a De-registration response as detailed in Table 9.1.2.6-1.