3GPP TSG-SA WG6 Meeting #44-e meeting	S6-211627 Rev3

12th – 20th July 2021, Online	(revision of S6-21xxxx)

Source:	China Mobile

Title:	Pesudo-CR on Architectural requirements

Spec:	3GPP TS 23.554 v1.0.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

- 1. Introduction

Contribute the content of clause 4

- 2. Reason for Change

Contribute the content of clause 4

- 3. Conclusions

<Conclusion part (optional)>

- 4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.554 v1.0.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.262: "Message Service within the 5G System".[3]	GSMA PRD RCC.07: "RCC.07 Rich Communication Suite 9.0 Advanced Communications Services and Client Specification".

[3]	GSMA PRD RCC.07: "RCC.07 Rich Communication Suite 9.0 Advanced Communications Services and Client Specification".

[4]	OMA OMA-ERELD-LightweightM2M-V1_1-20180612-C: "Enabler Release Definition for LightweightM2M".

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals".

[6]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[7]	3GPP TS 23.502: "Procedures for the 5G System".

[8]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[9]	3GPP TS 29.122: "T8 reference point for northbound Application Programming Interfaces (APIs)".

[10]	3GPP TS 29.522: "5G System; Network Exposure Function Northbound APIs; Stage 3".

[11]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access".

[12]	3GPP TS 23.501: "System Architecture for the 5G System (5GS); Stage 2".

[13]	3GPP TS 23.204: "Support of Short Message Service (SMS) over generic 3GPP Internet Protocol (IP) access; Stage 2".

[X]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

* * * Next Change * * * *

# 4	Architectural requirements

## 4.1	General

### 4.1.1	Description

This subclause specifies the general architecture requirements for MSGin5G service.

### 4.1.2	Requirements

[AR-4.1.2-a] The MSGin5G Client shall support one or more applications which need to use the MSGin5G message exchanging capabilities.

[AR-4.1.2-b] The MSGin5G Server shall support one or more Applications Servers which support the MSGin5G message exchanging capabilities.

[AR-4.1.2-c] The MSGin5G messaging related capabilities (e.g. registration, point-to-point messaging, group messaging, message delivery status, etc.) should be exposed as APIs to the Applications.

[AR-4.1.2-d]	The application architecture shall enable the communication between the UEs in different PLMNs. The UEs support MSGin5G service.

## 4.2	UE types

### 4.2.1	Description

This subclause specifies the requirements for UE types supported by MSGin5G service.

### 4.2.2	Requirements

[AR-4.2.2-a] The application architecture shall support the message exchanging between the following UE types:

1.	MSGin5G UE:

1)	light weight constrained devices (e.g. sensors, actuators) and

2)	unconstrained devices with advanced capabilities (e.g. washing machine, micro-ovens)

2.	Legacy 3GPP UE

3.	Non-3GPP UE

[AR-4.2.2-b] The application architecture shall enable the unconstrained devices to act as a UE Message Gateway to constrained devices to communicate with MSGin5G Server.

## 4.3	Communication models

### 4.3.1	Description

This subclause specifies the requirements for MSGin5G communication models.

### 4.3.2	Requirements

[AR-4.3.2-as] The application architecture shall support the following message communication models:

1	Point-to-Point message;

2	Application-to-Point message/ Point-to-Application message;

3	Group message;

4	Broadcast message.

[AR-4.3.2-b]	The application architecture shall enable the communication frequency including sporadic and periodic messaging exchange.

[AR-4.3.2-c]	The application architecture shall enable the handling of messages with different priorities, including normal and high, in different way.

[AR-4.3.2-d]	The application architecture shall support the interconnecting between MSGin5G and other different messaging delivery mechanisms. e.g. SMS specified in 3GPP TS23.040 [X], or RCS message as specified in GSMA PRD RCC.07 [3]).

* * * Next Change * * * *

## 5.2	Application Architecture

Figure 5.2-1 shows the application architecture of the MSGin5G Service.  The MSGin5G Service shall fulfil the service requirements which are enumerated in 3GPP TS 22.262 [2] and the architecture requirements enumerated in clause 4..