3GPP TSG-SA WG6 Meeting #45-BIS-e meeting	  S6-212235

11th – 19th October 2021, Online

Source:	ZTE Corporation

Title:	FFAPP Architectural requirements

Spec:	3GPP TS 23.545 v0.0.0

Agenda item:	9.1

Document for:	Approval

Contact:			Shao Weixiang, shao.weixiang@zte.com.cn

1. Introduction

This contribution provides a proposal for FFAPP Architectural requirements.

2. Reason for Change

Add FFAPP Architectural requirements.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.545 v0.0.0.

* * * First Change * * * *

# 4	Architectural requirements

## 4.1	General

4.1.1	Description

This subclause specifies the general requirements for FF application layer functional architecture.

### 4.1.2	Requirements

[AR-4.1.2-a] The FAE architecture shall support one or more FF applications.

[AR-4.1.2-b] The FAE capabilities shall be offered via APIs to the FF applications.

[AR-4.1.2-c] The FAE capabilities shall enable the FAE client to discover available FF services (as mentioned in 3GPP TS 22.104 [y]).

[AR-4.1.2-d] The FAE client shall be able to communicate to multiple FAE servers.

[AR-4.1.2-e] The SEAL layer shall provide location management service, group management service, network resource management service, network slice capability management service to support FAE capabilities and FF applications.

## 4.2	Supporting time sensitive communication services

4.2.1	Description

This subclause specifies the requirements for FF application layer functional architecture with respect to TSC type of traffic.

### 4.2.2	Requirements

[AR-4.2.2-a] The FF application layer functional architecture shall provide support for time sensitive communication services.

[AR-4.2.2-b] The FF application layer functional architecture shall support control plane mechanisms related to TSN AF (as defined in 3GPP TS 23.501 [x]).

[AR-4.2.2-c] The FF application layer functional architecture shall support mechanisms to adapt the TSN 5GS bridge as per application requirements by utilizing the TSN AF functionalities (as defined in 3GPP TS 23.501 [x]).

* * * Second Change * * * *

# 2	References

[x]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[y]	3GPP TS 22.104: "Service requirements for cyber-physical control applications in vertical domains; Stage 1".

* * * Third Change * * * *

## 3.3	Abbreviations

AF	Application Function

API	Application Programming Interface

SEAL	Service Enabler Architecture Layer for Verticals

TSC	Time Sensitive Communication

TSN	Time Sensitive Networking

UE	User Equipment

* * * End Change * * * *