3GPP TSG-SA WG6 Meeting #46-e	S6-212504

e-meeting, 15th – 23rd November 2021

3GPP TSG-WG SA2 Meeting #147E e-meeting  	S2-2108175

Elbonia, October 18 – 22, 2021	(revision of S2-2107545)

# Title:	Reply LS on MBS broadcast service continuity and MBS session identification

# Response to:	LS (R2-2108914) on the MBS broadcast service continuity and MBS session identification from RAN2

# Release:	Release 17

# Work Item:	NR_MBS-Core, 5MBS

Source:	SA2

To:	RAN2, RAN3

Cc:	SA4, SA6

Contact Person:

Name:	Meng Li

Tel. Number:

E-mail Address:	raymond DOT limeng AT huawei DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None.

1. Overall Description:

SA2 would like to thank RAN2 for the LS. SA2 has discussed the three questions in the LS and provides the feedback of the answers as follows:

Question 1: Can an “MBS ID” (e.g. SAI) be defined for NR MBS for use in SIB and the upper layer signalling (e.g. USD), to avoid too many TMGIs from being broadcast in System Information?

SA2 answer: SA2 agrees to introduce an additional MBS identifier for reducing the volume broadcasted in SIB. The identifier can be associated with several TMGIs to avoid that those TMGIs need to be broadcasted in the System Information.SA2 did not yet conclude whether this additional identifier will denote service areas. SA2 intends to agree related CRs that detail the concept and related procedures in the next meeting. The additional MBS identifier can be used in SIB and the service announcement for Broadcast.

Question 2: Can the mapping between frequency and MBS service/session be provided in the upper layer signalling (e.g. USD), as in LTE SC-PTM?

SA2 answer: SA2 confirms that the information of additional MBS identifier can be provided to the UEs in the upper layer signalling. SA2 does not get consensus whether frequency can be provided in the upper layer signaling.

Question 3: For both broadcast and multicast session, is sessionID parameter or alike required in NR or is TMGI sufficient to identify the MBS session?

SA2 answer: In MBMS, sessionID is used for distinguishing the MBMS session retransmissions, and labeled by BM-SC. The session retransmission is not defined in 5G MBS, and thus the sessionID parameter or alike is not specified.

2. Actions:

To RAN2, RAN3, SA4, SA6.

ACTION: 	SA2 respectfully asks to take the above information into consideration.

3. Date of Next TSG SA WG2 Meetings:

TSG-SA2 Meeting #148E		November 15 – 19, 2021			Elbonia

TSG-SA2 Meeting #149E		February 14 - 25, 2022			Elbonia