3GPP TSG-SA WG6 Meeting #46-e	S6-211267

e-meeting, 15th – 23rd November 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0302 | rev |  | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Connection authorisation mechanisms |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2021-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 3GPP TR 23.700-79 studied the key issue 2 on authorisation for connection of non-3GPP devices with an MC gateway UE and developed solutions in subclause 7.3 (Connection authorisation with an MC server via an MC gateway UE) and subclause 7.5 (Connection authorisation for non-3GPP devices that do not host an MC client). This contribution brings the developed solutions to TS 23.280 for normative specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Section 11.5.1 is updated to capture the information flows and procedures for enabling connection authorisation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.5, 11.5.1, 11.5.1.1 (new), 11.5.1.2 (new), 11.5.1.2.1 (new), 11.5.1.2.2 (new), 11.5.1.2.2.1 (new), 11.5.1.2.2.2 (new), 11.5.1.2.3 (new), 11.5.1.3 (new), 11.5.1.3.1 (new), 11.5.1.3.2 (new), 11.5.1.3.2.1 (new), 11.5.1.3.2.2 (new), 11.5.1.3.3 (new), A.7 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR should be implemented to the spec after implementing S6-212495 Introduction of subclauses to capture MC gateway UE function details. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

## 11.5	Procedures and information flows

### 11.5.1	Connection authorisation mechanisms

#### 11.5.1.1	General

### The connection of non-3GPP devices via an MC gateway UE require authorisation verification by the MC system. Two different types of non-3GPP devices are supported, those which can host MC service client and those which cannot host MC service clients.

#### 11.5.1.2	Connection authorisation for non-3GPP devices that host an MC client

##### 11.5.1.2.1	General

The solution is applied to non-3GPP devices which can host an MC client. The MC server performs authorization for the use of the MC gateway UE by the MC client, i.e. the binding between the MC gateway UE and the MC client is authorized and controlled by the MC server.

##### 11.5.1.2.2	Information flows

###### 11.5.1.2.2.1	Connection authorization request

Table 11.5.1.2.2.1-1 describes the information flow connection authorization request sent from the MC client, which resides on a non-3GPP device, to the MC gateway UE, and from the MC gateway UE to the MC server.

Table 11.5.1.2.2.1-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be authorised. |  |  |

NOTE:	The MC service ID used for MC service authorisation and the GW MC service ID used for connection authorization may have different values. Both identities are configured by the Mission Critical Organisation.

###### 11.5.1.2.2.2	Connection authorization response

Table 11.5.1.2.2.2-1 describes the information flow connection authorization response sent from the MC server to the MC gateway UE, and from the MC gateway UE to the MC client residing on a non-3GPP device.

Table 11.5.1.2.2.2-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Response | M | Result of the connection authorization request, service feasibility, and connection evaluation. |

##### 11.5.1.2.3	Connection authorisation procedure

The procedure for connection authorisation via an MC gateway UE towards an MC server is shown in figure 11.5.1.2.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device.

-	The MC client has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC client has been provided with an appropriate GW MC service ID.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system.

-	The MC service user has selected an MC gateway UE or alternatively, the MC client has performed a selection by internal criteria.

NOTE:	The internal criteria are outside the scope of the present document.

Figure 11.5.1.2.3-1: Connection authorisation with an MC server via an MC gateway UE

1.	The MC client requests connection authorization via the MC gateway UE with an MC server. The client of the MC service user provides the GW MC service ID.

2.	The MC gateway UE checks whether the requested MC service, as indicated by the GW MC service ID, is supported by the MC service gateway. The MC gateway UE may also check whether sufficient resources are avaible or if any other local criteria are met. If the MC service is supported, the procedure continues with step 3, otherwise the procedure proceeds with step 7.

3.	The MC gateway UE sends the connection authorization request to the MC server.

4.	The MC server performs an authorization check, to verify that access via the MC gateway UE is permitted.

5.	The MC server sends the connection authorization response to the MC gateway UE.

6.	The MC gateway UE marks the MC client as authorized to have MC service access via the MC gateway UE.

7.	The MC gateway UE sends the connection authorization response to the MC client.

After successful connection with the MC gateway UE, the MC client has access to the MC server and may continue with user authentication and service authorization.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

#### 11.5.1.3	Connection authorisation for non-3GPP devices that do not host an MC client

##### 11.5.1.3.1	General

The solution is applied to non-3GPP devices which cannot host an MC client. The MC server performs authorization for the use of the MC gateway UE by the MC client, i.e. the binding between the MC gateway UE and the MC client is authorized and controlled by the MC server.

NOTE:	The interworking between the MC client hosted at the MC gateway UE and an MC service user is out of scope of the present document, nevertheless, the connection authorisation performed by the MC gateway UE shall enable the non-3GPP devices to get the access to MC services requested by the service user.

##### 11.5.1.3.2	Information flows

###### 11.5.1.3.2.1	Connection authorization request

Table 11.5.1.3.2.1-1 describes the information flow connection authorization request sent from the MC client, which resides on a MC gateway UE, to the MC server.

Table 11.5.1.3.2.1-1: Connection authorization request

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| NOTE: The GW MC service ID indicates for which MC service the connection is to be authorised. |  |  |

NOTE:	The MC service ID used for MC service authorisation and the GW MC service ID used for connection authorization may have different values. Both identities are configured by the Mission Critical Organisation.

###### 11.5.1.3.2.2	Connection authorization response

Table 11.5.1.3.2.2-1 describes the information flow connection authorization response sent from the MC server to the MC client residing on the MC gateway UE.

Table 11.5.1.3.2.2-1: Connection authorization response

| Information element | Status | Description |
| --- | --- | --- |
| GW MC service ID | M | The GW MC service ID of the requesting MC service user. |
| Result | M | Success or failure of the connection authorization request (authorization successful/failed; service not supported). |

##### 11.5.1.3.3	Connection authorisation procedure

The procedure for connection authorisation of an MC client hosted by the MC gateway UE towards an MC server is shown in figure 11.5.1.3.3-1.

Pre-conditions

-	The MC service user wishes to have access to MC services using a non-3GPP device, where the MC client is hosted by the MC gateway UE.

-	The MC service user has selected an MC gateway UE or alternatively, the non-3GPP has performed a selection by internal criteria.

NOTE:	The internal criteria are outside the scope of the present document.

-	The MC client, which is hosted by the MC gateway UE, has been configured with the necessary parameters needed for connectivity with the MC gateway UE.

-	The MC gateway UE has performed service authorization for one or more MC services with the MC system.

Figure 11.5.1.3.3-1: Connection authorisation of an MC client hosted by an MC gateway UE

1.	The MC client, hosted by the MC gateway UE, requests connection authorization with an MC server by providing the GW MC service ID. The MC gateway UE sends the connection authorization request to the MC server.

2.	The MC server performs an authorization check, to verify that access using the MC gateway UE is permitted.

3.	The MC server sends the connection authorization response to the MC client residing on the MC gateway UE.

The MC client has now access to the MC server and may continue with user authentication and service authorization.

If the MC service user wishes to have access to another MC service, the above procedure is repeated. The MC service user may select a different MC gateway UE for the new MC service, if multiple MC gateway UEs are available.

* * * * * * * NEXT CHANGE * * * * * * *

# A.7	Initial MC gateway UE configuration data

The initial MC gateway UE configuration data is essential to the MC gateway UE to successfully connect MC clients to the MC system. The initial MC gateway UE configuration data can be the same or different across MC gateway UEs.

Data in table A.7-1 is provided to the MC gateway UE during the bootstrap process and can be configured on the MC gateway UE offline using the CSC-11 reference point or via other means.

Table A.7-1: Initial MC gateway UE configuration data (on-network)

| Reference | Parameter description |
| --- | --- |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MC service IDs for MCPTT |
|  | > GW MC service ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MC service IDs for MCVideo |
|  | > GW MC service ID |
| Subclause 5.15 of 3GPP TS 22.280 [3] | List of permitted GW MC service IDs for MCData |
|  | > GW MC service ID |

NOTE:	Configured MC service IDs indicate the support of an MC service.

* * * * * * * END CHANGE * * * * * * *