3GPP TSG-SA WG6 Meeting #46-e	S6-212xxx

e-meeting, 15th – 23rd November 2021	(revision of S6-212648)

Source:	Huawei, Hisilicon

Title:	Key issue on data transmission quality measurements

Spec:	3GPP TR 23.700-34 v0.1.0

Agenda item:	10.9

Document for:	Approval

Contact:	Niranth (namogh@huawei.com), Yaxin (wangyaxin11@huawei.com)

1. Introduction

This pCR proposes a key issue on data transmission quality measurements for maintaining user experience.

2. Reason for Change

SEAL data delivery enabler can provide detailed end to end quality measurement reports since the application data is processed in SEAL data delivery enabler layer and transmitted in SEALDD traffic formats between SEALDD clients and SEALDD servers.

In order to improve user experience, it is beneficial for SEAL data delivery enabler to provide data transmission quality measurement mechanisms (e.g. packet loss, network performance, connectivity status, media stream analytics) to application layer functions. SEAL data delivery enabler can also provide analytics as inputs to other enabler layer (e.g. SEAL, EDGEAPP, NSCALE) for maintaining user experience.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-34 v0.1.0.

* * * First Change * * * *

- 4.x	Key issue #x: Support for data transmission quality measurements

SEAL data delivery enabler can provide detailed end to end quality measurement reports since the application data is processed in SEAL data delivery enabler layer and transmitted between SEALDD clients and SEALDD servers.

NOTE: It is assumed that SEALDD server should be co-located with application server or deployed close to the application server.

SEAL data delivery enabler can employ data transmission quality measurement mechanisms (e.g. packet loss, network performance, connectivity status, media stream analytics) to analyse the end to end (UE to SEALDD server(s) supporting the Application Server) data transmission quality. SEAL data delivery enabler can also provide analytics as inputs to application layer e.g. for maintaining user experience.The following can be studied to support data transmission quality measurements aspects as enabler capabilities:

1.	How to support the data transmission quality measurements in SEALDD server/client?

2.	What APIs should SEALDD enabler provide to application clients/servers to support receiving transmission quality measurements?

3.	What APIs should SEALDD enabler provide to application clients/servers to support configuration of transmission quality measurements?

NOTE:	For media stream aspects, coordination with SA4 is required.