3GPP TSG-SA6 Meeting #47-e	S6-22xxxx

- Online, , 14th Feb 2022 - 22nd Feb 2022						was S6-220029

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0075 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selected T-EAS declaration procedure corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | VODAFONE Group Plc |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP |  |  |  |  |  | Date: |  |  | 2022-02-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Existing text contains the following errors: 1) 8.8.3.7 incorrectly indicates that a response is sent only in the case that the S-EAS is authorized to declare a selected T-EAS "If authorized, the S-EES responds to the received request with Selected target EAS notification declaration response message " 2) current text regarding including T-EES " If the EEC (serving the ACs) has subscribed for the target information notification, then the selected T-EES may be included in the target information " is incorrect. According to Table 8.8.4.10-1: ACR information notification NOTE 4 details of the T-EES shall be included if the T-EES is different from the S-EES. (NOTE 4: This IE shall be included if the selected T-EES is different from the S-EES. Otherwise, it may be skipped.) |  |  |  |  |  |  |  |  |
| Summary of change: |  | missing failure response to T-EAS declaration is included, T-EES information now included when required |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | failure response to T-EAS declaration is not sent, mandatory T-EES indication might be omitted |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 8.8.3.7	Selected T-EAS declaration

Figure 8.8.3.7-1 illustrates the interactions between the S-EAS and the S-EES for the selected T-EAS declaration.

Pre-conditions:

1.	The S-EAS has discovered and selected the T-EAS as described in clause 8.8.3.2.

Figure 8.8.3.7-1: Selected target EAS declaration procedure

1.	The S-EAS sends a Selected target EAS declaration request message to the S-EES. The request includes the identifier and endpoint of the selected T-EAS and may include ACID to indicate which AC the T-EAS will serve.

2.	The S-EES checks whether the requesting EAS is authorized to perform operation. The S-EES then sends a Selected target EAS notification declaration response indicating success or failure;

3.	The S-EES continues to execute procedures as follows:

-	determines the selected T-EES based on the declared T-EAS selection; and

-	if the EEC (serving the ACs) has subscribed for target information notification, then the S-EES sends an ACR information notification message to the EEC as described in clause 8.8.3.5.3 including the ACID which corresponds to the Selected target EAS, and if the T-EES is different from the S-EES also including the selected T-EES.