3GPP TSG-SA WG6 Meeting #47-e	S6-220252

e-meeting, 14th – 22nd February 2022	(revision of S6-xxxx)

Source:	Huawei, Hisilicon

Title:	Key issue on EAS discovery in roaming scenario

Spec:	3GPP TR 23.700-98 v0.4.0

Agenda item:	9.9

Document for:	Approval

Contact:	Yajie Hu <Huyajie1@huawei.com>

1. Introduction

This pCR provides a Key issue on EAS discovery in roaming scenario.

2. Reason for Change

In some deployment case, the EAS#1-3 can be deployed by the ECSP1/PLMN 1 but register to the EES#1 which belongs to the ECSP2/PLMN2 based on the business relationship. In such case, it is not clear how to discover and determine the EAS in the initial EAS discovery procedure. Furthermore, it is also not clear how to discover and determine the T-EAS when ACR happens.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98.

* * * First Change * * * *

## 4.X	Key issue #X: EAS discovery in roaming scenario

In some deployment case, the EAS1-3 can be deployed by the ECSP1/PLMN 1 but register to the EES which belongs to the ECSP2/PLMN2 based on the business relationship. The ECS#1-3 can only provide edge computing service for the user of ECSP1/PLMN 1. In such case, it is not clear how to discover and determine the EAS in the initial EAS discovery procedure. Furthermore, it is also not clear how to discover and determine the T-EAS when ACR happens.

Figure 4.x-1 EAS deployed by different ECSPs or PLMNs

The following study is needed:

- How can EES discover and determine a suitable EAS in the initial EAS discovery procedure?

- How can T-EES discover and determine a suitable T-EAS in the T-EAS discovery procedure when ACR happens across ECSP or PLMN?

* * * End of Change * * * *