3GPP TSG-SA WG6 Meeting #49-e	S6-220189

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

Source:	Qualcomm

Title:	Pseudo-CR on Requirements for common EAS

Spec:	3GPP TR 23.700-98 v0.6.0

Agenda item:	9.8

Document for:	Approval

Contact:	Nishant Gupta <gnishant@qti.qualcomm.com>

1. Introduction

This pCR specifies requirements for a common EAS discovery, such that a group of ACs get service from the same EAS.

2. Reason for Change

Key issue #17.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98 v0.6.0.

* * * First Change * * * *

## 5.x	Common EAS

### 5.x.1	General

This clause specifies requirements for common EAS discovery, such that a group of ACs can get service from the same EAS.

### 5.x.2	Requirements

1.	Edge Enabler Layer shall enable a group of application clients to discover and connect to a common EAS i.e., same EASID, EAS endpoint and EDN.

2.	Edge Enabler Layer shall enable a group of application clients to be redirected to a common EAS.

3.	Edge Enabler Layer shall find a common EAS such that the common EAS provides equivalent KPIs, e.g., latency, bit rate etc. for all the application clients in the group, if required by the application.

4.	Edge Enabler Layer shall ensure that the KPI equivalence is maintained when additional application clients require connecting to the common EAS.

5.	Edge Enabler Layer shall ensure that the KPI equivalence is maintained when one or more application clients stop using the common EAS.

6.	Edge Enabler Layer shall ensure that the KPI equivalence is maintained when application context is relocated due to mobility of one or more UEs.

7.	Edge Enabler Layer shall ensure that the KPI equivalence is maintained when application context is relocated due to non-mobility reasons, such as EAS unavailability.

NOTE 1:	Requirement of ACs to use a common EAS is managed at the application layer.

NOTE 2:	Application clients referenced in the requirement 1 and 2 can be running on the same UE or different UEs.