3GPP TSG-SA WG6 Meeting #49-e	S6-221230

e-meeting, 16th – 25th May 2022	(revision of S6-220723)

Source:	Huawei, Hisilicon

Title:	Solution on key issue#22 on EAS discovery in Edge Node sharing scenario

Spec:	3GPP TR 23.700-98 v0.6.0

Agenda item:	9.8

Document for:	Approval

Contact:	Yajie Hu <Huyajie1@huawei.com>

1. Introduction

This pCR provides a Solution on key issue#22 on EAS discovery in Edge Node sharing scenario.

2. Reason for Change

Based on the business relationship between ECSP1 and ECSP2, the EAS2 in EDN2 can be deployed by the ECSP1. Then the UE can obtain the edge computing service from the EAS2 in EDN2 to keep the service continuity.

Thus in such deployment case, it is not clear how can T-EAS be discovered and determined which allows ECSP subscribers (subscribers of ECSP1) in the T-EAS discovery procedure.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98 v0.6.0.

* * * First Change * * * *

## 7.X	Solution #x: T-EAS discovery in Edge services support across ECSPs

### 7.x.1	Architecture enhancements

None.

### 7.x.2	Solution description

#### 7.x.2.1	General

The following solution corresponds to the key issue#22 on EAS discovery in Edge Node sharing scenario in clause 4.22

#### 7.x.2.2	Procedure

In this solution, the T-EES is responsible to determine the T-EAS base on the  PLMN/ECSP information of the UE and the PLMN/ECSP information of the EAS. With this solution, the T-EES can discovery and determine the considering the PLMN and ECSP information of both the UE and the EAS, and thus S-EES and the S-EAS can get edge computing service from the T-EAS which belongs to the same PLMN or ECSP with the UE.

Pre-conditions:

- ECSP1 and ECSP2 have a service level agreement to share edge services.

- EAS deployed by the Home PLMN or the ECSP1.

Enhancement on the 23.558

- The EAS sends the EAS registration request to the EES. The request contains the HPLMN ID or the home ECSP ID indicating the EAS is deployed or belongs to which PLMN or ECSP.

- The EES sends the EES registration request to the ECS#2. The request contains the supported PLMN ID or supported ECSP ID consicdering the capability of the EAS which registered to the EES.

- The S-EES performs the Retrieve the target EES procedure to the ECS#1, ECS#2 as specified in clause7.4 and clause 7.5. The S-EES obtains the EES information based on the supported PLMN or ECSP information of the EES.

- The S-EES sends the EAS discovery request to the T-EES including the UE home PLMN or ECSP information.

- The T-EES determines the T-EAS considering the PLMN and ECSP information of both the UE and the EAS, e.g. UE home PLMN or ECSP, PLMN or ECSP of the EAS.

- The S-EES sends the EAS discovery resposne to the S-EAS. The response contains the T-EAS information.

### 7.x.3	Solution evaluation

* * * End of Change * * * *