3GPP TSG-SA WG6 Meeting #49-e	S6-221380

e-meeting, 16th – 25th May 2022	(revision of S6-221187)

Source:	Qualcomm

Title:	Pseudo-CR on EDGE-5 APIs

Spec:	3GPP TR 23.700-98 v0.6.0

Agenda item:	9.8

Document for:	Approval

Contact:	Nishant Gupta <gnishant@qti.qualcomm.com>

1. Introduction

This contribution provides a solution for KI#4 on EDGE-5.

2. Reason for Change

EDGE-5 is to enable ACs to request and avail the services of the EEL provided by the EEC. This pCR proposes multiple APIs to support this corresponding to the capabilities of the EEC. Following APIs are proposed:

-	AC registration;

-	EAS discovery;

-	ACR request; and

-	Subscription/Notification for EAS discovery and ACR.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98 v0.6.0

* * * First Change * * * *

## 7.x	Solution #x: EDGE-5 APIs

### 7.x.1	Architecture enhancements

None.

### 7.x.2	Solution description

#### 7.x.2.1	General

This solution corresponds to key issue #4 on EDGE-5 reference point between ACs and the EEC.

The solution provides an API for the AC to request for EEC’s services for edge enablement corresponding to EEC’s capabilities. Using this API, AC may request the EEC for EEL services. The solution defines APIs for one-time request/response operations for EAS discovery and ACR operations. Additionally, The AC can request an AC subscription. The EEC creates the subscription and when required, performs necessary operations such as EAS discovery, ACR etc., delivering notifications to the AC as required.

NOTE:	EEC can initiate any EDGE-1 or EDGE-4 operation without receiving a request or without receiving AC related information from the AC.

#### 7.x.2.1	Procedure

##### 7.x.2.2.1	General

The solution provides the following procedures:

-	AC registration request;

-	EAS discovery request;

-	ACR request;

-	AC subscription; and

-	AC notification.

NOTE:	Details on how the AC and EEC communicate with each other is out of scope.

Editor’s note: Whether a general AC subscription-notification to all EEC services or individual subscriptions for each EEC service or both is required FFS.

##### 7.x.2.2.2	AC registration request

Pre-conditions:

1.	The AC can communicate with the EEC.

NOTE 1:	Details on how the AC and EEC communicate with each other is out of scope.

Figure 7.x.2.2.2-1: AC registration request procedure

1.	The AC sends an AC registration request to the EEC. The request includes the AC profile, AC’s security credentials and optionally the EAS characteristics and EAS discovery filters. The request may also include a list of EEC’s services that AC requires the EEC to handle.

2.	The EEC checks AC’s security credentials and validates the request.

3.	If the request is successfully validated, the EEC registers the information provided in the request and responds back to the AC with AC registration response. The AC registration response includes the list of capabilities supported by the EEC e.g., which service continuity scenarios are supported by the EEC. If the request in step 1 included a list of EEC’s services AC requires from the EEC, the response also includes a list of EEC’s services that AC is authorized for.

NOTE 2:	The mechanisms used for authentication and authorization between AC and EEC is out of scope of this specification. EEC can use local policies, user preferences, ASP services agreement(s) (see clause 9) to authorize the request from the AC.

NOTE 3:	Additional procedures between AC and EEC to update the registration or deregister are necessary.

##### 7.x.2.2.3	EAS discovery request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 7.x.2.2.3-1: EAS discovery request procedure

1.	The AC sends an EAS discovery request to the EEC. The request includes AC’s security credentials and may include AC profile and EAS discovery filters.

2.	The EEC checks AC’s security credentials and validates the request.

3.	If the request is successfully validated, the EEC determines if the required EAS is available or not. The EEC may use information cached or preconfigured at the EEC or may use the EAS discovery procedures to query the EES. If step 1 includes the AC profile or EAS discovery filters, then the EEC may utilize the provided AC profile and filters, to form the EAS discovery request towards EES. If step 1 does not include AC profile and EAS discovery filters, and AC registration was performed, the EEC may utilize the AC profile provided by the AC during AC registration. The EEC also needs to take user privacy requirements, e.g., regarding the disclosure of location information towards the network into account. If required, e.g., when EAS discovery procedures returns a list of EASs, the EEC performs EAS selection based on the information received in step 1 and the AC profile. The EEC can perform EAS discovery with different EESs before selecting an EAS.

Editor’s note:	How the user or the AC can consent, e.g., to the disclosure of location information and the use of the AC ID in the signalling towards the network is FFS.

NOTE 1:	If required, the EEC can perform service provisioning procedure, or EEC registration procedure or both, before performing the EAS discovery procedures. EEC may already have captured EESs and EASs availability for present location; so that the AC’s request (step #1) can be replied to quickly and efficiently.

NOTE 2:	The EEC can aggregate multiple EAS discovery requests from more than one AC before initiating EAS discovery procedure towards the EES.

4.	The EEC responds back to the AC with the EAS discovery response. The response includes the EAS profile(s) of the available EAS(s).

##### 7.x.2.2.4	ACR request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 7.x.2.2.4-1: ACR request procedure

1.	The AC sends an ACR request to the EEC. The request includes AC’s security credentials, type of requested operation (i.e., ACR detection, ACR initiation) and AC profile. If the request is to initiate the ACR, the request may also include the target EAS information.

2.	The EEC checks AC’s security credentials and validates the request.

3.	If the request is successfully validated, the EEC process the request from the AC. If the type of requested operation in the request received in step 1 is:

-	ACR detection, then the EEC determines if ACR is required or not. If it is required, the EEC uses one of the EEC initiated ACR scenarios or launches ACR with action "determination", leading to S-EES executed ACR;

-	ACR initiation, then the EEC uses one of the EEC initiated ACR scenarios and initiate ACR. If the request in step 1 also includes target information, the EEC uses it to select the ACR targets;

-    EEC managed ACR, then the EEC monitors the need for ACR. If need for ACR is detected, then the EEC decides and initiates ACR using one of the EEC initiated ACR scenarios. The EEC also notifies the AC about the imminent ACR and may include the target information.

Editor’s Note: The ACR operations in ACR request and AC subscription request are FFS.

##### 7.x.2.2.5	AC subscription request

Pre-conditions:

1.	The AC can communicate with the EEC.

Figure 7.x.2.2.5-1: AC subscription request procedure

1.	The AC sends an AC subscription request to the EEC. The request includes AC’s security credentials, a list of EEC’s services that AC requires the EEC to handle, and related parameters such as AC profile, EAS discovery filters if the request includes EAS discovery subscription, type of ACR operations (i.e., ACR detection, ACR initiation, ACR monitoring, EEC managed ACR) if the request includes ACR management.

2.	The EEC checks AC’s security credentials and validates the request.

3.	If the request is successfully validated, the EEC creates the subscription and sends an AC subscription response message to the AC. The response includes the list of services that the EEC will handle and related details.

4.	The EEC executes the services e.g., EAS discovery, ACR management, and notifies the AC with information as necessary. The EEC may use locally cached information or configurations while providing services to the AC.

NOTE:	Additional procedures between AC and EEC to update the subscription or to unsubscribe are necessary.

##### 7.x.2.2.6	AC notification

Pre-conditions:

1.	The AC has subscribed to the EEC.

Figure 7.x.2.2.6-1: EAS discovery notification procedure

1.	An event occurs at the EEC that satisfies trigger conditions for notifying a AC e.g., EEC detects a need for Application Context Relocation.

2.	The EEC sends an AC notification to the AC with relevant information related to the event triggered in step 1.

### 7.x.3	Solution evaluation

* * * Next Change * * * *

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI # 1 | KI # 2 | KI # 3 | KI # 4 | KI # 5 | KI # 6 | KI # 7 | KI # 8 | KI # 9 | KI # 10 | KI # 11 | KI # 12 | KI # 13 | KI # 14 | KI # 15 | KI # 16 | KI # 17 | KI # 18 | KI # 19 | KI # 20 | KI # 21 | KI # 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #4 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #5 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #8 |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #9 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Sol #10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #11 |  | X |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #12 |  |  | X |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Sol #13 |  |  |  |  |  | X |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #14 |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #15 |  |  |  |  |  |  |  | X |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Sol #16 |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |
| Sol #17 |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |
| Sol #18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |
| Sol #19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |
| Sol #20 | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #21 |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Sol #22 |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |
| Sol #23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |
| Sol #24 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| Sol #25 |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |
| Sol #26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |
| Sol #27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| Sol #28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| Sol #29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | X |  |  |  |  |  |
| Sol #x |  |  |  | X |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |