3GPP TSG-SA WG6 Meeting #49-e	S6-221229

e-meeting, 16th – 25th May 2022	(revision of S6-220720)

Source:	Huawei, Hisilicon

Title:	Solution for EAS discovery in Edge Node sharing scenario

Spec:	3GPP TR 23.700-98 v0.6.0

Agenda item:	9.8

Document for:	Approval

Contact:	Yajie Hu <Huyajie1@huawei.com>

1. Introduction

This pCR provides a Solution on EAS discovery in Edge Node sharing scenario.

2. Reason for Change

The following open issues are identified in Key issue #22 EAS discovery in Edge Node sharing scenario:

Open Issues :

- How can EES discover and determine a EAS which allows (subscribers of OP B) to avail its services?

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98 v0.6.0.

* * * First Change * * * *

## 7.X	Solution #x: EAS discovery in Edge Node sharing scenario

### 7.x.1	Architecture enhancements

None.

### 7.x.2	Solution description

#### 7.x.2.1	General

The following solution corresponds to the key issue#22 on EAS discovery in Edge Node sharing scenario in clause 4.22

#### 7.x.2.2	Procedure

In this solution, the EES is responsible to determine the EAS base on the  PLMN/ECSP information of the UE and the PLMN/ECSP information of the EAS. With this solution, the EES can discovery and determine the considering the PLMN and ECSP information of both the UE and the EAS, and thus UE can get edge computing service from the EAS which belongs to the same PLMN or ECSP with the UE.

Pre-conditions:

- ECSP1 and ECSP2 have a service level agreement to share edge services.

- EAS deployed by the Home PLMN or the ECSP1.

Enhancement on the 23.558

- The EAS sends the EAS registration request to the EES. The request contains the home ECSP ID (e.g HPLMN ID) or indicating the EAS is deployed or belongs to which PLMN or ECSP.

- The EES sends the EES registration request to the ECS#2. The request contains the supported ECSP ID (e.g supported HPLMN ID) consicdering the capability of the EAS which registered to the EES.

- The EEC performs the service provisioning procedure to the ECS#1, ECS#2 as specified in clause7.4 and clause 7.5. The EEC obtains the EES information based on the ECSP (e.g supported PLMN ID) information of the EES.

- The EEC sends the EAS discovery request to the EES including the UE home ECSP (e.g HPLMN ID information.

- The the EES determines the EAS considering the PLMN and ECSP information of both the UE and the EAS, e.g. UE home PLMN or ECSP, PLMN or ECSP of the EAS.

### 7.x.3	Solution evaluation

* * * End of Change * * * *