3GPP TSG-SA WG6 Meeting #49-e	S6-221155

e-meeting, 16th – 25th May 2022	(revision of S6-22xxxx)

Source:	Lenovo

Title:	ADAE layer architecture update

Spec:	3GPP TR 23.700-36 v0.2.0

Agenda item:	9.12

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This contribution proposes an update to ADAE layer functional architecture.

2. Reason for Change

This contribution proposes an update to the fucntional architecture to:

- add the relation with SEAL (ADAES can be a new service within SEAL framework)

- provide more detains on the network interfaces

- provide a new subclause on the relation to 3GPP data analytics existing framework.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-36 v0.2.0.

* * * Change * * * *

## 5.3	Functional Architecture

### 5.3.1	General

This clause provides the overall functional architecture description, which includes the on-network and off-network functional models.

5.3.2	On-network Functional Architecture

For the on-network functional architecture, both service-based representation and reference point representation are provided.

Figure 5.3.2-1 depicts the application data analytics enablement architecture in the non-roaming case, using the reference point representation showing how various entities interact with each other.

Figure 5.3.2-1 Architecture for application data analytics enablement – reference points representation

The application data analytics enablement client communicates with the application data analytics enablement server over the ADAE-UU reference point. The application data analytics enablement client provides the support for application data analytics enablement functions to the VAL client(s) over ADAE-C reference point. The VAL server(s) communicates with the application data analytics enablement server over the ADAE-S reference point. The application data analytics enablement server, acting as AF, may communicate with the 5G Core Network functions (over N33 reference point to NEF and N6 reference point to UPF) and OAM (over ADAE-OAM interface).

Figure 5.3.2-2 exhibits the service-based interfaces for providing and consuming application data analytics enablement services. The application data analytics enablement server could provide service to VAL server and ADAE client through interface SAdaee.

Figure 5.3.2-2 Architecture for application data analytics enablement – Service based representation

Figure 5.3.2-3 illustrates the service-based representation for utilization of the 5GS network services based on the 5GS SBA specified in 3GPP TS 23.501 [4].

Figure 5.3.2-3 Architecture for application data analytics enablement utilizing the 5GS network services based on the 5GS SBA – Service based representation

The application data analytics enablement server can be deployed as a SEAL server; hence enhancements to SEAL architecture (as specified in TS 23.434 [5]) are needed to incorporate the ADAE service. Figure 5.3.2-4 illustrates the service-based representation including ADAE server as part of the SEAL framework.

Figure 5.3.2-4 SEAL functional model representation using service-based interfaces and including ADAE function

### 5.3.3	Off-network Functional Architecture

Figure 5.3.3-1 illustrates the generic off-network functional model for ADAE.

Figure 5.3.3-1: Generic off-network functional model

In the vertical application layer, the VAL client of UE1 communicates with VAL client of UE2 over VAL-PC5 reference point. An application data analytics enablement client of UE1 interacts with the corresponding application data analytics enablement client of UE2 over ADAE-PC5 reference points. The UE1, if connected to the network via Uu reference point, can also act as a UE-to-network relay, to enable UE2 to access the VAL server(s) over the VAL-UU reference point.

If the ADAE server is deployed as SEAL server, the off network functional architecture is similar to SEAL off-network architecture (as specified in TS 23.434 [5]).

### 5.3.4	ADAE internal Architecture based on 3GPP data analytics framework

In 3GPP SA2 (TS 23.288 [2]), the following entities have been defined as part of the data analytics framework:

- NWDAF provides network data analytics services at the 5GC

- DCCF coordinates the collection and distribution of data requested by NF/AF consumers. Data Collection Coordination is supported by a DCCF. Data Consumers can send requests for data to the DCCF rather than directly to the NF/AF Data Source.

- ADRF stores historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the consumer. After the consumer obtains data and/or analytics, consumer may store historical data and/or analytics in an ADRF. Whether the consumer directly contacts the ADRF or goes via the DCCF or via the Messaging Framework is based on configuration.

ADAE server can reuse the existing 3GPP data analytics framework for the data collection coordination, delivery and storage as provided by DCCF and ADRF functionalities. As illustrated in Figure 5.3.4-1, A-DCCF and A-ADRF can be defined as functionalities within the internal ADAE architecture and can offer similar functionality as proposed in 5GC but at application layer.

Figure 5.3.4-1 illustrates the generic functional model for ADAE when re-using the existing data analytics model.

Figure 5.3.4-1: Generic functional model based on network data analytics model

In this model, an Application layer - Data Collection and Coordination Function (A-DCCF) is used to fetch data or put data into an Application level entity (e.g. A-ADRF, Data Source). Such A-DCCF coordinates the collection and distribution of data requested by ADAE server (over ADCCF-1, ADAE-X). ADAE server can also directly interact with the Data Sources via ADAE-Y.

Also, Application layer – Analytics and Data Repository Function (A-ADRF) can be used to store historical data and/or analytics, i.e. data and/or analytics related to past time period that has been obtained by the ADAE server (via AADRF-1) or other NFs/NWDAF. ADAE server can also fetch historical data from ADRF. Whether the ADAE server directly contacts the ADRF or goes via the A-DCCF is based on configuration.

Data Sources can be 5GS data sources (5GC, OAM) or enablement layer data sources (SEAL, EEL) or external data sources at the DN side (VAL server/EAS) and VAL UEs.  A-DCCF and A-ADRF can be used only for interacting with certain data sources (e.g. 5GC, OAM) based on configuration, and can be hidden from the VAL layer.

NOTE: If the Data Source is the VAL UE, then the data collection mechanism shall reuse the SA4 mechanism based on EVEX study (TS 26.531 [7]).

* * * Next Change * * * *

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

ADAE	Application Data Analytics Enabler

A-ADRF	Application layer - Analytical Data Repository Function

A-DCCF	Application layer - Data Collection and Coordination Function

ADRF	Analytical Data Repository Function

ASP	Application Service Provider

DCCF	Data Collection and Coordination Function

MDAS		Management Domain Analytics Service

NWDAF	Network Data Analytics Function

OAM	Operation, Administration and Maintenance

SEAL		Service Enabler Architecture Layer

VAL	Vertical Application Layer

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[3]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[4]	3GPP TS 23.501: ""System Architecture for the 5G System; Stage 2".

[5]		3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[7]	3GPP TS 26.531: “Data Collection and Reporting; General Description and Architecture”

* * * End of Changes * * * *