3GPP TSG-SA WG6 Meeting #49-e	S6-221382

e-meeting, 16th – 25th May 2022	(revision of S6-221189)

Source:	Qualcomm

Title:	Pseudo-CR on Requirements for common EAS

Spec:	3GPP TR 23.700-98 v0.6.0

Agenda item:	9.8

Document for:	Approval

Contact:	Nishant Gupta <gnishant@qti.qualcomm.com>

1. Introduction

This pCR specifies requirements for a common EAS discovery, such that a group of ACs get service from the same EAS.

2. Reason for Change

Key issue #17.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98 v0.6.0.

* * * First Change * * * *

## 5.x	Common EAS

### 5.x.1	General

This clause specifies requirements for common EAS discovery, such that a group of ACs can get service from the same EAS.

### 5.x.2	Requirements

1.	Edge Enabler Layer shall enable a group of application clients to discover a common EAS i.e., same EASID, EAS endpoint and EDN.

Editor’s note:	Impact of single EDN vs. across EDN is FFS.

2.	Edge Enabler Layer shall lookup for a common EAS such that the common EAS provides similar KPIs for all the application clients in the group, if required by the application.

NOTE 1:	What KPIs e.g. latency, bit rate etc. EEL uses and how will these KPIs are obtained to lookup for the common is subject of the solutions of this key issue.

3.	Edge Enabler Layer shall enable a group of application clients to be redirected to a different common EAS.

NOTE 2:	Redirection to a different common EAS can happen due to several reasons such as additional ACs requiring to connect to the common EAS, EAS unavailability etc.

NOTE 3:	Requirement of ACs to use a common EAS based on application client and EAS requirements is determined and managed at the application layer.

NOTE 4:	Application clients can be running on the same UE or different UEs.