3GPP TSG-SA WG6 Meeting #49-e	S6-221425

e-meeting, 16th – 25th May 2022	(revision of S6-220896, 1228)

Source:	Huawei, Hisilicon

Title:	Solution for T-EAS discovery for linkage of AC with multiple EAS(s)

Spec:	3GPP TR 23.700-98 v0.6.0

Agenda item:	9.8

Document for:	Approval

Contact:	Yajie Hu <Huyajie1@huawei.com>

1. Introduction

This pCR update the solution on Bundled EASs

2. Reason for Change

The following open issues are identified in Key issue #18: Linkage between EASs :

Open issue:

-	How can the EEL identify linkages between the EASs?

-	What are the impacts on EEL procedures due to linkage between the EASs e.g. when the linked EASs are served by the same EES and require ACR due to UE mobility?

For the second bullet, In this solution, the ECS is responsible to determine the T-EES which the linkage T-EASs all registered in this T-EES in the retrieve T-EES procedure. Then the T-EES can discover the linkage T-EAS and return the linkage T-EASs information to the S-EES. With this solution, the linkage of T-EASs can be discovered when AC needs to  connect to the linkage EASs.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-98.

* * * First Change * * * *

## 7.26	Solution #26: Bundled EASs

### 7.26.1	Architecture enhancements

None.

### 7.26.2	Solution description

#### 7.26.2.1	General

This solution corresponds to KI#18 on linkages between EASs.

Editor's Note:	The solution assumes that the EASs belonging to a bundle is part of the same EDN. Support for EASs spread across EDNs is FFS.

The solution extends the AC profile and EAS profile to introduce the following new IEs and their handling by different EEL functions:

-	EAS bundle information: EAS bundle information (e.g., EAS bundle ID, information of associated EASs) establishes an association between the EASs. Edge Enabler Layer handles the Edge Application Servers belonging to the same bundle as required by the bundle requirements. When included in the EAS profile, EAS Bundle information denotes the bundle to which the EAS belongs. When included in the AC profile EAS Bundle information is used to perform different Edge Enabler Layer operations, such as EAS discovery.

-	EAS bundle requirements: This IE provides the Edge Enabler Layer the requirements that apply to the bundle of EAS. The requirements may include combined discovery and combined ACR.

Both, EAS bundle information and EAS bundle requirements, are provided by the ASP. EAS bundle information can be a list of EASs or a bundle ID, however, detailed format of the EAS bundle ID is out of scope.

Table 7.26.2.1-1: AC Profile

| Information element | Status | Description |
| --- | --- | --- |
| ACID | M | Identity of the AC. |
| AC Type | O | The category or type of AC (e.g. V2X). This is an implementation specific value. |
| Preferred ECSP list | O | When used in a service provisioning request, this IE indicates to the ECS which ECSPs are preferred for the AC. The ECS may use this information in the selection of EESs. |
| AC Schedule | O | The expected operation schedule of the AC (e.g. time windows) |
| Expected AC Geographical Service Area | O | The expected location(s) (e.g. route) of the hosting UE during the AC's operation schedule. This geographic information can express a geographic point, polygon, route, signalling map, or waypoint set. |
| AC Service Continuity Support | O | Indicates if service continuity support is required or not for the application. The IE also indicates which ACR scenarios are supported by the AC and which of these are preferred by the AC. |
| List of EASs | O | List of EAS that serve the AC along with the service KPIs required by the AC |
| > EASID | M | Identifier of the EAS |
| > Expected AC Service KPIs | O | KPIs expected in order for ACs to receive currently required services from the EAS, as described in Table 8.2.3-1 |
| > Minimum required AC Service KPIs | O | Minimum KPIs required in order for ACs to receive meaningful services from the EAS, as described in Table 8.2.3-1 |
| EAS bundle information | O | Information of the bundle of EASs of which AC wants services. |

Table 7.26.2.1-2: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| List of EAS bundle information | O | List of EAS bundles to which the EAS belongs and related bundling requirements. |
| > EAS bundle information | M | Identifier of the bundle to which EAS belongs. |
| > EAS bundle requirements | O | Requirements associated with the bundle |
| >> Coordinated EAS discovery | O | Indicates if EAS discovery request for one of the bundled EAS is received, then EAS discovery response should include information of all the EASs belonging to the bundle. |
| >> Coordinated ACR | O | Indicates if EAS ACR is initiated for one of the bundled EAS, then ACR should be initiated for all the EASs belonging to the bundle. The IE may further indicate what actions must be taken if ACR for one or more bundled EAS fails e.g., ACR for all other EAS that are part of the bundle must be cancelled or not. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [2]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

#### 7.26.2.2	Handling of EAS bundle information by the EEC

If AC profile contains an EAS bundle information, the EEC uses it while performing operations on EDGE-1 interface.

#### 7.26.2.3	Handling of EAS bundle information and EAS bundle requirements by the EES

The EES may receive the EAS bundle information from the EEC, EAS or another EES and the EAS bundle requirements from the EAS.

Upon receiving the EAS bundle information and EAS bundle requirements as part of the EAS registration request from the EAS, the EES stores the information and associates the EAS with other EASs providing the same EAS bundle information. The EAS bundle requirements are used by the EES while providing services on EDGE-1, EDGE-3 and EDGE-9 interfaces.

Upon receiving the EAS bundle information as part of the:

-	EAS discovery request from the EEC, EAS or another EES, and depending on the EAS bundle requirements the EES provides in the EAS discovery response information of all the EASs which are part of the EAS bundle.

Editor's Note: How EES provides EAS discovery response for all EASs in a bundle is FFS.

-	ACR request from the EEC or the EAS, and depending on the EAS bundle requirements the EES takes appropriate ACR related action (e.g., initiate ACR, cancel ACR etc.) for all the EASs which are part of the EAS bundle.

-  Retrieve T-EES request from the S-EES, and depending on the EAS bundle information, the ECS provides in the Retrieve T-EES response inforamtin of the T-EES(s within the same EDN for the bundle EAS.

#### Enhancements to 3GPP TS 23.558 clause 8.8.4.6	Retrieve EES request

#### 8.8.4.6	Retrieve EES request

Table 8.8.4.6-1 describes the information elements to retrieve T-EES information from the ECS.

Table 8.8.4.6-1: Retrieve EES request

| Information element | Status | Description |
| --- | --- | --- |
| EESID | M | Unique identifier of the EES. |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| EASID | M | The EASID. |
| EAS bundle information | O | Information of the bundle to which EAS belongs. |
| Target DNAI | O | The target DNAI information which can be associated with potential T-EES(s) and/or T-EAS(s). |
| UE Identifier | O | The identifier of the UE (i.e. GPSI or identity token) |
| UE location | O | The location information of the UE. The UE location is described in clause 7.3.2. |

* * * End of Change * * * *