3GPP TSG-SA WG6 Meeting #50-e	S6-222026

e-meeting, 22nd – 31st August 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0061 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for constrained devices using gateway UE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2022-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clause 8.11.4 and 8.11.5: - A constrained device is only allowed to send point-to-point, point-to-application and group messages and is not allowed to send a topic message or a broadcast message, but there seems to be no reason for this restriction. Clause 8.11.4: - In step 2 the case when the maximum size is not exceeded is missing. Clause 8.11.5: Message segregation is missing from step 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clause 8.11.4 and 8.11.5: - Allow the procedure not to exclude any message type (changes to figure 8.11.4-1 and table 8.11.4-1 and step 2 in 8.11.4 and table 8.11.5-1). Clause 8.11.4: - Include case where max size is not exceeded. Clause 8.11.5: - Added possibility for UE1 to segregate messages. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Messaging procedure for constraint devices cannot be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.4, 8.11.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * Changes * * *

### 8.11.4	Constrained device sending message using Gateway UE

The signalling flow for the Application Client-2 on the UE-2 (which is a Constrained UE) to send message using gateway UE functionality on MSGin5G UE-1 is illustrated in figure 8.11.4-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The UE-2 is Constrained UE and is successfully registered with MSGin5G UE-1 acting as a gateway UE.

Figure 8.11.4-1: Application Client-2 on UE-2 sends message using gateway UE functionality on MSGin5G UE-1

1)	An Application Client-2 on the UE-2 sends a request to send MSGin5G message to the MSGin5G Client-1. The information elements defined in Table 8.11.4-1 are included in the message.

Table 8.11.4-1: Information elements for Request to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Recipient address | O | Indicates details of the recipient. This IE is mandatory to initiate Point-to-Point messaging and Point-to-Application messaging. |
| Recipient Group address | O | Indicates target group. This IE is mandatory to initiate Group messaging. |
| Broadcast Area ID | O | The service identifier of the Broadcast Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. |
| Messaging Topic | O | Indicates which Messaging Topic this message is related to. This IE is mandatory for a message distribution based on topic and is not present in other message scenarios. |
| Application ID | O | Identifies the application(s) for which the payload is intended. |
| Payload | M | Payload of the message. MSGin5G Server/Client is unaware of the content. |
| Delivery status required | O | Indicates whether delivery status is required or not |

Editor's note:	If Priority type needs to be communicated from application clients is FFS.

2)	Upon receiving the request from the Application Client-2 in UE-2, the MSGin5G Client-1 constructs the related IEs specified in table 8.3.2-1 and sends an MSGin5G message based on these IEs.

a)	if the size of the received message exceeds the maximum allowed packet size, the MSGin5G Client-1 sends the message as specified in clause 8.5; or

b)	If the size of the received message does not exceed the maximum allowed packet size, the MSGin5G Client-1 sends the message as specified in clause 8.7; or

c)	If the size of the received message does not exceed the maximum allowed packet size, the MSGin5G Client-1 may apply message aggregation as specified in clause 8.4 before sending the message as specified in clause 8.7.

NOTE 1:	The MSGin5G Client-1 may also reject the request to send MSGin5G message based on local condition (like available power or connectivity to access network or any other reason outside the scope of 3GPP).

3)	The MSGin5G Client-1 sends response to sent MSGin5G message to Application Client-2 on UE-2. The response message includes information elements as specified in Table 8.11.4-2.

Table 8.11.4-2: Information elements for Response to send MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |
| Failure reason | O | Indicates failure reason. This IE is included only if Result IE is set to failure. |

NOTE 2:	If the MSGin5G Client-1 has decided to reject the request to send the message or the MSGin5G Client-1 received reject response from MSGin5G Server in step 2, the MSGin5G Client-1 sends failure response to the Application Client-2 and stops performing further steps.

4)	If delivery status is requested while sending the message in step 1, the MSGin5G Client-1 may receive MSGin5G message delivery status report from the MSGin5G Server.

5)	Upon receiving the MSGin5G message delivery status report, the MSGin5G Client-1 sends the message delivery status report to the Application Client-2 on UE-2. The message delivery status report includes information elements as specified in Table 8.11.4-3.

Table 8.11.4-3: Information elements for MSGin5G message delivery status

| Information element | Status | Description |
| --- | --- | --- |
| Delivery status | M | Indicates delivery status |

### 8.11.5	Constrained device receiving message via Gateway UE

The signalling flow for Application Client-2 on the UE-2 (which is Constrained UE) to receive message (Group message, Point-to-Point message or Application-to-Point message) using gateway UE functionality on MSGin5G UE-1 is illustrated in figure 8.11.5-1.

Pre-conditions:

1.	The MSGin5G UE-1 is connected to an access network that provides connectivity to the MSGin5G Server.

2.	The UE-2 is Constrained UE and is successfully registered with MSGin5G UE-1 acting as a gateway UE.

Figure 8.11.5-1: UE-2 receives message using gateway UE functionality on MSGin5G UE-1

1)	The MSGin5G Client-1 receives either a Group message or a Point-to-Point message or Application-to-Point message as specified in clause 8.2.3 for the Application Client-2 on UE-2 for which the MSGin5G Client-1 is acting as Gateway UE. The MSGin5G Client-1 performs reassembly if the received message is segmented or segregation if the received message is an aggregated message. The MSGin5G Client-1 may also perform segment recovery procedure as specified in clause 8.4.6 to recover missing segments.

2)	Upon successfully receiving a message for the Application Client-2 on UE-2, the MSGin5G Client-1 sends message received request to Application Client-2 based on Application ID on the UE-2. The request includes information elements as specified in Table 8.11.5-1.

Table 8.11.5-1: Information elements for Message received request

| Information element | Status | Description |
| --- | --- | --- |
| Originator address | O | Indicates details of the originator. This IE is mandatory for Point-to-Point messaging and Application-to-Point messaging. |
| Group Service address | O | Indicates group for which the message is received. This IE is mandatory for Group Message. |
| Messaging Topic | O | Indicates the Topic for which the message is received. This IE is mandatory for a Topic Message. |
| Payload | M | Payload of the message. MSGin5G Server/Client is unaware of the content. |
| Delivery status required | O | Indicates whether delivery status is required or not |
| Priority type | O | Application priority level requested for this message as specified in Table 8.3.2-1.. |

3)	Upon successfully receiving the message, the Application Client-2 on UE-2 sends the message received response to the MSGin5G Client-1.

4)	If delivery status is requested in the received message, the Application Client-2 on UE-2 sends message delivery status to the MSGin5G Client-1.

5)	Upon receiving the delivery status, the MSGin5G Client-1 sends message delivery status report to the MSGin5G Server.

* * End of Changes * * *