3GPP TSG-SA WG6 Meeting #50-e	S6-222445

e-meeting, 22nd Aug – 31st Aug 2022	(revision of S6-222178)

Source:	Samsung

Title:	Pseudo-CR on Ad hoc group communication involving multiple MC systems

Spec:	3GPP TR 23.700-76 V1.1.0

Agenda item:	9.3

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR proposes the solution for Key issue #1 and in particular to support the ad hoc group communication involving multiple MC service systems.

Also the IF related to sharing the security key material is confusing because of the nomenclature used.

2. Reason for Change

Below EN is captured to support ad hoc group communication involving multiple MC service systems.

Editor's note:	Ad hoc group communication procedure involving multiple MC service systems and usage of functional alias as target are FFS.

This pCR addresses the above EN by proposing the solution and relevant description.

This pCR renames the existing IF names related to the sharing of key material by the initiator of the AHGC.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-76 V1.1.0.

* * * First Change * * * *

#### 7.1.3.2	Ad hoc group communication involving multiple MC system

##### 7.1.3.2.1	Ad hoc group communication setup – Participants list provided by the initiator

Figure 7.1.3.2.1-1 below illustrates the ad hoc group communication setup procedure initiated by an authorized user involving the MC service users from multiple MC system.

Pre-conditions:

- The authorized user at MC service client 1 wants to invite MC service users at MC service client 2, MC service client 3 and MC service client 4 for the ad hoc group communication.

- MC service client 1 and MC service client 2 are registered to the MC service provider 1. MC service client 3 and MC service client 4 are registered to MC service provider 2.

2.	Number of participants being invited for the communication is within the limit.

3.	End-to-End encryption is supported for this ad hoc group communication.

4.	MC service client 1 has all the security related information required to communicate with the participants of the ad hoc group communication.

Figure 7.1.3.2.1-1: Ad hoc group communication setup involving multiple MC systems

1-5a.	Same as described in sub-clause 7.1.3.1.1.

5b-5c.	The MC service server sends the ad hoc group communication requests for MC service client 3 and MC service client 4 to the MC service server 2. While sending the ad hoc group communication requests, the MC service server shall remove the information elements that are not required to be conveyed to the target MC service clients (e.g. Block sharing participants list indicator, MC service ID list of the users who are required to acknowledge) and shall include the security key material as received in steps 4b and 4c.

6a-6b.	The MC service server 2 forwards the ad hoc group communication requests to the MC service client 3 and MC service client 4.

7a-7c.	The receiving MC service clients are notified about the incoming ad hoc group communication.

8.	The MC service client 2 accept the ad hoc group communication request and send ad hoc group communication response to the MC service server 1.

9a-9b.	The MC service client 3 and MC service client 4 accepts the ad hoc group communication request and sends ad hoc group communication responses to the MC service server 2.

10a-10b.	The MC service server 2 forwards the responses to the MC service server 1.

11.	The MC service server sends the ad hoc group communication response to MC service client 1 through the signalling path to inform about successful communication establishment.

12.	The MC service server may notify the initiating MC service user of all MC service users who acknowledged the ad hoc group communication request and joined the ad hoc group communication. This notification may be sent to the initiating MC service user by the MC service server more than once during the communication when MC service users join or leave the MC service ad hoc group communication.

13.	MC service client 1, MC service client 2, MC service client 3 and MC service client 4 establish media plane and floor control resources.

NOTE:	How to handle the end-to-end encryption when the MC service client 1 does not have the required security related information to communicate with the participants of the ad hoc group communication is in the scope of SA3.

##### 7.1.3.2.2	Ad hoc group communication setup – Participants list determined by the MC service server

Figure 7.1.3.2.2-1 below illustrates the ad hoc group communication setup procedure initiated by an authorized user wherein the list of participants is determined by the MC service server based on the citeria received from the MC service client and determined MC service users are from multiple MC systems.

Pre-conditions:

1.	The MC service user at MC service client 1 is authorized to initate ad hoc group communication.

2.	End-to-End encryption is supported for this ad hoc group communication.

Figure 7.1.3.2.2-1: Ad hoc group communication setup involving multiple MC systems

1-4.	Same as described in sub-clause 7.1.3.1.3.

5.	MC service server 1 if it needs to involve the partner system based on the agreement and based on the criteria for determining the participants list, sends the ad hoc group communication get userlist request to the MC service server 2. This request carries the criteria specified in the step 1 .

6.	MC service server 2 evaluates the criteria and determines the particpants satisfying the criteria and sends the response containing the list of MC service users satisfying the criteria

7.	The MC service server 1 sends the ad hoc group communication request towards the MC service server 2. This request carries the list of MC service users who needs to be invited for this ad hoc group communication and are registered with MC service server 2. It also carries the pre-configured group ID whose configuration is to be applied for this ad hoc group communication if end-to-end encryption is requested.

8a-8b.	The MC service server 2 sends the ad hoc group communication request towards the MC service client 3 and MC service client 4.

9.	The MC service server 1 sends the ad hoc group communication requests towards the MC service client 2. While sending the ad hoc group communication request, the MC service server shall remove the information elements that are not required to be conveyed to the target MC service clients. This request carries the pre-configured group ID whose configuration is to be applied for this ad hoc group communication if end-to-end encryption is requested. The MC service server considers the ad hoc group communication participants as implicitly affiliated to the ad hoc group.

10a-10c.	The receiving MC service clients are notified about the incoming ad hoc group communication.

11.	The MC service client 2 accept the ad hoc group communication request and send ad hoc group communication response to the MC service server 1.

12a-12b.	The MC service client 3 and MC service client 4 accepts the ad hoc group communication request and sends ad hoc group communication responses to the MC service server 2.

13a-13b.	The MC service server 2 forwards the responses to the MC service server 1.

14.	The MC service server 1 sends the ad hoc group communication response to MC service client 1 through the signalling path to inform about successful communication establishment.

15.	The MC service server 1 may notify the initiating MC service user of all MC service users who acknowledged the ad hoc group communication request and joined the ad hoc group communication. This notification may be sent to the initiating MC service user by the MC service server 1 more than once during the communication when MC service users join or leave the MC service ad hoc group communication.

16.	MC service client 1, MC service client 2, MC service client 3 and MC service client 4 establish media plane and floor control resources.

* * * Next Change * * * *

##### 7.1.3.1.1	Ad hoc group communication setup

Figure 7.1.3.1.1-1 below illustrates the ad hoc group communication setup procedure initiated by an authorized user.

Pre-conditions:

1.	The authorized user at MC service client 1 wants to invite MC service users at MC service client 2, MC service client 3 and MC service client 4 for the ad hoc group communication.

2.	Number of participants being invited for the communication is within the limit for non pre-configured approach.

3.	End-to-End encryption is supported for this ad hoc group communication.

4.	MC service client 1 has all the security related information required to communicate with the participants of the ad hoc group communication.

Figure 7.1.3.1.1-1: Ad hoc group communication setup

1.	User at MC service client 1 would like to initiate an ad hoc group communication. MC service client 1 initiates the ad hoc group communication by sending the ad hoc group communication request containing the list of participants to the MC service server. Encryption supported information element shall be set to true since end-to -end encryption is supported. The ad hoc group communication request may also contain block sharing participants list indicator information element set to true if the originating user want to suppress sharing of the participants list to other users. The ad hoc group communication request may include the MC service group ID that is created by the MC service client 1 or not include the MC service group ID that allow the MC service server to assign one for use.  An SDP offer containing the MC service client media parameters is included. If there is a floor request to transmit, then the ad hoc group communication request contains an indication of an implicit floor request. If the MC service user of MC service client 1 has selected a functional alias, then the ad hoc group communication request contains that functional alias. If the ad hoc group communication request contains an implicit floor request it may also include location information.

If the MC service user at MC service client 1 initiates an MC service emergency ad hoc group communication or the MC service emergency state is already set for the MC service client 1 (due to a previously triggered MC service emergency alert):

i.	the MC service ad hoc group communication request shall contain an emergency indicator;

ii.	if the MC service emergency state is not set already, MC service client 1 sets its MC service emergency state. The MC service emergency state of MC service client 1 is retained until explicitly cancelled by the user of MC service client 1.

2.	The MC service server accepts the ad hoc group communication request if the ad hoc group communication is supported and authorized. Otherwise reject the ad hoc group communication request and do not continue with the rest of the steps. If authorised, it validates whether the number of invited participants is within the configured limit before proceeding with the communication setup.

If functional alias is present, the MC service server checks whether the provided functional alias is allowed to be used and has been activated for the user.

If location information was included in the ad hoc group communication request, the MC service server checks the privacy policy of the MC service user to decide if the location information of MC service client 1 can be provided to other users on the communication (refer to Annex A.3 "Authorisation to provide location information to other MC service users on a communication when talking").

If an emergency indicator is present in the received MC service ad hoc group communication request, the MC service ad hoc group is considered to be in the in-progress emergency state until this ad hoc group communication is terminated; and

If an imminent peril indicator is present in the received MC service ad hoc group communication request, the MC service ad hoc group is considered to be in the in-progress imminent peril state until this ad hoc group communication is terminated.

The MC service server considers the ad hoc group communication participants as implicitly affiliated to the ad hoc group.

3.	The MC service server shall send the ad hoc group communication request return message to MC service client 1 containing the below:

i.	The MC service ad hoc group ID (e.g. either provided by MC service client 1 which is acceptable to the MC service server or generated by the MC service server in the case where the MC service ad hoc group ID created by the MC service client 1 is not acceptable or the case where the MC service ad hoc group ID was not provided by MC service client 1); and

ii.	Result of whether the ad hoc group communication is authorized or not

If the ad hoc group communication request is not authorized, MC service client 1 shall not proceed with the rest of the steps.

4a-4c.	MC service client 1 sends ad hoc group communication security material request targeting each of the participants containing the security material to be shared with the participants for use in this ad hoc group communication.

NOTE 1:	If end-to-end encryption is not supported, then the ad hoc group communication security material requests are not sent.

5a-5c.	The MC service server sends the ad hoc group communication requests towards the MC service clients of the invited users based on step 1. While sending the ad hoc group communication requests, the MC service server shall remove the information elements that are not required to be conveyed to the target MC service clients (e.g. Block sharing participants list indicator, MC service ID list of the users who are required to acknowledge)

6a-6c.	The receiving MC service clients are notified about the incoming ad hoc group communication.

7a-7c.	The receiving MC service clients accept the ad hoc group communication requests and send ad hoc group communication responses to the MC service server. The response may also contain a functional alias of the responding MC service user, which is verified (valid and activated for the user) by the MC service server.

8.	The MC service server sends the ad hoc group communication response to MC service client 1 through the signalling path to inform about successful communication establishment.

NOTE 2:	Steps 5 to step 7 can start to occur before all of step 4 is completed since the MC service server do not require to wait for the previous ad hoc group communication request to complete before sending the ad hoc group request to another participant.

9.	If the initiating MC service user requires the acknowledgement from the invited MC service users, and the required MC service users do not acknowledge the communication setup within a configured time (the "acknowledged communication setup timeout"), then the MC service server may proceed with or abandon the communication and then notify the initiating MC service user that the acknowledgements did not include all required members according to ad hoc group communication policy from the user profile configuration. The MC service server may notify the initiating MC service user of all MC service users who did not acknowledge the ad hoc group communication request within the configured time. This notification may be sent to the initiating MC service user by the MC service server more than once during the communication when MC service users join or leave the MC service ad hoc group communication.

10.	MC service client 1, MC service client 2, MC service client 3 and MC service client 4 establish media plane and floor control resources.

NOTE 3:	Step 10 can occur any time following step 8 if the conditions to proceed with the communication are met.

Editor's note:	Ad hoc group communication procedure involving the usage of pre-configured group is FFS.

Editor's note:	Ad hoc group communication procedure wherein the MC system determining the list of participants to be invited for the ad hoc group communication is FFS.

* * * Next Change * * * *

#### 7.1.2.9	Ad hoc group communication get userlist (MC service server – MC service server)

Table 7.1.2.9-1 describes the information flow ad hoc group communication get userlist from one MC service server to another MC service server.

Table 7.1.2.9-1: Ad hoc group communication get userlist

| Information element | Status | Description |
| --- | --- | --- |
| Criteria for determining the participants | M | Carries the details of criteria or meaningful label identifying the criteria or the combination of both which will be used by the MC service server for determining the participants e.g., it can be a location based criteria to invite particpants in a particular area |

#### 7.1.2.10	Ad hoc group communication get userlist response (MC service server – MC service server)

Table 7.1.2.10-1 describes the information flow ad hoc group communication get userlist response from one MC service server to another MC service server.

Table 7.1.2.10-1: Ad hoc group communication get userlist response

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list | M | List of MC service IDs meeting the criteria specified in the ad hoc group communication get userlist |

#### 7.1.2.11	Group Call Notify (MC service server – MC service client)

Table 7.1.2.11-1 describes the information flow group call notify from MC service server to MC service client.

Table 7.1.2.11-1: Group Call Notify

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | The MC service ID of the calling party |
| Functional alias | O | The functional alias of the calling party |
| MC service ad hoc group ID | M | The MC service group ID associated with the ad hoc group communication |
| MC service ID list (see NOTE) | O | The list of the invited MC service users who did not acknowledge the ad hoc group call request |
| NOTE: Only applicable to acknowledged group calls. |  |  |