3GPP TSG-SA WG6 Meeting #51-e meeting	  S6-222681

10th – 19th October 2022, Online

Source:	ZTE Corporation

Title:	UE registration

Spec:	3GPP TS 23.545 v0.6.0

Agenda item:	8.6

Document for:	Approval

Contact:			Shao Weixiang, shao.weixiang@zte.com.cn

1. Introduction

This contribution provides FF UE registration.

2. Reason for Change

Add solution for FF UE registration.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.545 v0.6.0.

* * * First Change * * * *

## 7.x	FF UE registration

### 7.x.1	General

The FAE capabilities provide support for registering FF UEs at the FAE server. The FAE server uses the registration information to allow discovery of FF services  to the appropriate FF UEs.

### 7.x.2	FF UE registration procedure

Pre-condition:

-	The FAE client has discovered the FAE server and is aware of the address of the FAE server (e.g. FDQN).

Figure 7.x.2-1: Procedure for registering the FAE client at the FAE server

1.	The FAE client sends a registration request to the FAE server.

2.	The FAE server performs authentication and authorization check (e.g. based on pre-provisioned security information or by interacting with FAE application specific server).

3.   The FAE server sends an acknowledgement to the FAE client.

### 7.x.3	FF UE deregistration procedure

Pre-condition:

-	The FAE client has already registered with the FAE server as described in subclause 7.x.2.

Figure 7.x.3-1: Procedure for deregistering the FAE client at the FAE server

1.	The FAE client sends a deregistration request to the FAE server.

2.	The FAE server sends a deregistration response to the FAE client.

### 7.x.4	FF UE registration update procedure

Pre-condition:

-	The FAE client has already registered with the FAE server as described in subclause 7.x.2.

Figure 7.x.4-1: Procedure for registration update by the FAE client at the FAE server

1.	The FAE client sends a registration update request to the FAE server.

2.	The FAE server sends a registration update response to the FAE client.

### 7.x.5	Information flows

#### 7.x.5.1	FF UE Registration request

Table 7.x.5.1-1 describes the information flow for a FAE client to register with the FAE server.

Table 7.x.5.1-1: FF UE Registration request

| Information element | Status | Description |
| --- | --- | --- |
| FF UE ID | M | Identifier of the FF UE |
| FF UE Profile | O | Set of parameters describing the FF UE, which include: - FF UE category, e.g. robot, mobile panel, AGV, etc. - FF service of UE, - … |
| Proposed registration lifetime (NOTE) | O | Proposed to registration lifetime. |
| NOTE: If Proposed registration lifetime IE is not included, then the registration lifetime is valid until explicit deregistration is peformed. |  |  |

#### 7.x.5.2	FF UE Registration response

Table 7.x.5.2-1 describes the information flow for FAE server to respond for registration request from the FAE client.

Table 7.x.5.2-1: FF UE Registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the FAE server in response to registration request indicating success or failure |
| Registration lifetime | O | The registration lifetime provided by FAE server if registration is successful |

#### 7.x.5.3	FF UE Deregistration request

Table 7.x.5.3-1 describes the information flow for a FAE client to deregister from the FAE server.

Table 7.x.5.3-1: FF UE Deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| FF UE ID | M | Identifier of the FF UE |

#### 7.x.5.4	FF UE Deregistration response

Table 7.x.5.4-1 describes the information flow for FAE server to respond for deregistration request from the FAE client.

Table 7.x.5.4-1: FF UE Deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the FAE server in response to the deregistration request |

#### 7.x.5.5	FF UE registration update request

Table 7.x.5.5-1 describes the information flow for a FAE client to update registration with the FAE server.

Table 7.x.5.5-1: FF UE registration update request

| Information element | Status | Description |
| --- | --- | --- |
| FF UE ID | M | Identifier of the FF UE |
| FF UE Profile | O | Set of parameters describing the FF UE, which include: - FF UE category, e.g. robot, mobile panel, AGV, etc. - FF service of UE, - … |
| Proposed registration lifetime | O | Proposed to registration lifetime. |

#### 7.x.5.6	FF UE registration update response

Table 7.x.5.6-1 describes the information flow for FAE server to respond for registration update request from the FAE client.

Table 7.x.5.6-1: FF UE registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the FAE server in response to registration update request indicating success or failure |
| Registration lifetime | O | The registration lifetime provided by FAE server if registration update is successful |

* * * End Change * * * *