3GPP TSG-SA WG6 Meeting #51-e meeting	  S6-222682

10th – 19th October 2022, Online

Source:	ZTE Corporation

Title:	Application specific server registration

Spec:	3GPP TS 23.545 v0.6.0

Agenda item:	8.6

Document for:	Approval

Contact:			Shao Weixiang, shao.weixiang@zte.com.cn

1. Introduction

This contribution provides FF application specific server registration.

2. Reason for Change

Add solution for FF application specific server registration.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.545 v0.6.0.

* * * First Change * * * *

## 7.x	FF application specific server registration

### 7.x.1	General

The FAE capabilities provide support for registering FF application specific server at the FAE server.  The FASS registration with FAE server is required to register the FF services that FASS provides, so that they can be utilized by FF UEs as per FF UE registration.

7.x.2	FF application specific server registration procedure

Pre-condition:

-	The FF application specific server has connected to the serving network successfully.

-	The FAE Server address has been provisioned on the FF application specific server.

Figure 7.x.2-1: Procedure for registering the FF application specific server at the FAE server

1.	The FF application specific server sends a registration request to the FAE server. The request includes security credentials required for the FASS to register to the FAE server.

2.	Upon receiving the request, the FAE server validates the FASS registration request and verifies the security credentials.

NOTE:	The authentication procedures in step 2 are built on top of the transport layer mechanism specified in Annex Y.4 of 3GPP TS 33.501 [x].

3.   The FAE server sends an acknowledgement to the FF application specific server. If the registration is successful, the FAE server stores the FASS Profile information

### 7.x.3	FF application specific server deregistration procedure

Pre-condition:

-	The FF application specific server has already registered with the FAE server as described in subclause 7.x.3.

Figure 7.x.3-1: Procedure for deregistering the FF application specific server at the FAE server

1.	The FF application specific server sends a deregistration request to the FAE server.

2.   The FAE server validates the FASS de-registration request and verifies the security credentials. The FAE server deletes any applicable FASS Profile information that it has stored.

3.	The FAE server sends a deregistration response to the FF application specific server.

### 7.x.4	Information flows

#### 7.x.4.1	FASS Registration request

Table 7.x.4.1-1 describes the information flow for a FF application specific server to register to establish association with the FAE Server.

Table 7.x.4.1-1: FASS Registration request

| Information element | Status | Description |
| --- | --- | --- |
| FASS ID | M | Identifier of the FF application specific server |
| FF service ID | O | The identifier of the FF service specified by the application provider. |
| Authorization Information | O | The authorization information used to determine whether the FASS is allowed to register. |
| FASS Profile | O | The elements in FASS Profile include the information of the FF application specific server, e.g. OT application name, OT application provider, OT application scenario description, OT application category, etc. |

#### 7.x.4.2	FASS Registration response

Table 7.x.4.2-1 describes the information flow for FAE server to respond for registration request from the FF application specific server.

Table 7.x.4.2-1: FASS Registration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the FAE server in response to registration request indicating success or failure |

#### 7.x.4.3	FASS Deregistration request

Table 7.x.4.3-1 describes the information flow for a FF application specific server to deregister from the FAE server.

Table 7.x.4.3-1: FASS Deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| FASS ID | M | Identifier of the FF application specific server |

#### 7.x.4.4	FASS Deregistration response

Table 7.x.4.4-1 describes the information flow for FAE server to respond for deregistration request from the FF application specific server.

Table 7.x.4.4-1: FASS Deregistration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result from the FAE server in response to the deregistration request |

* * * Second Change * * * *

# 2	References

[x]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

* * * End Change * * * *