3GPP TSG-SA WG6 Meeting #51-e	S6-222744

e-meeting, 10th – 19th October 2022	(revision of S6-22xxxx)

Source:	China Mobile

Title:	Update of Overview and Application architecture

Spec:	3GPP TS 23.435

Agenda item:	8.10

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution provides a update of overview and Application architecture.

2. Reason for Change

The update of overview and Application architecture is needed.

4. Proposal

It is proposed to agree the following changes to TS 23.435.

* * * First Change * * * *

## 4.x	Application layer network slice management

Application layer network slice management is provided to the VAL to better meet consumer's requirement without having to interact with 5GS frequently, based on network slice status collected from 5GS and QoE collected from application layer.

* * * Next Change * * * *

# 7	Application architecture for NSCALE

The application architecture are specified in clause 16 TS 23.434 [2]

* * * Next Change * * * *

## 9.4	Application layer network slice management

### 9.4.1	General

When NSCE receives a request for application layer network slice management(AL_NS_LCM) from VAL server, the NSCE server performs the service operations including subscribing the event which may trigger the network slice lifecycle management, making the network slice lifecycle management recommendation/decision, triggering the network slice management operations, notifying the consumer about the network slice information.

### 9.4.2	Procedure

Figure 9.4.2-1 illustrates a procedure of application layer network slice lifecycle management based on network slice related data and QoE collected from application layer.

Pre-conditions:

1.	The NSCE server has authenticated and authorized to the capabilities to collect current network slice from 5GS.

2.	The NSCE server has authenticated and authorized to the capabilities trigger the network slice LCM operations.

- Figure 9.4.2-1: Application layer network slice lifecycle management

1.	The VAL server sends the application layer network slice lifecycle management(AppLayer-NS-LCM) subscribe request to NSCE server,  with network slice requirements (e.g. delay, throughput, load, the maximum number of users supported, etc.). The request can indicate whether the notification is needed before performing the AppLayer-NS-LCM. The request can also indicate the trigger conditions, such as by providing the monitored parameters and the corresponding thresholds as described in clause 9.4.3.2.

2.	After receiving the request, the NSCE server checks that the user is authenticated and authorized to perform the corresponding AppLayer-NS-LCM operations, and filters the unauthorized requests, if any.

3.	Once authenticated and authorized, the NSCE server sends the AppLayer-NS-LCM response to the VAL server.

4.	According to network slice requirements and/or the trigger conditions, NSCE server triggers the provision of network slice status and QoE metrics.

4a.	The network slice status could be collected through subscribing or requesting to 5GS. For example, to monitor the slice load, it could subscribe to/request the relevant service(s), such as AnalyticsExposure defined in TS 23.502 [4] clause 5.2.6.16, provisioning data report exposure for NSI in clause 5 of TS 28.532 [8]. If the trigger conditions are not indicated in the subscription, the NSCE server can help to configure an appropriate trigger condition, such as report period or thresholds.

4b.	Also, the NSCE could get the information of QoE metrics from the application layer domain by QoE metric subscribe as described in clause 9.4.3.5.

5.	Once the trigger condition or the combination of trigger conditions is met, based on requirements and network slice capabilities with updated network slice status and QoE metrics collected in step 4, such as changing in QoE, changing in network slice status etc., the NSCE determines whether and what network slice LCM operations should be taken and makes the decision(s)/recommendation(s), such as modifyNsi/AllocateNsi/DeallocateNsi request as specified in TS 28.531 [5].

6.	Optionally, if it is indicated in the request to notify the VAL server/consumer before performing the AppLayer-NS-LCM, the NSCE sends the network slice LCM recommendation(s) with network slice information to VAL server, to see whether takes the recommendation(s) or not.

7.	Optionally, the VAL server sends the response to NSCE server.

8.	Based on decision made by VAL server or NSCE server, the NSCE server performs the corresponding operation(s).

9. According to the corresponding operation(s) result, the NSCE server sends the response to the VAL server.

### 9.4.3	Information flows

#### 9.4.3.1	General

The following information flows are specified for AppLayer_NS_LCM:

-	AppLayer_NS_LCM subscribe, response and notification;

-	QoE metrics subscribe, response and notification;

-	Network slice LCM recommendation request and response;

#### 9.4.3.2	Application layer network slice lifecycle management subscribe request

Table 9.4.3.2-1 describes information elements for the application layer network slice lifecycle management subscribe request from the VAL server to the NSCE server.

Table 9.4.3.2-1: AppLayer-NS-LCM Subscribe

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. VAL server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the NSCE service. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the requestor should be sent to. |
| Network slice requirements | M | The requirement of network requirements or updated requirements when the network slice was created. |
| List of S-NSSAI(s) | M | Identifier of the interested network slice |
| AppLayer-NS-LCM notification indication | M | Indicates whether to notify the VAL server/consumer about the AppLayer-NS-LCM |
| >AppLayer-NS-LCM notification address (NOTE) | M | The address (e.g. URL) of the consumer that can receive the AppLayer-NS-LCM notification |
| Trigger condition | O | Indicates the monitored parameters and the corresponding thresholds which could trigger the AppLayer-NS-LCM |
| Proposed expiration time | O | Proposed expiration time for the subscription |
| NOTE: When the AppLayer-NS-LCM notification indication is needed, this information element should be provided. |  |  |

#### 9.4.3.3	Application layer network slice lifecycle management response

The information elements specified in the Table 9.4.3.3-1 is used for the application layer network slice lifecycle management response sent from the NSCE server to the VAL server.

Table 9.4.3.3-1: AppLayer-NS-LCM Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the AppLayer-NS-LCM request was successful. |
| Failure response (NOTE) | O | Indicates that the AppLayer-NS-LCM request failed. |
| > Cause | O | Indicates the cause of AppLayer-NS-LCM request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.4.3.4	Application layer network slice lifecycle management notify

The information elements specified in the Table 9.4.3.4-1 is used for the application layer network slice lifecycle management notify sent from the NSCE server to the VAL server.

Table 9.4.3.4-1: AppLayer-NS-LCM Notify

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the AppLayer-NS-LCM request was successful. |
| >network slice information | M | Network slice information (i.e. NEST) with network slice identifier(i.e. S-NSSAI) |
| Failure response (NOTE) | O | Indicates that the AppLayer-NS-LCM request failed. |
| > Cause | O | Indicates the cause of AppLayer-NS-LCM request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.4.3.5	QoE metrics subscribe

Table 9.4.3.5-1 describes information elements for the QoE metrics subscribe from the NSCE server to the VAL server.

Table 9.4.3.5-1: QoE metrics Subscribe

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. NSCE server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Notification Target Address | O | The Notification Target Address (e.g. URL) where the notifications destined for the requestor should be sent to. |
| Event Filter | M | The associated filter on a network slice to be notified |
| >Application indicator | O | Indicator of the interested application (i.e. App ID) |
| >QoE type indicator | O | QoE metric type including latency, throughput, jitter, etc. |
| Event Reporting information | M | Information indicates whether the notification is threshold based or the notification is periodical and optionally the Immediate reporting flag |
| >Threshold | O | Threshold of QoE metrics |
| >Reporting period | O | Indicating the metrics reporting period |
| >Immediate reporting flag | O | Indicating the request needs immediate reporting or not |
| Proposed expiration time | O | Proposed expiration time for the subscribe |

#### 9.4.3.6	QoE metrics response

Table 9.4.3.6-1 describes the information elements for the QoE metrics response from the VAL server to the NSCE server.

Table 9.4.3.6-1: QoE metrics Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the QoE metrics request was successful. |
| > Subscription ID | M | Subscription identifier corresponding to the subscription. |
| >QoE metrics report | O | List of result values for the observed or computed QoE metrics value if the immediate reporting is needed. |
| Failure response (NOTE) | O | Indicates that the QoE metrics request failed. |
| > Cause | O | Indicates the cause of QoE metrics request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 9.4.3.7	QoE metrics notification

Table 9.4.3.7-1 describes the information elements for the QoE metrics notification from the VAL server to the NSCE server.

Table 9.4.3.7-1: QoE metrics Notify

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Indicates that the QoE metrics request was successful. |
| QoE metrics report | M | List of result values for the observed or computed QoE metrics value. |

#### 9.4.3.8	Network slice LCM recommendation request

Table 9.4.3.8-1 describes information elements for the Network slice LCM recommendation request from the NSCE server to the VAL server.

Table 9.4.3.8-1: Network slice LCM recommendation Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. NSCE server ID). |
| Security credentials | M | Security credentials resulting from a successful authorization. |
| Network slice LCM recommendation | M | Recommended network slice lifecycle management operation |
| >Network slice identifier(i.e. S-NSSAI) | M | Unique identifier of network slice (i.e. S-NSSAI) |
| >Recommend network slice LCM action | M | Recommend network slice LCM action(i.e. modifying the configuration, allocating a network slice) |
| >Network slice information | O | Network slice information if the action is taken(i.e. NEST) |

#### 9.4.3.9	Network slice LCM recommendation response

Table 9.4.3.9-1 describes the information elements for the Network slice LCM recommendation response from the VAL server to the NSCE server.

Table 9.4.3.9-1: Network slice LCM recommendation Response

| Information element | Status | Description |
| --- | --- | --- |
| Recommendation accepted flag | M | Indicates that the Network slice LCM recommendation request was accepted or not. |

### 9.4.4	APIs

#### 9.4.4.1	General

Table 9.4.4.1-1 illustrates the API for application layer network slice management.

Table 9.4.4.1-1: API for application layer network slice management

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Ensce_AppLayerNSLCM | Subscribe | Subscribe/Response | VAL server |
|  | Notify | Notify | VAL server |
| Eval_QoEMetrics | Subscribe | Subscribe/Response | NSCE |
|  | Notify | Notify | NSCE |
| Eval_NSLCMRecommendation | Subscribe | Subscribe/Response | NSCE |

#### 9.4.4.2	Ensce_AppLayerNSLCM_Subscribe operation

API operation name: Ensce_AppLayerNSLCM_Request

Description: The consumer subscribes for AppLayer_NS_LCM.

Inputs: See clause 9.4.3.2.

Outputs: See clause 9.4.3.3.

See clause 9.4.2 for details of usage of this operation.

#### 9.4.4.3	Ensce_AppLayerNSLCM_Notify operation

API operation name: Ensce_AppLayerNSLCM_Notify

Description: The consumer is notified with result of AppLayer_NS_LCM.

Inputs: See clause 9.4.3.4.

Outputs: None

See clause 9.4.2 for details of usage of this operation.

#### 9.4.4.4	Eval_QoEMetrics_Subscribe operation

API operation name: Ensce_AppLayerNSLC_Request

Description: The consumer subscribes for the QoE metrics.

Inputs: See clause 9.4.3.5.

Outputs: See clause 9.4.3.6.

See clause 9.4.2 for details of usage of this operation.

#### 9.4.4.5	Eval_QoEMetrics_Notify operation

API operation name: Eval_QoEMetrics_Notify

Description: The consumer is notified with the QoE metrics.

Inputs: See clause 9.4.3.7.

Outputs: None

See clause 9.4.2 for details of usage of this operation.

#### 9.4.4.6	Eval_NSLCMRecommendation _Request operation

API operation name: Eval_NSLCMRecommendation _Request

Description: The consumer request for whether to take the recommendation of the AppLayer_NS_LCM.

Inputs: See clause 9.4.3.8.

Outputs: See clause 9.4.3.9.

See clause 9.4.2 for details of usage of this operation.