3GPP TSG-SA WG6 Meeting #51-e	S6-222794

e-meeting, 10th – 19th October 2022	(revision of S6-22xxxx)

Source:	Huawei, Hisilicon

Title:	SEALDD enabled E2E redundant transmission

Spec:	3GPP TS 23.433 v0.0.0

Agenda item:	8.14

Document for:	Approval

Contact:	Yaxin (wangyaxin11@huawei.com), Niranth (namogh@huawei.com)

1. Introduction

This contributions provides a proposal for SEALDD enabled E2E redundant transmission.

2. Reason for Change

SA6 studied and concluded KI#2 about SEALDD enabled E2E redundant transmission, and the corresponding solutions need to be specified. The procedure of solution#2 in TR 23.700-34 v1.0.0 is transposed to TS 23.433. Detailed discriptions about how SEALDD server manages the connection with the VAL server are clarified to make the whole procedure clear for deployment.

3. Proposal

It is proposed to agree the following changes to 3GPP TS 23.433 v0.0.0.

* * * First Change * * * *

## 9.X	SEALDD enabled E2E redundant transmission

### 9.x.1	General

The following clauses specify procedures, information flow and APIs for SEALDD enabled E2E redundant transmission.

SEALDD client and SEALDD server transfer SEALDD traffic via two redundant PDU sessions as specified in clause 5.33.2.1 of 3GPP TS 23.501 [6].

Figure 9.x.1-2 shows the data traffic flow of E2E redundant transmission. For uplink data delivery, VAL client sends application traffic to SEALDD client, the SEALDD client duplicates the application packets and maps them into two SEALDD traffic. Then the two SEALDD traffic are transferred to SEALDD server via the two redundant PDU sessions shown in figure 9.x.1-1. The SEALDD server eliminates the redundant packets and recovers the application traffic. The recovered application traffic is transferred to VAL server by the SEALDD server. For downlink data delivery, VAL server sends application traffic to SEALDD server, the SEALDD server duplicates the application packets and maps them into two SEALDD traffic. The two SEALDD traffic are transferred to UE via the two redundant PDU sessions. The SEALDD client eliminates the redundant SEALDD packets and recovers the application traffic, then sends the application traffic to the VAL client.

Figure 9.x.1-2: E2E redundant transmission traffic flow

Figure 9.x.1-3 shows the data traffic flow of E2E redundant transmission for multiple VAL servers. In this scenario, SEALDD server and SEALDD client use different SEALDD flow IDs and SEALDD traffic descriptors to identify SEALDD traffic for different VAL servers.

Figure 9.x.1-3: E2E redundant transmission traffic flow for multiple VAL servers

For outbound data delivery, VAL application traffic is sent to SEALDD enabler layer, the SEALDD enabler duplicates the application packets and maps them into two SEALDD traffic (with the same Flow ID). Then according to the SEALDD traffic descriptors of the SEALDD flow, the SEALDD traffic is sent out with different destination addresses or ports and different source addresses or ports. For inbound data delivery, two SEALDD traffic (with different source addresses or ports and different destination addresses or ports) are received. According to the SEALDD traffic descriptors, SEALDD enabler decides they belong to the same Flow. Then after packet elimination and reordering, the two SEALDD traffic is aggregated to one VAL application traffic.

9.x.2	Procedure

#### 9.x.2.1	E2E redundant transmission path establishment procedure

Figure 9.x.2.1-1 illustrate the procedure for redundant transmission establishment. This procedure can be triggered by the VAL server for data transfer per application layer transaction.

Pre-conditions:

- The VAL server has discovered and selected the SEALDD server by CAPIF functions.

- The VAL server has established application signaling connection with the VAL client and can get the VAL client’s UE ID or UE address via the application signaling connection.

Figure 9.x.2.1-1: E2E redundant transmission path establishment

1.	The VAL server decides to use SEALDD service to help ensuring data transmission quality for a specific application transaction and send a Sdd_URLLCTransmission request to the SEALDD server discovered by CAPIF. The request includes UE ID/address, identifiers of the application traffic (e.g. VAL server ID, the URI/address of the VAL server), SEALDD-S Data transmission connection information of the VAL server side, QoS requirements.

NOTE 1:	If both signalling and data traffic of the VAL server is transmitted via SEALDD service, the URI/address of the VAL server is not needed.

2.	Upon receiving the request, the SEALDD server decides to establish redundant transmission path. The SEALDD server allocates two different addresses or ports for the two redundant transmission paths and sends an AF request to 5GS to create or update URSP rules as described in clause 4.15.6.10 of 3GPP TS 23.502 [7] for the UE(s) going to use the redundant transmission service. The AF request includes Identifiers of the UE(s) and application traffic descriptor containing the addresses or ports allocated by SEALDD server.

3.	If the processing of the request was successful, SEALDD server allocates address/port of the SEALDD server to receive the packets from the VAL server for application data transfer as SEALDD-S data transmission connection information of the SEALDD server side. The SEALDD server responds with a SEALDD service response (including SEALDD-S data transmission connection information of the SEALDD server side for the current application transaction) and indicates the VAL server that redundant transmission service should be activated. The VAL server and SEALDD server can use SEALDD-S data transmission connection information to establish the data transmission connection between VAL server and SEALDD server for application data transfer.

4.	Application transaction information is provisioned to the VAL client by the VAL server via application signalling. If the redundant transmission requirement is not preconfigured or notified to the VAL client, the VAL server may notify the VAL client(s) which are going to use the redundant transmission service through application layer message.

NOTE 2:	The application signalling may be transmitted via direct application layer connection or via the SEALDD layer.

NOTE 3:	The VAL client can be preconfigured that the VAL service should always be transmitted via redundant transmission. Or this application layer notification may be notified to the UE in other period before the VAL application traffic is really transmitted.

5.	The VAL client sends a SEALDD service request to use E2E redundant transmission for the application traffic of the current application transaction.

6.	The SEALDD client discover and select the proper SEALDD server for the VAL application.

Editor’s Note: Whether interaction between VAL client and SEALDD client, SEALDD server determination steps should be described in this procedure are FFS.

Editor’s Note:	How to discover and select the proper SEALDD server is discussed in KI #6 and KI #7 of TR 23.700-34 and this step will be updated according to the specification of KI #6 and KI #7.

7.	The SEALDD client allocates a SEALDD flow ID mapping to the identifiers of the application traffic and the application transaction information. The SEALDD client sends Sdd_URLLCTransmissionConnection_Establish request to SEALDD server. The request includes the SEALDD client ID, SEALDD flow ID, the identifiers of the application traffic for SEALDD server to identify the specific application traffic and the application transaction information.

Editor’s Note: What format of application transaction information can be used (e.g. VAL service ID, Data transmission Session ID, Application transaction ID) is FFS.

8.	Upon receiving the request, the SEALDD server sends SEALDD traffic descriptor for redundant transmission of the SEALDD server side (i.e. the addresses or ports for the redundant transmission paths allocated in step 2 and the transport protocol used for the SEALDD traffic) to SEALDD client.

9.	The UE uses the SEALDD traffic descriptor of the SEALDD server side to trigger the two redundant PDU Sessions establishment procedure via 5GS as specified in clause 5.33.2.1 of 3GPP TS 23.501 [6].

10.	[Optional] The SEALDD client sends Sdd_URLLCTransmissionConnection_Update request to SEALDD server. The request includes the SEALDD client ID, the SEALDD flow ID, the SEALDD traffic descriptors for redundant transmission of the SEALDD client side (i.e. UE addresses and ports of the two redundant PDU Sessions). The two redundant SEALDD traffic use the same SEALDD flow ID for identification.

11.	[Optional] The SEALDD server sends response to SEALDD client. After this step, the SEALDD client and SEALDD server both get the whole SEALDD traffic descriptors (including the UE's addresses/ports and SEALDD server's addresses/ports for the SEALDD traffic transmission). The SEALDD client and SEALDD server store the mapping between the application traffic and SEALDD traffic.

12.	[Optional] If the connection between VAL server and SEALDD server is not established in step 3, the SEALDD server establishes connection with VAL server for the VAL client to transmit application traffic mapping to the redundant SEALDD traffic according to the SEALDD-S information negotiated in step 1-3

NOTE 4:	Step 10 and Step 11 are optional. If the redundant PDU sessions are already established before step 7, the IP addresses of the UE may be notified to the SEALDD server in step 7. In other cases, after the establishment of the two redundant PDU sessions, the SEALDD client may communicate with SEALDD server through the redundant PDU sessions to let the SEALDD server know the UE's address(es) of the redundant PDU session to fulfil the traffic mapping or the SEALDD client and SEALDD server may use other mapping mechanisms, it is up to the transport protocol used by SEALDD client and SEALDD server for the SEALDD traffic.

13.	The SEALDD client responds with a SEALDD service response.

After the negotiation and establishment of the connections, the SEALDD client gets the mapping information between the application transaction and SEALDD flow ID. The SEALDD server gets the mapping information between the SEALDD flow ID, the  the SEALDD-S connection for the application transaction. Upon receiving application traffic from VAL client, the SEALDD client duplicates the application packets and maps them into two SEALDD traffic with SEALDD traffic descriptors as negotiated with SEALDD server in step 8 and step 10. The two SEALDD traffic is sent through two redundant PDU sessions to the SEALDD server. The SEALDD server maps the two SEALDD traffic to the same application traffic according to the stored SEALDD traffic descriptors, SEALDD client ID and SEALDD flow ID. After packet elimination and reordering the SEALDD server sends the aggregated application traffic to VAL server via the connection established in step 3 or 12 according to the mapping information. The downlink application traffic sent from VAL server to VAL client is processed similarly.

### 9.x.3	Information flows

#### 9.x.3.1	SEALDD URLLC transmission request

Table 9.x.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the URLLC transmission service.

Editor’s Note: Whether the information flow and API between SEALDD server and VAL server should be merged to one generic API is FFS.

Table 9.x.3.3-1: SEALDD URLLC transmission request

| Information element | Status | Description |
| --- | --- | --- |
| Identifiers of the application traffic | M | Information to identify application traffic (e.g. VAL server ID, URI/address of the VAL server) |
| VAL client information | M | Information to identify the VAL client (e.g.UE ID/address) |
| SEALDD-S Data transmission connection information | M | Address/port of the VAL server to receive the packets from the SEALDD server for application logic processing |

#### 9.x.3.2	SEALDD URLLC transmission response

Table 9.x.3.2-1 describes the information flow from the SEALDD server to the VAL server for responsing the URLLC transmission request.

Table 9.x.3.4-1: SEALDD URLLC transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| SEALDD-S Data transmission connection information | M | Address/port of the SEALDD server to receive the packets from the VAL server for application data transfer |

#### 9.x.3.3	SEALDD URLLC transmission connection establishment request

Table 9.x.3.3-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the URLLC transmission connection establishment.

Table 9.x.3.3-1: SEALDD URLLC transmission connection establishment request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | M | Identity of the SEALDD client. |
| SEALDD flow ID | M (See NOTE) | Identity of the SEALDD flow. |
| Identifiers of the application traffic | M (See NOTE) | Information to identify the application traffic. |
| SEALDD traffic descriptors | O | A pair of SEALDD traffic descriptors (e.g. address, port, transport layer protocol) of the SEALDD client side used to estabilish redundant SEALDD connection. |
| NOTE: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and a specific application transaction. |  |  |

#### 9.x.3.4	SEALDD URLLC transmission connection establishment response

Table 9.x.3.4-1 describes the information flow from the SEALDD server to the SEALDD client for responsing the URLLC transmission connection establishment.

Table 9.x.3.4-1: SEALDD URLLC transmission connection establishment response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of estabilishing the SEALDD connection. |
| SEALDD traffic descriptors | O | A pair of SEALDD traffic descriptors (e.g. address, port, transport layer protocol) of the SEALDD server side used to estabilish redundant SEALDD connection. |

#### 9.x.3.5	SEALDD URLLC transmission connection update request

Table 9.x.3.5-1 describes the information flow from the SEALDD client to the SEALDD server for requesting the URLLC transmission connection update.

Table 9.x.3.5-1: SEALDD URLLC transmission connection update request

| Information element | Status | Description |
| --- | --- | --- |
| SEALDD client ID | M | Identity of the SEALDD client. |
| SEALDD flow ID | M (See NOTE) | Identity of the SEALDD flow. |
| Identifiers of the application traffic | O (See NOTE) | Information to identify the application traffic. |
| SEALDD traffic descriptors | O | A pair of SEALDD traffic descriptors (e.g. address, port, transport layer protocol) of the SEALDD client side used to estabilish redundant SEALDD connection. |
| NOTE: The SEALDD flow ID is used by the SEALDD client and SEALDD server to identify different application traffic, and it is mapped to the identifiers of the application traffic and a specific application transaction. |  |  |

#### 9.x.3.6	SEALDD URLLC transmission connection update response

Table 9.x.3.6-1 describes the information flow from the SEALDD server to the SEALDD client for responsing the redundant transmission connection update.

Table 9.x.3.6-1: SEALDD URLLC transmission connection update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of updating the SEALDD connection. |

### 9.X.4	APIs

#### 9.x.4.1	General

Table 9.x.4.1-1 illustrates the APIs exposed by SEALDD server for URLLC transmission.

Table 9.x.4.1-1: List of SEALDD server APIs for redundant transmission

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_URLLCTransmission | Request | Request/Response | VAL server |
| Sdd_URLLCTransmissionConnnection_Establish | Request | Request/Response | SEALDD client |
| Sdd_URLLCTransmissionConnnection_Update | Request | Request/Response | SEALDD client |

#### 9.x.4.2	Sdd_URLLCTransmission Request operation

API operation name: Sdd_URLLCTransmission Request

Description: The consumer requests for one time for URLLC transmission service.

Inputs: See clause 9.x.3.1.

Outputs: See clause 9.x.3.2

See clause 9.x.2.1 for details of usage of this operation.

#### 9.x.4.3	Sdd_URLLCTransmissionConnection_Establish Request operation

API operation name: Sdd_URLLCTransmissionConnection_Establish Request

Description: The consumer requests for one time for URLLC transmission connection establishment.

Inputs: See clause 9.x.3.3.

Outputs: See clause 9.x.3.4.

See clause 9.x.2.1 for details of usage of this operation.

#### 9.x.4.4	Sdd_URLLCTransmissionConnection_Update Request operation

API operation name: Sdd_URLLCTransmissionConnection_Update Request

Description: The consumer requests for one time for URLLC transmission connection update.

Inputs: See clause 9.x.3.5.

Outputs: See clause 9.x.3.6.

See clause 9.x.2.1 for details of usage of this operation.