3GPP TSG-SA WG6 Meeting #52-bis-e	S6-230212

e-meeting, 11th – 20th January 2023	(revision of S6-23xxxx)

Source:	China Mobile

Title:	Slice configuration recommendation

Spec:	3GPP TS 23.436

Agenda item:	8.16

Document for:	Approval

Contact:	Shaowen Zheng, zhengshaowen@chinamobile.com

1. Introduction

This contribution adds the procedure and information element for Slice configuration recommendation.

2. Reason for Change

The procedure and information element of Slice configuration recommendation are concluded to be moved to the normative work.

4. Proposal

It is proposed to agree the following changes to TS 23.436.

* * * First Change * * * *

## 6.x	Slice configuration recommendation

This feature introduces application layer analytics to provide network slice configuration recommendation based on collected network slice performance and analytics, historical network slice status, and network performance to help the consumer manage the network slice.

* * * Next Change * * * *

## 8.x	Slice configuration recommendation

### x.1	General

This clause provides a procedure for network slice configuration recommendation based on collected network slice performance and analytics, historical network slice status, and network performance. The consumer can be either the VAL server or other consumers such as SEAL NSCE server.

### 8.x.2	Procedure

Figure 8.x.2-1 illustrates the procedure for network slice configuration recommendation.

Pre-conditions:

1.	The ADAES is registered and capable of interacting with 5GS to collected network slice data.

Figure 8.x.2-1: ADAES support for network slice configuration recommendation

1.	The consumer of the ADAES sends a subscription request to ADAES and provides the target S-NSSAI, DNN, slice requirement, area of the interest, interest time period of the historical data (e.g. last year), the required confidence level, whether offline and/or online analytic are needed etc.

2.	The ADAES sends a subscription response to the consumer.

3.	The ADAES subscribes to the Data Sources with the respective Data Collection Event ID and the requirement for data collection related to the request slice(s). Such requests can be sent to SEAL NSCE server, OAM, NWDAF or the combination of them.

4.	Based on subscription, the ADAES may receive performance and analytics data from SEAL NSCE server as defined in the TS 23.435[6] clause 9.7.

5.	Based on subscription, the ADAES may receive Network slice / NSI related performance data from OAM as defined in TS 28.552 [x].

6.	Based on subscription, the ADAES may receives Network slice related Observed Service experience statistics, Load level information of a Network Slice defined from NWDAF as defined in TS 23.288[4]

7.	If the data is collected from multiple sources, the ADAES combines or correlates the data/analytics from steps 3-5, and stores the data into A-ADRF if needed.

8.	The ADAES server sends the network slice data retrieval request to collect the historical data from A-ADRF.

9.	The ADRF provides network slice historical data to the ADAES.

10.	The ADAES analyzes the network slice usage pattern based on the network slice historical data. When the stored historical data does not cover the required interest time period of the historical data, ADAES analyzes the slice usage pattern based on the existing stored historical data.Based on the slice requirement, slice performance and derived slice usage pattern, The ADAES provides network slice configuration recommendation.

11.	The ADAES sends the network slice configuration recommendation to the consumer. The recommendation may be related to parameters in the slice serviceProfile if the consumer is the SEAL NSCE server. Or the recommendation may be related to slice resource /functional configuration (e.g. slice capacity, coverage) if the consumer is the management system.

### 8.x.3	Information flows

#### 8.x.3.1	General

The following information flows are specified for network slice configuration recommendation based on 8.x.2.

#### 8.x.3.2	Network slice configuration recommendation subscription request

Table 8.x.3.2-1 describes information elements for the network slice configuration recommendation subscription request from the consumer (VAL server / NSCE server) to the ADAE server.

Table 8.x.3.2-1: Network slice configuration recommendation subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the analytics consumer |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “slice-specific application performance analytics” |
| Analytics type | M | Whether analytics is offline and/or online |
| Analytics filter information | M | Filter information for the analytics event |
| >Slice identifier | M | The identifier of the target slice or slice instance, i.e. S-NSSAI. |
| >Slice requirement | M | The requirement of network requirements or updated requirements when the network slice was created. The GST defined by GSMA (see clause 2.2 in [5]) and the performance requirements defined in clause 7 TS 22.261 [6] are all considered as input for the network slice related requirements. |
| >DNN | O | The target DNN for which the request applies |
| >Target VAL UE ID(s) | O | The VAL UE(s) for which the analytics subscription applies |
| >Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies (for procedure in 8.2.2) |
| >Area of Interest | O | The geographical or service area for which the subscription request applies |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Time validity | O | The time validity of the request |
| Interest time period of the historical data | O | Interest time period of the historical data (e.g. last year), |

#### 8.x.3.3	Network slice configuration recommendation subscription response

Table 8.x.3.3-1 describes information elements for the Network slice configuration recommendation subscription response from the ADAE server to the consumer (VAL/NSCE server).

Table 8.x.3.3-1: Network slice configuration recommendation subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the AppLayer-NS-LCM request was successful. |
| >Recommend network slice LCM action | M | Recommend network slice LCM action(i.e. modifying the configuration, allocating a network slice) |
| >Network slice information | O | Network slice information if the action is taken(i.e. NEST) |
| Failure response (NOTE) | O | Indicates that the AppLayer-NS-LCM request failed. |
| > Cause | O | Indicates the cause of AppLayer-NS-LCM request failure |
| NOTE: One of these IEs shall be present in the message. |  |  |

#### 8.x.3.4	Network slice configuration recommendation notification

Table 8.x.3.4-1 describes information elements for the network slice configuration recommendation notification from the ADAE server to the Consumer.

Table 8.x.3.4-1: network slice configuration recommendation notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “slice-specific application performance analytics” |
| >Recommend network slice LCM action | M | Recommend network slice LCM action(i.e. modifying the configuration, allocating a network slice) |
| >Network slice information | O | Network slice information if the action is taken(i.e. NEST) |

#### 8.x.3.5	Network slice data retrieval request

Table 8.x.3.5-1 describes information elements for the Network slice data retrieval request from the ADAE server to the ADRF.

Table 8.x.3.5-1: Network slice data retrieval request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Data Collection Event ID | M | The identifier of the data collection event |
| Network slice identifier | M | The identifier of the interested network slice |
| VAL service ID | O | The identifier of the VAL service which is associated with network slice |
| Data Collection requirements | M | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed |
| Target VAL UE ID(s) and address | O | The VAL UE(s) identifiers and IP address(es) for which the data collection subscription apply |
| Target VAL server ID | O | This identifier shows the target VAL server for which the data collection subscription applies (for procedure in 8.2.2) |
| Area of Interest | O | The geographical or service area for which the requirement request applies |
| Time validity | O | The time validity of the request |

#### 8.x.3.6	Network slice data retrieval response

Table 8.x.3.6-1 describes information elements for the Network slice data retrieval response from the ADRF to the ADAE server.

Table 8.x.3.6-1: Network slice data retrieval response

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The result of the data collection subscription request (positive or negative acknowledgement) |
| Network slice identifier | M | The identifier of the interested network slice |
| Target VAL UE ID and address | M (NOTE) | The VAL UE(s) identifiers and IP address(es) for which the data apply |
| Target VAL server ID | M (NOTE) | This identifier of the target VAL server for which the data applies (for procedure in 8.2.2) |
| Analytics ID | O | The identifier of the analytics event. This ID can be for example “VAL server performance analytics” for procedure in 8.2.2, or “VAL session performance analytics” for procedure in 8.2.3. |
| Data Type | O | The type of reported data samples which can be UE data, network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non real time). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter (e.g. RTT deviation) based on subscription |
| NOTE: One of these shall be present based on the data collection event |  |  |

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]		3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[3]	3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture"

[4]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

[5]	3GPP TS 28.104: "Management and orchestration; Management Data Analytics".

[6]		3GPP TS 23.435: "Procedures for Network Slice Capability Exposure for Application Layer Enablement Service".

[x]		3GPP TS 28.552: "Management and orchestration;5G performance measurements".