3GPP TSG-SA WG6 Meeting #52	S6-223416

Toulouse, France 14th – 18th November 2022	(revision of S6-223197)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0136 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | VAL service area identifier usage |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Interpretation of Location means different to different entities depending upon the context. For example, for a consumer, location is invariably civic address. For Network, it is one of Tracking Area/Routing Area (TA/RA), eNODEB ID, Cell ID. For a Vertical Application Layer (VAL), it can refer to the jurisdictional area of service. Every vertical application server will have an area defined within which it provides the required service to the VAL users. VAL service area can be defined through geographical co-ordinates and can be assigned an ID (VAL service Area ID) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Wherever location co-ordinates are being shared by the VAL server with SEAL server VAL service area ID is introduced. It either augments the location information or in some cases it can be present instead of geographical co-ordinates. The changes are done in the following areas : - Location Report and location information notification from SEAL LM to VAL server can carry the VAL service Area ID in addition to location information. SEAL server can add the VAL service area ID corresponding to the location information - GET UE(s) information request can carry the VAL service area ID instead of the geographical co-ordinates - Monitor location subscription request can carry the VAL service Area IDs as areas of interest. - Location area monitoring subscription request can include the VAL service Area ID as location information criteria - Location based group creation - Location based group update |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Usage of VAL service Area ID brings in more flexibility when using or consuming the functionality from SEAL involving location information. VAL service area ID can be shared across the SEAL servers and any change/update in the VAL service area can be easily managed. When there is change / update in VAL service Area, then VAL server can just update the VAL service area at SEAL domain only once and need not update its subscription to various APIs where location information is utilized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.2.2, 9.3.2.4, 9.3.2.7, 9.3.2.9, 9.3.2.10, 9.3.2.11, 9.3.2.13, 9.3.2.14, 9.3.2.17, 9.3.2.X(new), 9.3.2.Y(new), 9.4.X(new), 9.3.10, 9.3.12, 10.3.2.34, 10.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.3.2.4	Location reporting trigger

Table 9.3.2.4-1 describes the information flow from the location management client or VAL server to the location management server for triggering a location reporting procedure.

Table 9.3.2.4-1: Location reporting trigger

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M (see NOTE 1) | Identity of the requesting authorized VAL user or VAL UE or VAL server |
| Identity | M (see NOTE 1) | Identity of the requested VAL user or VAL UE |
| VAL service ID | O | Identity of the VAL service for which the location reporting trigger is set. |
| Immediate Report Indicator | O (see NOTE 2) | Indicates whether an immediate location report is required |
| Requested location information | O (see NOTE 2) | Identifies what location information is requested |
| Triggering criteria | O (see NOTE 2) | Identifies when the requesting client/VAL server is expecting to receive the location report from the requested VAL user/ VAL UE. It may include VAL service Area ID based criteria when this information flow is from VAL server to the location management server. |
| Minimum time between consecutive reports | O see (NOTE 2) | Defaults to 0 if absent otherwise indicates the interval time between consecutive reports |
| Endpoint information | O | Information of the endpoint of the requesting VAL server to which the location report notification has to be sent. It is provided if Immediate Report Indicator is set to required. |
| NOTE 1: The identity of the requesting VAL user/UE/VAL server and the requested VAL user/UE should belong to the same VAL service. NOTE 2: At least one of these rows shall be present. |  |  |

#### 9.3.2.9	Get UE(s) information request

Table 9.3.2.9-1 describes the information flow for a VAL server to get UE(s) information at the LM server.

Table 9.3.2.9-1: Get UE(s) information request

| Information element | Status | Description |
| --- | --- | --- |
| Location information | M | Location information around which the UE(s) information is requested (e.g. geographic area or VAL service area ID). |
| Application defined proximity range information | M | Description of the range information over which the UE(s) information is required. |

#### 9.3.2.11	Monitor Location Subscription Request

Table 9.3.2.11-1 describes the information flow from the VAL server to the LM server for monitoring the location deviation of the VAL UE(s).

Table 9.3.2.11-1: Monitor Location Subscription Request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identifier of the VAL users or VAL UE whose location monitoring is requested to be monitored in a given location. |
| Area of Interest | M | Geographic area location information where the VAL server wishes to monitor the VAL UE's location adherence. This could be geographical co-ordinates or VAL service area identified by the VAL service area ID. |
| Notify Interval | M | Periodic time interval in which the LM server needs to notify the VAL UE's location information to the VAL server. |
| Notification Target URI | M | Target URI where the VAL server wishes to receive the notifications about VAL UE's location. |
| Timeout | O | A timeout period when subscription response is not received. |

#### 9.3.2.14	Location area monitoring subscription request

Table 9.3.2.14-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription request.

Table 9.3.2.14-1: Location information monitoring subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| Location Information criteria | M | Location information to be monitored. It includes the geographic location information or a VAL service area identified by the VAL service area ID or a reference UE along with the application defined proximity range from the reference UE. |
| Time between consecutive reports | O | It indicates the interval time between consecutive reports |
| Immediate Report Indicator | O | Indicates whether an immediate location report is required |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled) |

#### 9.3.2.17	Location area monitoring subscription modify request

Table 9.3.2.17-1 describes the information flow from the VAL server to the location management server for location area monitoring subscription modify request.

Table 9.3.2.17-1: Location information monitoring subscription modify request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server, VAL UE or SEAL Server |
| Subscription identity | M | Identifies the subscription |
| Location Information criteria | M | Location information to be monitored. It includes the geographic location information or a VAL service area ID or a reference UE along with the application defined proximity range from the reference UE. |
| Time between consecutive reports | O | It indicates the interval time between consecutive reports |
| Immediate Report Indicator | O | Indicates whether an immediate location report is required |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled) |

* * * Next Change * * * *

#### 9.3.2.X	VAL service area configuration request

Table 9.3.2.X-1 describes the information flow from the the VAL server to the location management server for VAL service area configuration request.

Table 9.3.2.X-1: VAL service area configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server |
| VAL service area ID list | M | List of one or more VAL service area identifers |
| Geographical co-ordinates list | M | List of one more more geographical co-ordinates corresponding to each VAL service area ID |

#### 9.3.2.Y	VAL service area configuration response

Table 9.3.2.Y-1 describes the information flow from the location management server to the VAL server for VAL service area configuration response.

Table 9.3.2.Y-1: VAL service area configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL Service area configuration status | M | It indicates the result of VAL service area configuration request. |

* * * Next Change * * * *

### 9.4.X	SS_VALServiceAreaConfiguration API

#### 9.4.X.1	General

API description: This API enables the VAL server to configure the list of VAL service area identifiers and the corresponding geographical co-ordinates with SEAL LM server.

#### 9.4.X.2	Configure_VAL_Service_Area

API operation name: Configure_VAL_Service_Area

Description: Configuration of VAL service area identifiers.

Known Consumers: VAL server.

Inputs: See subclause 9.3.2.X

Outputs: 9.3.2.Y.

See subclause 9.3.X.1 for the details of usage of this API operation.

* * * Next Change * * * *

### 9.3.10	Obtaining UE(s) information at a location

Figure 9.3.10-1 describes the procedure for obtaining UE(s) information at a location.

Pre-condition:

-	The VAL server has a jurisdiction over a geographical area for which the location management server is configured to operate.

-	The UE(s) in the geographical area have provided its location information to the location management server.

Figure 9.3.10-1: Obtaining UE(s) information at a location

1.	The VAL server sends get UE information request to the location management server. The request contains a location information or VAL service area identified by the VAL service area ID and application defined proximity range.

2.	The location management server determines the UE(s) whose location are within the application defined proximity range of the location information or VAL service area identified by the VAL service area ID provided in step 1.

3.	The location management server sends get UE information response to the VAL server with a list of UE(s) and its corresponding location information or VAL service area ID as determined in step 1.

* * * Next Change * * * *

### 9.3.12	Location area monitoring information procedure

#### 9.3.12.1	Location area monitoring subscribe procedure

Figure 9.3.12.1-1 illustrates the high level procedure of location area monitoring subscription request. The same procedure can be applied for location management client and other SEAL servers that would like to subscribe to the list of UEs moving in or moving out of the specific location area. The subscribe request can be for a reference UE for which the subscriber is authorized to monitor location information.

Figure 9.3.12.1-1: Location area monitoring subscription procedure

1.	The VAL server sends a location area monitoring subscription request to the location management server to subscribe to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.14-1. The location information criteria may include the geographic location information or the VAL service area identified by the VAL service area ID where the UEs moving in or moving out to be monitored, or it may include reference UE information where in the UEs moving in or moving out of given application defined proximity range from the reference UE (target UE) to be monitored. The reference UE information may include VAL UE ID.

2.	The location management server shall check if the VAL server is authorized to initiate the location area monitoring subscription request.

3.	The location management server replies with a location area monitoring subscription response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.15-1.

* * * Next Change * * * *

#### 10.3.2.34	Location-based group creation request

Table 10.3.2.34-1 describes the information flow location-based group creation request from the group management client or VAL server to the group management server.

Table 10.3.2.34-1: location-based group creation request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the group management client performing the request. |
| Location criteria | M | Criteria to combine Users or UEs in a location (e.g. geographical area or VAL service area ID). |
| VAL service ID list (see NOTE 1) | O | List of VAL services whose service communications are to be enabled on the group. |
| VAL service specific information (see NOTE 2) | O | Placeholder for VAL service specific information |
| NOTE 1: This information element shall be included in the message for creating a group configured for multiple VAL services. NOTE 2: The details of this information element are specified in VAL service specific specification and are out of scope of the present document. |  |  |

* * * Next Change * * * *

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message and the location criteria may be the VAL service area identified by the VAL service area ID.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information or the VAL service area identified by the VAL service area ID.

3.	The location management server composes the list of users or UEs within the requested location.

4.	The group management server receives the composed list of users or UEs from the location management server. The location management server may include VAL service IDs associated with the list of users or UEs.

5.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group's configuration information. If VAL service ID list was received in step 1 as part of group formation criteria and not in step 4 for the composed list of users or UEs, the group management server shall retrieve the VAL service data from the configuration management server as described in clause 11.3.2.13 and form the group with relevant users or UEs.

NOTE:	The exact policies are out of scope of the present document.

6.	The group management server provides a location-based group creation response to the group management client or the VAL server.

* * * Next Change * * * *

### 9.3.x	VAL Service Area configuration

#### 9.3.x.1	General

The VAL service area refers to the geographical area served by a VAL server. The VAL server may have more than one VAL service area defined. These VAL service areas are configured by the VAL server to the location management server and and are assigned with the unique identifier. When the VAL service area configurations in place the VAL server may use the VAL service area ID as part of the location information in the SEAL APIs requiring location information. These configurations can be updated by the VAL server in-order to modify the geographical area associated with the VAL service area ID and when is updated, the SEAL internally manage the updates without requiring the VAL server to re-invoke the APIs where the updated VAL service area ID are being used.

#### 9.3.x.2	Managing VAL service area configuration

Editor's note: It is FFS how the updates to the VAL Service area ID are being handled by the SEAL layer.

* * * End Change * * * *