3GPP TSG-SA6 Meeting #52	S6-223481

# Toulouse, France, 14th Nov 2022 - 18th Nov 2022						(revision of S6-223190)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0139 | rev | 1 | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | eEDGE_Application traffic influence trigger from EAS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile (Suzhou) Software |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As per conclusion of TR 23.700-98 solution #9, the procedure and information element for Application traffic influence trigger from EAS are to be addressed in the normative work. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Information flows for the procedure are specified in 8.x.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The new features that are introduced and concluded in Rel. 18 study will not work. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8. 6. x. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change  * * * *

8.6.x	Application traffic influence trigger from EAS

8.6.x.1	General

An EAS can explicitly request EES to influence the EAS traffic from UE(s) with necessary information. Then the EES can trigger the AF request to influence traffic routing towards the 3GPP CN for one or more UE(s) accessing the EAS.

8.6.x.2	Procedure

Figure 8.6.x.2-1 illustrates the proceduce of application traffic influence trigger from EAS.

Figure 8.6.x.2-1: Application traffic influence trigger from EAS

1.	The EAS sends an EAS traffic influence request.

2.	The EES performs an authorization check to verify whether the EAS has the authorization to request application traffic influence.

3.	Upon successful authorization, the EES includes target DNAI, traffic descriptor information and N6 routing information at target DNAI in the Nnef_TrafficInfluence_Create/Update Request to the NEF, or Npcf_PolicyAuthorization_Create/Update Request to the PCF, to influence the traffic for EAS as described in 3GPP TS 23.501, clause 5.6.7.1.

4.	The EES sends the EAS traffic influence response.

8.6.x.3	Information flows

8.6.x.3.1	General

The following information flows are specified for application traffic influence trigger from EAS:

-	Application traffic influence trigger from EAS request and response.

8.6.x.3.2	Application traffic influence trigger from EAS request

Table 8.6.x.3.2-1: application traffic influence trigger from EAS request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | Identifier of the EAS |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| Target UE Identifier(s) | M | Indicates the target UE(s) or any UE. |

8.6.x.3.3	Application traffic influence trigger from EAS response

Table 8.6.x.3.3-2: application traffic influence trigger from EAS response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the traffic influence request was successful. |
| Failure response | O | Indicates that the traffic influence request has failed. |
| > Cause | O | Indicates the cause of failure |

8.6.x.4	APIs

8.6.x.4.1	General

Table 8.6.x.4.1-1 illustrates the APIs for application traffic influence trigger from EAS.

Table 8.6.x.4.1-1: Eees_TrafficInfluenceEAS API

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Eees_TrafficInfluenceEAS | Create | Request/Response | EAS |

8.6.x.4.2	Eees_TrafficInfluenceEAS_Create operation

API operation name: Eees_TrafficInfluenceEAS_Create

Description: The consumer requests traffic influence for EAS via EES.

Inputs: See clause 8.6.x.3.2.

Outputs: See clause 8.6.x.3.3.

See clause 8.6.x.2 for details of usage of this operation.