3GPP TSG-SA WG6 Meeting #53	S6-230663

Athens, Greece 27th February – 3rd March 2023 	(revision of S6-230164)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0111 | rev | 0000 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Store and Forward update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Convida Wireless LLC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-02-20 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Moves some of the S&F functionality which was shown on the inbound to be on the outbound. Includes deferred delivery functionality covered within S&F |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new MSGin5G client registration state *at the MSGin5G Server) is added to support differed delivery. This is accomplished via updates to the UE registration and de-registration functions. The S&F functionality split between MSGin5G inbound, outbound and the S&F-specific procedure is updated |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.1; 8.2.2; 8.3.2; 8.3.3; 8.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.1	MSGin5G UE Registration

The signalling flow for MSGin5G UE registration is illustrated in figure 8.2.1-1. The procedure assumes that the MSGin5G UE is responsible for starting initial registration to the MSGin5G Server in order to establish association with the MSGin5G Server to receive MSGin5G Services.

This procedure is used by MSGin5G UE also for registration update to provide new registration parameters values. This procedure is also used by MSGin5G UE for registration resume after performing a registration hold using the procedure in clause 8.2.2.

Pre-conditions:

1.	The MSGin5G UE has connected to the serving network successfully.

2.	The MSGin5G UE has successfully completed the Configuration procedure; alternatively, a UE Service ID and  the MSGin5G Server address have been pre-configured on the MSGin5G UE.

3.	Both the MSGin5G UE and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

4.  When using the procedure to perform registration update, the MSGin5G UE has performed an initial registration (clause 8.2.1). When using the procedure to perform registration resume, the MSGin5G UE has performed an initial registration (clause 8.2.1) and a registration hold (clause 8.2.2.)

Figure 8.2.1-1: MSGin5G Client registration

1.	The MSGin5G UE sends an MSGin5G UE registration request to the MSGin5G Server. The request includes the registration type, UE Service ID and MSGin5G Client Profile information as detailed in Table 8.2.1-1.

Table 8.2.1-1: MSGin5G UE registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE. |
| Registration type | O | Indicates whether the request is for initial, update or resume registration. When the IE is not present initial registration is assumed. |
| MSGin5G Client Profile | O | Set of parameters describing the MSGin5G Client |
| >MSGin5G Client Triggering Information | O | UE Identifier (i.e., MSISDN, external ID), port number(s) and associated protocol (e.g., SMS, NIDD, etc.) for device triggering. The MSGin5G Server uses the information in step 5 of clause 8.8.3. See Table 8.2.1-2. |
| >MSGin5G Client Communication Availability | O | Communication availability information for the MSGin5G Client to receive MSGin5G messages. This IE informs the MSGin5G Server if the client has a specific application-level schedule/periodicity to its MSGin5G communications, which may be used in conjunction with UE reachability monitoring to determine whether and when MSGin5G communications are attempted. See Table 8.2.1-3. |
| > MSGin5G Client Supported Maximum MSGin5G segment size | O | The Maximum MSGin5G segment size can be used by the MSGin5G Server to deliver message to the client served by it in its MSGin5G service domain. The MSGin5G message sent to the MSGin5G Client should be segmented by the MSGin5G Server serves the receiver if the message size is bigger than the MSGin5G Client Supported Maximum MSGin5G segment size. The value of this IE is decided by the MSGin5G Client, and is depended on the MSGin5G Client capabilities, e.g. supported transport, computing capability or application processing time limitation. If this IE is not included, the MSGin5G Server shall use the pre-configured global value within the MSGin5G service domain. |

Table 8.2.1-2: MSGin5G Client Triggering Information

| Information element | Status | Description |
| --- | --- | --- |
| MSGin5G UE ID | M | Identity of the UE hosting the MSGin5G Client (e.g., the External Identifier defined in TS 23.682 [8], or an MSISDN) |
| MSGin5G Client Ports | M | List of port numbers that the MSGin5G Client listens on for device triggers from the MSGin5G Server. Also included with each port number is an associated protocol (e.g., SMS, NIDD, etc.). |

Table 8.2.1-3: MSGin5G Client and Non-MSGin5G UE Communication Availability

| Information element | Status | Description |
| --- | --- | --- |
| Scheduled communication time | M | Time when the UE becomes available for communication. |
| Communication duration time | M | Duration time of periodic communication. |
| Periodic communication indicator | O | Identifies whether the client communicates periodically or not, e.g., on demand. |
| Periodic communication interval | O | Interval Time of periodic communication. This IE is mandatory if the Periodic communication indicator indicates periodic communications. |
| Data size indication | O | Indicates the expected data size to be exchanged during the communication duration. |
| Store and forward option | O | Indicates opting out of store and forward services for incoming MSGin5G requests. |

2.	Upon receiving the request, the MSGin5G Server initiates authentication procedures with the MSGin5G Client and authorizes the MSGin5G Client.

NOTE:	The authentication procedures in step 2 are built on top of the transport layer mechanism specified in Annex Y.2 of 3GPP TS 33.501 [16].

3.	The MSGin5G Server sends an MSGin5G UE registration response to the MSGin5G UE. The response includes the information elements as detailed in Table 8.2.1-4.

The MSGin5G Server checks whether the registration type corresponds to the existing registration status, so for a successful procedure:

- For an initial registration, the existing registration status is “unregistered”.

- For a registration update, the existing registration status is “registered”.

- For a registration resume, the existing registration status is “registration hold”.

If the registration is successful, the MSGin5G Server stores the UE Service ID and associated MSGin5G Client Profile information. For all registration types, the MSGin5G Server sets the registration status to “registered”.

If the registration is not successful, the MSGin5G Server leaves the registration status unchanged.

Table 8.2.1-4: MSGin5G UE registration response

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE. |
| Registration result | M | Indication if the registration is success or failure |
| Failure Cause | O | The reason for failure |

### 8.2.2	MSGin5G UE De-Registration

This procedure is used by MSGin5G UE to perform de-registration or registration hold.  By de-registering, the MSGin5G UE informs the MSGin5G Server that it wishes to terminate its association with the MSGin5G Server. By performing registration hold, the MSGin5G UE informs the MSGin5G Server that it anticipates registering again.

NOTE 1:	Both de-registration and registration hold imply that Client Triggering Information and the Client Communication Availability Information are no longer valid.

The procedure assumes that the MSGin5G UE is responsible for initiating the de-registration or registration hold from the MSGin5G Server.  The signalling flow for MSGin5G UE de-registration procedure is illustrated in figure 8.2.2-1.

Pre-conditions:

1.	The MSGin5G UE is registered to the MSGin5G Server.

Figure 8.2.2-1: MSGin5G UE de-registration

1.	The MSGin5G UE determines to de-register from the MSGin5G Server or to perform registration hold.

2.	The MSGin5G UE sends an MSGin5G UE de-registration request to the MSGin5G Server that includes the de-registration type and UE Service ID, as detailed in Table 8.2.2-1.

Table 8.2.2-1: MSGin5G UE de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the MSGin5G UE. |
| De-registration type | O | Indicates whether the request is for de-registration or registration hold. When the IE is not present, de-registration is assumed. |

3.	The MSGin5G Server may initiate authentication procedures with the MSGin5G Client and authorizes the MSGin5G Client. The MSGin5G Server deletes any applicable UE Service ID and associated MSGin5G Client Profile information that it has stored.

NOTE 2:	The authentication procedures in step 3 are built on top of the transport layer mechanism specified in Annex Y.2 of 3GPP TS 33.501 [16].

4.	If the existing registration status is “registered”  the MSGin5G Server checks the de-registration type and:

- For a registration hold, it sets the registration status to “registration hold”.

- For a de-registration, it sets the registration status to “de-registered”.

The MSGin5G Server an MSGin5G UE de-registration response as detailed in Table 8.2.2-2 to the MSGin5G UE.

Table 8.2.2-2: MSGin5G UE de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the MSGin5G UE. |
| De-registration result | M | Indication if the de-registration is success or failure |
| Failure Cause | O | The reason for failure |

* * * Next Change * * * *

### 8.3.2	MSGin5G inbound messages into the MSGin5G Server

Figure 8.3.2-1 shows the procedure for an MSGin5G UE that initiates an MSGin5G message request.

Figure 8.3.2-1: New MSGin5G message request from UE

Figure 8.3.2-2 shows the procedure for an Application Server that initiates an API request specified in clause 9.1.1.1 for sending an MSGin5G message to UE.

Figure 8.3.2-2: Application Server initiates a request for sending an MSGin5G message.

Figure 8.3.2-3 shows the procedure for a Legacy 3GPP Message Gateway or a non-3GPP Message Gateway that sends a new MSGin5G message request to the MSGin5G Server on behalf of a Legacy 3GPP UE or Non-3GPP UE.

Figure 8.3.2-3: New MSGin5G message request sending from Message Gateway

The following procedure applies to the above figures 8.3.2-1, 8.3.2-2 and 8.3.2-3 with the exception that step 1 only applies to figure 8.3.2-1.

1.	The Application Client in the UE sends a request to the MSGin5G Client for invoking the MSGin5G Client to send a new MSGin5G message to a recipient or to multiple recipients.

Editor's note:	Whether the APIs provided by the MSGin5G Client to the Application Client is to be specified in another clause of the TS is FFS.

2.	As shown in figure 8.3.2-1 or 8.3.2-3, the MSGin5G Client or Message Gateway sends the MSGin5G message request to the MSGin5G Server and includes the IEs as listed in table 8.3.2-1 in the request; or as shown in figure 8.3.2-2, the Application Server sends an API request to the MSGin5G Server for sending an MSGin5G message, the API request includes the IEs as listed in table 8.3.2-1.

Table 8.3.2-1: Request to MSGin5G Server for sending MSGin5G message

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the sending MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the sending Application Server. |
| Recipient UE Service ID/AS Service ID (see NOTE 1, NOTE 2) | O | The service identity of the receiving MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the receiving Application Server. This IE is mandatory for Point-to-Point messaging, AS-to-Point messaging, AOMT messaging and MOAT messaging and is not present in other message scenarios. |
| Group Service ID (see NOTE 1) | O | The service identifier of the target MSGin5G Group. This IE is mandatory for a Group Message and is not present in other message scenarios. |
| Broadcast Area ID (see NOTE 1) | O | The service identifier of the Broadcast Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. |
| Messaging Topic (see NOTE 1) | O | Indicates which Messaging Topic this message is related to. This IE is mandatory for a message distribution based on topic and is not present in other message scenarios. |
| Application ID | O | Identifies the application(s) for which the payload is intended. This list of Application IDs IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. This list of Application IDs IE may be included when the message is sent to an Application Server or to an Application Client. MSGin5G Server is unaware of the content. |
| Message ID | M | Unique identifier of this message. |
| Delivery status required | O | Indicates if delivery acknowledgement from the recipient is requested. |
| Payload | O | Payload of the message. MSGin5G Server/Client is unaware of the content. If the request is sent from MSGin5G Client or Message Gateway to the MSGin5G Server, the maximum size of this IE is a configurable value that shall not exceed 2048 octets. |
| Priority type (see NOTE 3) | O | Application priority level requested for this message. The application priority levels include High, Normal and Low. The default Priority type of an MSGin5G message is Normal. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Segmentation set identifier | O | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. Mandatory IE to be present in every segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. The Total Segments needs to be included only in the first segment of the message. |
| Message segment number | O | An incrementing message segment number that indicates segmented message number of each segmented message within a set of segmented messages |
| Last segment flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. The Last Segment Flag needs to be included only in the last segment of the message. Message segment number of the segment with "Last Segment Flag" set can be considered as total segments. |
| Store and forward flag | M | An indicator of whether store and forward services are requested for this message. If the value indicates that store and forward services are requested by the sender, the store and forward procedure in clause 8.3.6 applies. |
| Store and forward parameters | O | Parameters used by MSGin5G Server for providing store and forward services, as detailed in table 8.3.2-2. This IE shall be included only if the value of the Store and forward flag IE indicates that store and forward services are requested. The MSGin5G store and forward procedure is detailed in clause 8.3.6. |
| NOTE 1: Only one of these IEs may be included to represent the type of message request. The MSGin5G Client may construct the related IEs based on the information received from Application Client, e.g. adds the MSGin5G service domain. When the recipient is the MSGin5G server, none of these IEs are included. NOTE 2: When the originator is an Application Server, (i.e. Originating AS Service ID is present), this IE shall be a UE Service ID. NOTE 3: The MSGin5G message with high priority should not be aggregated. The other usages of the priority level of the message are implementation specific and is out of scope of this document. |  |  |

Table 8.3.2-2: Store and forward parameters

| Value | Status | Description |
| --- | --- | --- |
| Message expiration time | O | Indicates message expiration time used for providing store and forward services if the destination is not available for communications, The MSGin5GServer attempts delivery at or before the message expiration time, or when the recipient becomes available. |
| Application specific store and forward information | O | Application specific information about store and forward handling, e.g. a delivery time/date. |

3.	The MSGin5G Server verifies that the sender is authorized to send the message. If the sender is not authorized step 4 is skipped and the message considered rejected.

4.   If the message destination is the MSGin5G Server, then the MSGin5G Server consumes the message and proceeds to step 5 indicating a successful request. For UE, GW or AS destinations with de-registered  registration status the MSGin5G Server discards the message and proceeds to step 5 indicating a failure. Otherwise the request is considered successful and MSGin5G Server retains the message for further processing.

5.	The MSGin5G Server sends a message response indicating failure to the originating entity if the message is rejected or the recipient is not registered/de-registered and includes the IEs as listed in table 8.3.2-3 in the response.  If the request was successful, the MSGin5G Server may send a corresponding successful message response with the IEs listed in table 8.3.2-3.

Table 8.3.2-3: Message Response

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The identity of the MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the identity of the Application Server that initiated the previous Request. |
| Message ID | M | Identifier of the initiating Request. |
| Delivery Status | O | Indicates if delivery is a failure, or if the message is stored for deferred delivery. |
| Failure Cause | O | The reason for failure |

### 8.3.3	MSGin5G outbound messages from the MSGin5G Server

Figure 8.3.3-1 shows the procedure for the MSGin5G Server that forwards an MSGin5G message.

Figure 8.3.3-1: MSGin5G message towards UE

Figure 8.3.3-2 shows the same procedure (step 1 only), however for the MSGin5G Server that sends the message to an Application Server by application request.

Figure 8.3.3-2: Message towards an Application Server

Figure 8.3.3-3 shows the procedure for the MSGin5G Server that sends an MSGin5G message to a Legacy 3GPP Message Gateway, a Non-3GPP Message Gateway, or a Broadcast Message Gateway.

Figure 8.3.3-3: MSGin5G message towards a Message Gateway

The following procedure applies to the above figures 8.3.3-1, 8.3.3-2 and 8.3.3-3 with the exception that step 2 only applies to figure 8.3.3-1.

Preconditions:

1. An MSGin5G message with destination different than the MSGin5G Server is available.at MSGin5G Server

Procedural steps:

1.	If the value of the Store and forward flag IE in the MSGin5G message request indicates that store and forward services are requested by the sender, the MSGin5G Server determines whether the  destination UE is available.

If the destination is not available, the MSGin5G stores the message and the procedure in 8.3.6 applies instead. Otherwise, the MSGin5G Server sends the MSGin5G message request and includes the IEs as listed in table 8.3.3-1.

NOTE:	The MSGin5G Server uses UE reachability status monitoring specified in clause 8.9.2 to or the recipient's Communication Availability information specified in clause 8.2 to determine whether the destination is available for message delivery.

Table 8.3.3-1: MSGin5G message request from MSGin5G Server

| Information element | Status | Description |
| --- | --- | --- |
| Originating UE Service ID/AS Service ID | M | The service identity of the originating MSGin5G Client, Legacy 3GPP UE, Non-3GPP UE or the originating Application Server. This IE is copied from the associated inbound message. |
| Recipient UE Service ID/AS Service ID (see NOTE1, NOTE 2) | O | The service identity of the receiving entity. This IE is present in case the recipient is an individual UE or Application Server For Point-to-Point messaging, AS-to-Point messaging/Point-to-AS messaging, this IE is copied from the associated inbound message. For Group messaging, this IE can be a Recipient UE Service ID only. This IE is fetched from the participant information of the recipient in the group profile. For message delivery based on Messaging Topic subscription, this IE is the UE Service ID/AS Service ID of the Messaging Topic subscriber. |
| Broadcast Area ID (see NOTE 1) | O | The identifier of the Service Area where the message needs to be broadcast. This IE is mandatory in the Broadcast Message and is not present in other message scenarios. This IE is copied from the associated inbound message. |
| Application ID | O | Identifies the application for which the payload is intended. This list of Application ID(s) IE is required when the message is sent to one or multiple Application Clients served by same MSGin5G Client. This list of Application ID(s) IE may be included when the message is sent to an Application Server or to an Application Client. This list of IEs is copied from the associated inbound message. |
| Message ID | M | Unique identifier of this message. This IE is copied from the associated inbound message request. |
| Delivery status required | O | Indicates if delivery acknowledgement from the recipient is requested. This IE is copied from the associated inbound message. |
| Payload | O | Payload of the message. This IE is copied from the associated inbound message. |
| Message is segmented | O | Indicates this message is part of a segmented message. |
| Group Service ID | O | The service identifier of a Group. This IE is included if message delivery is based on Group messaging. This IE is copied from the Recipient Group Service ID IE in the associated inbound message. |
| Messaging Topic | O | Indicates which Messaging Topic this message is related to. This IE is included if message delivery is based on a Messaging Topic subscription. This IE is copied from the Messaging Topic IE in the associated inbound message. |
| Segmentation Set Identifier | O | All segmented messages associated within the same set of segmented messages (i.e. associated with the same MSGin5G message) are assigned the same unique identifier. Mandatory IE to be present in every segmented message. |
| Total number of message segments | O | Indicates the total number of segments for the message. The Total Segments needs to be included only in the first segment of the message. |
| Message segment number | O | An incrementing message segment number that indicates segmented message number of each segmented message within a set of segmented messages. |
| Last Segment Flag | O | An indicator of whether this segmented message is the last segment in the set of segmented messages or not. The Last Segment Flag needs to be included only in the last segment of the message. Message segment number of the segment with "Last Segment Flag" set can be considered as total segments. |
| Priority type | O | Application priority level requested by the message originator for this message. |
| NOTE 1: Only one of these IEs shall be included to represent the type of message request. The MSGin5G Client may construct the IEs specified in table 8.11.5-1 based on the IEs above in the received MSGin5G message, e.g. removes the MSGin5G service domain, if the Payload is sent to Application Client. NOTE 2: When the originator is an Application Server, (i.e. Originating AS Service ID is present), this IE shall be a UE Service ID. |  |  |

If the received MSGin5G message request is for Group Message, the MSGin5G Server shall, in addition to the Recipient Group Service ID IE, add the Recipient UE Service ID of each individual group member, excluding the message originator.

If the received MSGin5G message request is for a Topic Message, the MSGin5G Server shall, in addition to the Messaging Topic IE, add the Recipient UE Service ID/AS Service ID of each individual Topic subscriber, excluding the message originator.

* * Next  Change * *

### 8.3.6	MSGin5G Store and Forward

Figure 8.3.6-1 shows the procedure for providing store and forward services for MSGin5G message requests.

This procedure applies when an MSGin5G inbound message is received at the MSGin5G Server and the recipient UE is not available; otherwise, the procedure detailed in clause 8.3.2 applies.

Pre-conditions:

1.	An inbound MSGin5G message requiring store and forward services has been stored at the MSGin5G Server.

2.  If the message destination is an Application Server, the MSGin5G Server has determined that it is in registered state.

If the message destination is an MSGin5G client, the MSGin5G Server has determined that either:

- the MSGin5G client is registered and that the recipient UE is not available; or

- or the MSGin5G client is registration hold state,

Figure 8.3.6-1: Store and forward procedure

1.	.  The MSGin5G Server uses the Application specific store and forward information IE (see Table 8.3.2-2) to determine expiration time of the stored message.

.

2.	Before the Message expiration time expires, the MSGin5G Server may trigger the Recipient UE based on the MSGin5G device triggering procedure in clause 8.9.3.

.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes store and forward status information in the Delivery Status IE, e.g., that the delivery had been deferred.

3.	The recipient UE becomes available.

NOTE:	If the destination MSGin5G client was in registration hold when the message was received by the MSGin5G server, the MSGin5G client must first perform a registration resume before availability can be determined. For a registered MSGin5G client, the MSGin5G Server can use UE reachability status monitoring specified in clause 8.9.2 or the recipient's Communication Availability information specified in clause 8.2 to determine whether the recipient is available for message delivery.

4.	When the recipient UE becomes available, the MSGin5G Server attempts delivery of the request using the procedure specified in clause 8.3.3.

If the UE does not become available prior to the expiration time, the MSGin5G Server attempts delivery of the request at the message expiration time and the stored message is discarded afterwards.

5.	The MSGin5G Server may send a message response as defined in table 8.3.2-3 which includes store and forward status information in the Delivery Status IE, e.g., that the message was discarded.