3GPP TSG-SA WG6 Meeting #55	S6-231873

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0389 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Selected EES declaration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In service continuity between edge and cloud w/o CES, the CAS is not aware of the serving EES selected in edge, therefore, the CAS is unable to utilize the needed EEL service, for instance: - subscribe to receive ACT status notifications in S-EES executed ACR scenario for EELManagedACR - trigger CAS decided ACR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enable the CAS to know the selected EES in EDN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing functionality in EDGEAPP to support service continuity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.X (new), 8.8.4.x1 (new), 8.8.4.x2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.3.X	Selected EES declaration

Figure 8.8.3.X-1 illustrates the interactions between the EES and the CAS for the selected EES declaration.

Pre-conditions:

1.	A serving EES is selected for the EEC and the serving EES decides to inform CAS.

Figure 8.8.3.X-1: Selected EES declaration procedure

1.	The EES sends Selected EES declaration request message to the CAS. The request includes the information of the selected EES and may include ACID to indicate which AC the EES is intended for.

2.	The CAS checks whether the requesting EES is authorized to perform operation. If authorized, the CAS stores the received information.

3.	The CAS responds the request with Selected EES notification declaration response message.

The CAS, after kowning the selected EES for the UE, may subscribe to receive ACT status notifications as described in clause 8.8.3.6.2.3 for EELManagedACR, or trigger service continuity procedures towards the selected EES.

* * * Next Change * * * *

#### 8.8.4.x1	Selected EES declaration request

Table 8.8.4.x1-1 describes information elements for the selected EES declaration request sent from the EES to the CAS.

Table 8.8.4.x1-1: Selected EES declaration request

| Information element | Status | Description |
| --- | --- | --- |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| UE ID | M | The identifier of the UE. |
| Selected EES ID | M | Selected EES identifier. |
| Selected EES Endpoint | M | Endpoint of the selected EES. |
| EAS ID | M | The identifier of the EAS. |
| AC ID | O | The identifier of the AC. |

#### 8.8.4.x2	Selected EES declaration response

Table 8.8.4.x2-1 describes information elements for the selected EES declaration response sent from the CAS to the EES.

Table 8.8.4.x2-1: Selected EES declaration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response (NOTE) | O | Indicates that the request was successful. |
| Failure response (NOTE) | O | Indicates that the request failed. |
| > Cause | O | Indicates the failure cause. |
| NOTE: One of these IEs shall be present in the message. |  |  |

* * * END of Change * * * *