3GPP TSG-SA WG6 Meeting #55	S6-231909

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

Source:	Samsung

Title:	Pseudo-CR on corrections to the PIN registration procedures

Spec:	3GPP TS 23.542 V0.3.0

Agenda item:	8.19

Document for:	Approval

Contact:	Arunprasath R, arun.prasath@samsung.com

1. Introduction

This pCR corrects some of the procedures related to PINE registration.

2. Reason for Change

Figures and some editorial corrections.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.542 V0.3.0.

* * * First Change * * * *

##### 8.4.2.2.1	PINE registration directly to PIN server

Figure 8.4.2.2.1-1 illustrates PIN registration procedure based on request/response model.

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The UE Identifier is available;

3.	The PINE has been authorized to communicate with the PIN server;

Figure 8.4.2.2.2-1: PINE registration directly to PIN server

1.	The PINE (including PINE/PEMC/PEGC) sends a PINE Registration Request to the PIN server. The request includes the security credentials of the PINE received during authorization procedure and also the request may include the GPSI, MAC address, vendor name, device description, PINE Address.

If the PEMC trigger the request, this registration request carries the PIN ID of the PIN for which it is intending to register as PEMC and the PIN element may indicate whether it is to be assigned with primary or secondary PEMC role.

2.	The PIN server checks whether the UE identified by the GPSI has subscribed to be a PEMC. Also, the PIN server can check the whether the UE identified by the GPSI has subscribed to be a PEMC itself. If subscribed, it checks the PIN profile whether the requesting PIN element can be assigned with primary or secondary role and authorize the PIN element accordingly. In case if the PIN has already been created, the requesting PIN element is assigned with the secondary PEMC role irrespective of the role requested by the PIN element.

3.	The PIN server responds to the PINE with PINE Registration Response with allocated PIN client ID in successful response.

If the registration procedure fails, the PIN server should give the failure response that indicates the cause of registration request failure.

* * * Next Change * * * *

##### 8.4.2.2.4	PINE registration during the PIN join via the PEGC

Pre-conditions:

1.	The PINE has been pre-configured or has discovered the address (e.g. IP address, FQDN, URI) of the PIN server;

2.	The PINE already establishes the connection with PEGC;

3.	The PINE may get the PIN information from the PEMC, PEGC via PIN announcement after connecting to PEMC or PEGC.

Figure 8.4.2.2.4-1 illustrates the PINE registration during the PIN join via the PEGC procedure based on request/response model.

Figure 8.4.2.2.4-1: PINE registration to PIN server during PIN join via PEGC

1.	The PINE sends PIN join/discovery request to the PEGC. The PINE device identity is included.

-	For the PIN join request, the PIN ID is included.

-	For the PIN discovery request, the discovery criteria are included.

2.	The PEGC identifies the received message is the PIN join/discovery request, and the PINE is not registered and authorized due to no PIN client ID and credentials in the message.

3.	The PEGC returns the PIN join/discovery reject message to the PINE. The registration and authorization indication are also included to instruct the PINE to perform registration.

4.	The PINE if not registered, initiates the PIN registration towards the PIN server via the PEGC as described in clause 8.4.2.2.1.

5-6.	After the registration, the PINE will use the PIN client ID and credentials to initiate the PIN join/discovery again.

* * * Next Change * * * *

#### 8.4.2.3	Information flow/elements

##### 8.4.2.3.1	General

The following information flows are specified for PIN registration:

-	PINE Registration request and response;

##### 8.4.2.3.2	PINE Registration request

Table 8.4.2.3.2-1 describes information elements in the PINE Registration request from the PINE (including PEMC, PEGC, PINEs) to the PIN server.

Table 8.4.2.3.2-1: PINE Registration request

| Information element | Status | Description |
| --- | --- | --- |
| UE Identifier | M | The identifier of the hosting UE (i.e. GPSI or identity token) of PINE/PEMC/PEGC |
| Security credentials | M | Security credentials resulting from a successful authorization for the PIN service. |
| MAC address | O | MAC address of the requested PINEs. |
| Vendor name | O | The vendor name of the PINE |
| Device description | O | Description of the device. |
| PINE Address | O | The IP address of PINE, if available. |

##### 8.4.2.3.3	PINE Registration response

Table 8.4.2.3.3-1 describes information elements in the PINE Registration response from the PIN server to the PINE (including PEMC, PEGC, PINEs).

Table 8.4.2.3.3-1: PINE Registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (see NOTE) | Indicates that the PINE Registration request was successful. |
| > PIN client ID | M | Identifier of the newly assigned PIN client ID to PINEs. |
| > Role of PEMC | O | The UE identified by the GPSI has subscribed to be a PEMC. |
| Failure response | O (see NOTE) | Indicates that the PIN registration request failed. |
| > Cause | M | Provides the cause for PIN registration request failure. |
| NOTE: Atleast one of the IE shall be present. |  |  |